Liaison note from IALA to MSC on the protection of the AIS VDL

RESOLUTION MSC.140(76)

(adopted on 5 December 2002)

RECOMMENDATION FOR THE PROTECTION OF THE AIS VHF DATA LINK

Background

RESOLUTION MSC.140(76) was adopted to provide clarification to administrations regarding the protection of the AIS VHF Data Link (AIS VDL) in anticipation of the large volume of Class B devices that could have a negative impact on the AIS VDL.

Discussion

Since the adoption of RESOLUTION MSC.140(76)  and as identified in ITU-R M.1371 the following AIS devices are permitted:

# Quote

# 2	AIS equipment

## 2.1	AIS VHF data link (VDL) non-controlling stations

## 2.1.1	AIS shipborne station

2.1.1.1	Class A shipborne mobile equipment using SOTDMA technology as described in Annex 2 will comply with relevant IMO AIS carriage requirement:

2.1.1.2	Class B shipborne mobile equipment will provide facilities not necessarily in full accordance with IMO AIS carriage requirement.

–	Class B “SO” using SOTDMA technology as described in Annex 2;

–	Class B “CS” using CSTDMA as described in Annex 7.

### 2.1.2	Aids to navigation-AIS station

### 2.1.3	Limited base station (no VDL control functionality)

### 2.1.4	Search and rescue mobile aircraft equipment

### 2.1.5	Repeater station

### 2.1.6	AIS_Search and Rescue Transmitter (AIS-SART station)

## 2.2	AIS VDL controlling stations

### 2.2.1	Base station

Unquote

Therefore IALA feels that is important to provide additional clarification to RESOLUTION MSC.140(76) considering all AIS stations and their impact on the VDL.

Recommendation

Recognizing the continuing need to ensure the integrity of the AIS VHF data link, IALA requests that IMO amend RESOLUTION MSC.140(76) as follows:

Quote

RECOMMENDS that:

.1  any device which transmits on the radio channels allocated for AIS,  should  meet  the  appropriate  requirements  of  Recommendation ITU-R M.1371 (series);

.2  all AIS devices should be approved by the Administration;

.3  Administrations  should  take  steps  necessary  to  ensure  the  integrity  of  the  radio channels used for AIS in their waters.

Unquote

Action requested

MSC is invited to consider the above recommendation and take appropriate action

This document needs to be put into the IMO Liaison note template format