e-Navigation Committee						e-NAV7/8/23

7th Session								21 Sept 2009

EPFS Performance Requirements

1.	Introduction

This input from the General Lighthouse Authorities of the UK and Ireland provides an analysis of Electronic Position Fixing System (EPFS) requirements, present and future. It may be used as the basis for IALA inputs to IMO on the revision of Resolution A.953(23) and as a response to the Liaison Note from ANM 13 on this subject.

2.	Approach

The approach taken was to review existing documentation on EPFS requirements (especially Refs.1 & 2: IMO Resolutions A.915(22) & A.953(23)), setting out present requirements in terms of the accepted parameters: accuracy, availability, integrity, continuity and coverage.

The validity of these parameters was and their method of expression, was then considered with particular attention to the integrity model for maritime applications. Finally a specification of requirements for existing and planned systems has been proposed.

3.	Documentation Review

IMO Resolutions A.915(22) and A.953(23) are the main reference documents. Other applicable documents, such as IALA Recommendations R.121 (DGNSS) and R.129 (GNSS Vulnerability) use the IMO Resolutions as references.

A.915(22) sets out the Requirements for Future GNSS, A.953(23) gives the requirements to be met by systems for inclusion in the World Wide Radio Navigation System. These documents have been in place for 8 and 6 years respectively and it is acknowledged that the parameters require review. In particular, the accuracies may be unnecessarily stringent for some phases of the voyage, the continuity level is not achievable with existing or planned systems and the treatment of integrity is not satisfactory.

Revision of A.953(23) has now been included in the Work Program for the Safety of Navigation Sub-Committee and is expected to be dealt with at NAV 56 and 57 (2010 & 2011). The analysis in this document may provide the basis for an IALA input on this subject.

4.	Summary of Existing Requirements

A.953 defines requirements for navigation in three types of area:

•	Navigation in those harbour entrances, harbour approaches and coastal waters with a high volume of traffic and/or significant degree of risk;

•	Navigation in those harbour entrances, harbour approaches and coastal waters with a low volume of traffic and/or a less significant degree of risk;

•	Navigation in ocean waters.

Accuracy requirements are 10 m (95%) in harbour entrances, approaches and coastal waters, 100 m (95%) in ocean waters. Availability is 99.8% (calculated over 2 years) and Continuity 99.97% over a 3 hour period, for waters with a high volume of traffic and/or a significant degree of risk. Availability is 99.5% (calculated over 2 years) and Continuity 99.85% over a 3 hour period for low traffic volume/less significant risk. Availability is 99.8% (calculated over 30 days) for ocean waters, with continuity not applicable. In all areas, position update rate should be greater than once every 10 s, or 2 s if used for AIS, graphical display or for direct control of the ship. A broadcast warning within 10 s that the system is not to be used for navigation should be provided in harbour entrances, harbour approaches and coastal waters. In ocean waters this can be provided by Maritime Safety Information systems.

A.915(22) defines requirements by application and phase of voyage:

General navigation is categorised as ocean, coastal, port approach and restricted waters, port and inland waterways; Positioning is divided into Manoeuvring and Traffic Management applications, Search and Rescue, Hydrography, Oceanography, Marine Engineering, Construction and Aids to Navigation Management,  Port operations, Casualty Analysis, Offshore Exploration, Fisheries, Recreation and Leisure.

Navigational accuracy requirements are 10 m except for 1 m in port. Positioning accuracy requirements are also generally 10 m, except for 1 m for tugs, icebreakers, hydrography and aids to navigation management. 0.1 m is required for dredging, construction and cargo handling.

Regarding Integrity, Alert Limit is given as 2.5 x accuracy requirement (i.e. 25 m, 2.5 m or .25 m), Time to Alarm is 10 seconds in all cases, except cargo handling (1 s). Integrity Risk is given as 10 -5 in all cases.

Availability requirement is 99.8% in all cases and continuity 99.97% over 3 hours, where applicable (navigation in port approaches and ports, operational and traffic management applications).

Coverage requirements are global for ocean and coastal navigation, but regional or local for many positioning applications limited to port approaches and ports. Fix interval required is 1 second in all cases..

5.	Assessment of Parameters

As indicated above, the parameters used in existing documentation for maritime applications require review. The accuracy requirements are addressed in a separate paper prepared by the IALA ANM Committee (Ref. 3). This argues that the accuracy requirements for general navigation in A.915 are too stringent and proposes that they be replaced by: 8-20 m for port Approach and restricted waters, 20-50 m for coastal navigation and 50-100 m for ocean navigation. The reasoning for this is that higher accuracies than these irrelevant when compared with the size of the vessel. Specifying higher accuracies increases the cost of the positioning system. Therefore taking the upper limits of these ranges, 20, 50 and 100 m are proposed.

The availability requirements in A.953 are inconsistent, using calculation periods (2 years or 30 days) which are derived from particular systems, rather than user requirements. The normal definition of availability does not specify a period and the length of time used for its calculation is really a matter for the system provider, rather than the user.  (Where availability is calculated for a complete system, such as a GNSS, with multiple sub-systems, a period of 30 days is appropriate. However, if the service relies on individual stations (for example beacon DGNSS) the resultant outage periods would be too short). It is therefore suggested that the period should not be stated in the requirement, but left to the system provider.

Continuity is defined as the probability that a user will be able to determine position with specified accuracy and is able to monitor the integrity of the determined position over the (short) time interval applicable for a particular operation. The continuity requirement of 99.97% over 3 hours in the current documents is not achievable with any existing or planned system, as it would allow an outage of only 3 seconds. Such a level of continuity is not required for any navigational or normal operational application. It has been proposed that the time period be reduced to 15 minutes. This would correspond to the critical phase of a manoeuvre, whereas 3 hours was originally specified to cover a complete approach.

Coverage is referred to in A.915 as global, regional or local, depending on the application. It can be argued that the user does not really care whether the coverage is global or local, as long as it is there when they need it. The decisions about infrastructure for the transmitting, control and monitoring of signals are matters for the service provider. Ideally, the user should not need to know where the service is coming from, but just have some indication of its quality and reliability. This is likely to be the case in the future, when software defined receivers use all available signals to produce a best position solution, with a “usability” indicator.

The treatment of integrity in the existing documents is rather simplistic. The parameters are considered separately, as if they were independent of each other, whereas in fact they are inter-related. For example there is a trade-off between availability and accuracy. Continuity and availability are both related to Mean Time Between Failures. Integrity and accuracy are also linked. These relationships are considered in the discussion of the Integrity Models, which follows.

6.	Integrity Models

The present definition of integrity for maritime applications is the ability to provide users with warnings within a specified time when the system should not be used for navigation. The only parameter specified in A.953 was time to alarm (10 seconds). In A.915, alert limit was specified (error in metres at which alert is given – set at 2.5 x the required accuracy), time to alarm was specified in seconds – 10 seconds in most cases. ”Integrity risk” is given as a probability (of missed detection) over a set time period (10-5 over 3 hours).

Integrity for aviation applications has generally been based on four parameters: alert limit, time to alarm, probability of missed detection and probability of hazardously misleading information.

Loran-C provides integrity warnings within the signal (blink), which meets existing aviation integrity requirements and eLoran will achieve the same result through its data channel. In contrast, GPS and GLONASS were not originally designed to provide continuous integrity. A warning that a satellite should not be used can be provided by setting it unhealthy, but there are built-in delays due to the limited coverage of monitoring and uplink sites. The integrity requirements can be met by providing augmentation from terrestrial broadcasts (such as beacon DGNSS) or separate satellite systems (WAAS, EGNOS).

Galileo was always intended to provide inherent integrity, although only on certain services, such as Safety of Life. The Galileo Ground Segment includes a global network of Sensor Stations (GSS), collecting navigation data from the Galileo satellites and Up-link Stations (ULS) both for Tracking, Telemetry and Command and specific mission related up-links for dissemination of integrity related and other information.

Real-time integrity information is provided by a subset of satellites, which are connected (to a ULS) at any one time, and a minimum of 2 connected satellites are in view at all possible user locations world-wide.

Galileo will monitor the signal-in-space (SIS) within the Ground Segment using the measurements of the GSS. With the known positions of the GSS the actual position of the satellite and the error on the range can be estimated.

The integrity related information is disseminated via the navigation message included in the Safety of Life service signals and two kinds of Integrity Data are supplied:

1. Integrity tables that include flags to indicate the integrity status of each navigation data broadcast by each satellite, to be broadcast every 30 s, and

2. Integrity alerts to be broadcast with an alert process that enables updating of integrity information in real-time (each 1 s)

The Galileo integrity concept also has provision for external regional Integrity data in addition to the global data provided by the core ground segment.  In this way, specific geographic regions may take responsibility for disseminating integrity data for their region.

From these examples it can be seen that an alert limit, a maximum warning time and a maximum probability of missed detection should be sufficient to define the integrity requirement.

7.	Proposed Specification of Requirements

The proposed requirements specification which follows takes account of the arguments set out above, in particular linking availability, integrity and accuracy. However, it is not fundamentally different from the present approach.

An Electronic Position Fixing System is available when it provides the required integrity for the given accuracy level. Similarly the system has continuity if the required accuracy is provided with integrity over the specified time period.

The availability level of an electronic position fixing system should be 99.8%.

The continuity level should be 99.97% over any 15 minute period.

Accuracy required for general navigation is 100 m in the ocean phase, 50 m in coastal phase and 10 m for port approaches and restricted waters (all at 95% probability).

Integrity is provided when the user is notified within 10 seconds that accuracy exceeds 2.5 x the required level. The risk of not detecting such an event should not exceed 10-5 in any 3 hour period. In ocean waters integrity warnings can be provided by Maritime Safety Information systems.

Higher accuracies may be required for specialised navigation and positioning, for example 1 m in port, for tugs, icebreakers, hydrography and aids to navigation management and 0.1 m for dredging, construction and cargo handling. Methods of providing these enhanced levels of service would be determined by considerations of coverage and cost effectiveness, for example by onboard, local, regional or wide area augmentation.

8.	References

IMO, 2001. Resolution A.915(22), Requirements for Future GNSS.

IMO, 2003. Resolution A.953(23). World Wide Radio Navigation System.

IALA, 2009. ANM 13, Output to e-NAV on horizontal accuracy requirements.