AISM Association Internationale de Signalisation Maritime     IALA

International Association of Marine Aids to Navigation and Lighthouse Authorities

IALA Recommendation

R-###

On

the Performance and Monitoring of eLORAN Services in the Frequency Band 90 – 110 kHz

Edition 1.0

December 2009

20ter, rue Schnapper, 78100

Saint Germain en Laye, France

Telephone: +33 1 34 51 70 01    Fax: +33 1 34 51 82 05

e-mail:  iala-aism@wanadoo.fr       Internet:  www.iala-aism.org

This document has been reformatted to comply with the IALA template

Document Revisions

Revisions to the IALA Document are to be noted in the table prior to the issue of a revised document.

| Date | Page / Section Revised | Requirement for Revision |
| --- | --- | --- |

IALA Recommendation on ....... (Block text)

(Recommendation R-####)

THE COUNCIL:

RECALLING the function of IALA with respect to Safety of Navigation, the efficiency of maritime transport and the protection of the environment;

RECOGNISING the need to ensure that Enhanced LORAN (eLORAN) services in the frequency band 90 kHz – 110 kHz are operated in accordance with certain minimum standards that take into account relevant ITU-R Recommendations and IMO Resolutions;

RECOGNISING ALSO that the minimum standards should include the signal format, reference datum, availability, continuity, integrity, accuracy, signal monitoring, range and coverage, status reporting, validation, and the publication of information about the system;

NOTING IMO resolutions A.915(22) on Maritime Policy for the Future Global Navigation Satellite System (GNSS), and A.953(23) on World Wide Radionavigation System;

NOTING ALSO ITU-R Recommendation M.589-2 on the Technical characteristics of eLORAN (as revised) ………

CONSIDERING the proposals made by the e-Navigation Committee

ADOPTS the Minimum Standards for the Performance and Monitoring of eLORAN Services in the frequency band 90 – 110 kHz set out in the annex of this recommendation; and,

RECOMMENDS that National Members and other appropriate Authorities providing, or intending to provide, eLORAN services in the frequency band 90 – 110 kHz, to use the Minimum Standards set out in the annex to this recommendation.

Table of Contents

Document Revisions	2

Table of Contents	4

Index of Tables	4

Index of Figures	4

Annex	6

1	General	6

2	Signal Format	6

2.1	The eLORAN Pulsed Waveform	7

2.2	Time Synchronisation	7

2.3	The eLORAN Data Channel	7

3	Reference Datum	8

4	Availability and Continuity	8

4.1	Availability	9

4.2	Continuity	9

5	Integrity	10

6	Accuracy	10

7	Range/Coverage	10

8	Transmitter Performance	11

9	Receiver Performance	11

10	Monitoring	11

11	Validation	11

11.1	Elements of performance	12

11.2	Availability	12

11.3	Coverage:	12

11.4	Continuity	13

11.5	Control Station	13

12	Publication of Information	13

13	References	15

APPENDIX 1	Definitions and Glossary of Terms used in eLORAN	16

Index of Tables

Table 1	Title required	9

Table 2	Title required	13

Table 3	Differential eLORAN Type 10 = Phase Corrections	18

Index of Figures

Figure 1	Title required	7

Annex

Minimum Standards for the Performance and Monitoring of eLORAN Services in the frequency band 90 – 110 kHz

# General

Enhanced LORAN (eLORAN) is a terrestrial-based positioning, navigation and time distribution system designed for wide area use. eLORAN is a development of former LOng RAnge Navigation systems which has been updated to provide a dissimilar but complementary Position, Navigation & Timing (PNT) system to the widely adopted Global Navigation Satellite Systems (GNSS). The key enhancements in eLORAN are:

- Precise control of transmitter Time of Emission (TOE) referenced to Universal Time Co-ordinated (UTC), which facilitates the use of ‘all-in-view’ receivers, rather than dependence on a single chain of transmitters;

- Incorporation within the receiver of a spatial database applying corrections for Additional Secondary Factors (ASF), which result from signal refraction over land paths;

- Transmission of differential corrections via a data channel to reduce temporal errors and increase accuracy levels in specified areas, e.g. Port and port approaches.

eLORAN involves having, at precisely known locations, stations that transmit a signal in a compatible sequence with adjacent stations (the ‘eLORAN Network’), which are typically hundreds of miles apart and may be operated by different Administrations.  The eLORAN Network is organised in a series of chains comprising a Master Station and 2-4 Secondary Stations. Each station in the chain transmits at a precise interval known as the Group Repetition Interval (GRI); with a specified Emission Delay (ED) between the transmissions of the Master and each Secondary. The timing precision required is achieved by the installation of atomic time standards at each station and monitoring by offsite Control centres.

This Recommendation describes the use of transmitters in the 90 – 110 kHz LF radio-navigation frequency band to transmit eLORAN.

eLORAN provides substantial benefits for many infrastructure modes, by the provision of precision PNT signals with a capacity to penetrate urban and underground areas.  Within the marine field, the primary benefit is the provision of a complementary 2D navigation system with dissimilar failure modes to GNSS. Specifically, eLORAN provides a much higher signal strength than GNSS and is significantly more resistant to atmospheric and man-made interference effects.  The potential risk of loss of PNT information may be acceptable for some users, such as fishermen and pleasure boat owners; it is not acceptable for merchant vessels, normally operating with tighter safety margins and higher risks.

System performance is based on the assumptions that the system provider conforms to these guidelines and that the user equipment meets the design and installation standards as specified in Section 9.

# Signal Format

- The eLORAN signal comprises two basic components:

- The Loran pulsed waveform, in which the third zero crossing point is used as the time-stamp from which timing and hence ranging from the transmitter is derived.

- The Loran data component, which is constructed by modulating the latter parts of the eLORAN waveform.

The signal construction, data format and transmission characteristics are detailed in ITU-R Recommendation M.589-2 (Ref. 1)  (to be updated)

## The eLORAN Pulsed Waveform

The eLORAN Waveform is the basis of timing and hence ranging. Each transmitter broadcasts a group of eight to or ten pulses, in a pattern synchronised with adjacent transmitters. Each pulse consists of a 100 kHz carrier that rapidly increases in amplitude in a prescribed manner and then decays at a rate which depends on the particular transmitter and antenna characteristics.

Signal Power is expressed in terms of Peak Envelope Power in kilowatts.

- The eLORAN Waveform

## Time Synchronisation

eLORAN signals should be synchronised to Universal Coordinated Time (UTC).

Time of Emission (TOE) of the eLORAN signal from a station is defined as the start of the first pulse of a pulse group.

Emission Delay (ED) is defined as the difference between the master station’s TOE and the secondary station’s TOE in the same GRI.  The definition of ED implies that the standard deviation of ED is a measure of how well a secondary station is time synchronised.

The Signal Performance Standard requires:

- The 10 second exponential average of all navigation (not modulated) shall be within 25 ns of UTC;

- The 1 second exponential average of all navigation (not modulated) shall be within 100 ns of UTC;

- The peak to peak variation of 5 second exponential average of all navigation (not modulated) shall be less than 10 ns within a 20 minute period; this is a maritime Harbour Entrance & Approaches (HEA) receiver requirement.

## The eLORAN Data Channel

One of the essential features of eLORAN is the additional functionality provided by the Loran Data Channel, which may be used to transmit:

- Time of Day (UTC), Leap Seconds;

- Differential Loran corrections for temporal variations in phase for maritime & timing users.  Differential Loran has the capability to improve accuracy for harbour entrance to 10 m (95%), but requires harbour survey to measure spatial (ASF) variations;

- Authentication messages;

- Almanacs, containing information regarding new transmitters, chains, monitors, GRI’s etc., and makes it possible to change systems without rendering previous eLORAN receivers obsolete;

- Differential GNSS corrections.

## Modulation scheme

(tbc)

# Reference Datum

The datum used should be stated in each providing Administration's publications.  WGS84 is typically used by most service providers.  It should be noted that use of the incorrect datum could result in errors of up to several hundred metres.

# Availability and Continuity

With the migration to ‘all-in-view’ receivers, the system is less dependent on the performance of each individual transmitter than was the case with earlier forms of LORAN.  However, downtime on a particular transmitter will have an adverse impact on the geometry of the local eLORAN network, resulting in a higher dilution of position (DOP) for users within the coverage area. The availability of eLORAN should be expressed in terms of individual station availability.

In order to achieve adequate system performance, operating administrations are required to establish performance and maintenance standards for all aspects of eLORAN, including:

- Electrical power generation; Use of Standby Generator equipments of an appropriate power rating (e.g. 27kW will provide sufficient power to operate a 450 kW PEP transmitter);

- Timing equipment (including atomic clocks);

- Transmitters;

- Antenna systems;

- Signal & data monitoring;

- Central control stations;

- Communications links;

- ASF maps.

IMO resolution A.953(23) provides for three standards on accuracy, availability and continuity from a radionavigation system.  This concept can be developed to represent the situation when coverage is provided by a system comprising eLORAN transmissions alone, when ASFs are applied in the Coastal Region, and when differential Loran is used to provide higher accuracy performance.  The standards, in tabular form, are:

- eLoran Performance Requirements

| Area | Absolute Horizontal Accuracy | Signal Availability | Continuity | Augmentation |
| --- | --- | --- | --- | --- |
| Ocean | 1000 m | >99.0% over 30 days | N/A | None |
| Coastal/harbour with low level of risk | 100m | >99.0% over 30 days | 99.85% over 15 minutes | Receiver ASF mapping database essential |
| Coastal/harbour with high level of risk | 20 m | >99.8% over 30 days | 99.97% over 15 minutes | Differential LORAN reference station required within a maximum of 30 Miles |

## Availability

The availability standard adopted for an eLORAN service is related to the techniques used in planning and implementing the service.

Availability is defined in IMO Resolution A.915 (22) (Ref. 4) as:

“The percentage of time that an aid, or system of aids, is performing a required function under stated conditions.  The non-availability can be caused by scheduled and/or unscheduled interruptions.”

Signal availability is defined as the availability of a radio signal in a specified coverage area.

Mathematically this can be written as:

MTBO

Availability (A)* =

MTBO + MTSR

Where:

MTBO = Mean time between outages; based on a 30 day  averaging period.

MTSR = Mean time to service restoration; based on a 30 day averaging period.

This accounts for scheduled and unscheduled service interruptions, i.e. preventative and corrective maintenance.

* Alternatively expressed as UP TIME/TOTAL TIME where TOTAL TIME = 30 days

## Continuity

Continuity, or reliability, is the ability of a system to function within specified performance limits without interruption during a specified period (normally short term).

The equation below refers to an availability type calculation, i.e. the average over the continuity time interval (CTI) that the service is available or the probability that the service is available at any instant during the CTI.  It is more appropriate to calculate the probability that the service is available throughout the CTI.  The suggested approach is given below:

Assuming the service is functioning at the beginning of the operation, then the probability that it is still functioning at a time (t) later is:

C = exp (-t/MTBF)

This is the standard expression for reliability and excludes scheduled outages (i.e. uses MTBF) assuming that planned outages will be notified and the operation will not take place. The probability that the service will be available after a time CTI, that is the continuity is then:

C = exp (-CTI/MTBF)

If MTBF is very much greater than CTI, this can be approximated to:

C = 1 – (CTI/MTBF)

Where:

MTBF = Mean time between failures; based on a 2 year averaging period.

CTI = Continuity time interval; in the case of maritime continuity, equal to 3 hours.

There is no need to include the availability at the beginning of the time period of the operation because if there is no service, then the operation will not commence.

# Integrity

“Integrity is the ability to provide users with warnings within a specified time when the system should not be used for navigation.”

The criteria used for identifying an integrity failure should be stated by the Administration in their publications.  The time for broadcasting an integrity warning to the user, from the time the integrity problem is identified, should not be greater than 10 seconds.

When it has been determined that the error condition no longer exists, there may be a delay, similar to that used for initially identifying the error, before the error warning is withdrawn.

# Accuracy

The absolute horizontal accuracy should be better than 100m at the 95% probability level within the published coverage area.  The alert level is 2.5 times the accuracy as stated in IMO A915 (22).

Where differential Loran is provided to improve the accuracy level e.g. in Port and port approach areas, the absolute horizontal accuracy should be better than 20m at the 95% probability level within the published coverage area.  The alert level is 2.5 times the accuracy as stated in IMO A915 (22).

To achieve this level of accuracy, the effective centre of the transmission antenna should be surveyed to a 2D accuracy of better than 1 metre.  The time of emission requires to be corrected for the length of transmission lines between transmitter and antenna.

# Range/Coverage

Nominal ranges of stations over seawater paths should be published at a stated signal-to-noise level (-10dB) (Ref. ITU-R M.589).  Published coverage diagrams are normally based on software modelling predictions and should be verified by measurements.  The modelling process can be quite complex and difficult, especially over mixed land/sea paths.  Advice regarding modelling can be sought through IALA. In predicting coverage, each service provider should establish the required field strength by giving consideration to the following factors:

- Radiated power - Antenna system configuration, including horizontal and vertical polar diagrams;

- Land paths - Additional attenuation over land should be calculated from current ITU-R curves and practice applicable at 100 kHz; (Refs. 4 & 5)

- Fading due to skywave propagation of the station’s signal - At night the field strength at every point in the coverage area should be not less than that specified at the nominal range for at least 95% of the time;

- Atmospheric noise - Assumed levels of atmospheric noise should be in accordance with current ITU-R data and practice applicable at 100 kHz. It is recommended that the noise level be that which is not exceeded more than 95% of the time on average throughout the year;  (Refs. 6 & 7)

- Precipitation static - In those areas where precipitation static is known to be a significant problem, an appropriate factor should be added to the atmospheric noise;

- Man-made noise - In those situations, such as harbours, where man-made noise is significant in comparison with natural noise sources, the local man-made noise level should be taken into account;

- Interference - The protection ratios to be applied are those of the ITU-R, appropriate to the type of interferer and the frequency separation.  Both groundwave and skywave propagation should be considered.

# Transmitter Performance

The performance of the LF transmitter and its antenna can be affected by weather conditions. An automatic Antenna Tuning Unit (ATU) should be used to minimise such effects.

It is normal practice to duplicate the transmitter, timing and control units and power supply.The use of Uninterrupted Power Supplies to prevent outages in the event of short term loss of power supplies is strongly recommended.

There are well-established methods for the measurement of radiated power, field strength and antenna efficiency.

# Receiver Performance

In order to utilise the full eLORAN service capability, users should operate an eLORAN receiver conforming to the technical specifications set out in IEC 61108-1, incorporating a current ASF correction database.

The performance of the LF/MF receiver and its antenna can also be affected by environmental conditions, for example precipitation static in high latitudes and atmospheric noise from electrical storms. H-field antennas are usually better in this respect, although less sensitive than E-field (whip) antennas.  Shipborne noise can also limit performance and it is important to provide good grounding for installations with E-field antennas. Mounting location on the vessel can also be important and local interference and noise sources should be identified and dealt with.

# Monitoring

On-site monitors should be provided at each transmitting station, to check the transmitted signal and the data content. Additional signal monitoring is recommended using receivers placed at sites some distance from the reference station (Far Field Monitor), but within the coverage area, to validate broadcast site RF and signal performance.  Communication lines to a central control and monitoring site may link integrity monitors.  Data may either be logged on station and downloaded periodically or passed directly to the central control site.  It is recommended that this data be archived for a period sufficient to meet local litigation requirements.

# Validation

This validation procedure is provided to ensure the operational status of an eLORAN system.

## Elements of performance

Performance of a radionavigation broadcast service is defined by five basic components: accuracy, integrity, availability, coverage and continuity.  IMO Resolution A.953(23) provides performance standards for three navigation phases: ocean, coastal/harbour with low risk and coastal/harbour with high risk.  The performance standards are based on definitions of accuracy, availability and continuity as described in this recommendation and IMO Resolution A.915(22) Maritime Policy for future GNSS systems.  The calculation of maritime DGNSS system performance parameters should take account only of maritime coverage areas.

## Availability

The entire purpose of a radionavigation service is to provide navigation quality information to enable users in the service coverage area to calculate a more precise position and make safety-of-life decisions based upon this information.  While various conditions may disrupt transmitted eLORAN signals, the net result is the same in that users lose their ability to determine their position using this service.

Availability in the coverage area can be measured (1) at the broadcast site, (2) by the use of a far-field monitor, or (3) by using a combination of these two methods, as determined by the administration.  Availability is calculated based on periods of usable and unusable times during a total period of time.  Periods of time are measured to the second.  In dual-coverage areas, the combined availability will be higher than the individual availabilities.

Administrations will have the option of defining, in their operational doctrine when the GNSS constellation is deemed inadequate.  Typically this measurement incorporates some consideration of the GNSS constellation’s dilution of precision as seen by the reference station.

Unusable conditions: The eLORAN broadcast is unusable in the following conditions (or any continuous combination of conditions):

- Signal outage: When the broadcast is not transmitted for a period greater than the momentary unusable period..

- Signal low power: When the broadcast signal falls below the prescribed levels set by the administration, e.g. by 4 dB3 for a period greater than the momentary unusable period described below.  A signal is to be considered underpowered when the low-power conditions create a gap in the prescribed coverage.

- Signal unhealthy: The broadcast health bit is sometimes set to “unhealthy” when the position threshold of the Integrity Monitor is exceeded.

- Intermittent signals: An intermittently usable signal (one that is not usable for at least 20 continuous seconds) implies unusable periods precede and follow the momentarily usable period.  Such time is not accountable as usable and shall be included in the aggregate unusable period, beginning with the initial disruption of the usable period and ending with the resumption of usable time.  If the aggregate period is greater than the momentary unusable time defined below, then this period is unusable.

Usable time is a contiguous period of healthy differential corrections and associated messages of sufficient signal strength to cover the intended service area.  A period of usable time is a minimum of 20 seconds.

Time to alarm – The user can expect to be notified within 10 seconds when an out-of-tolerance position is seen by the integrity monitor.  When the signal broadcast is interrupted, users are notified of the signal loss by the receiver and should not expect further integrity messages.

## Coverage

tbc

## Continuity

Inherent in a radionavigation service is the capability to provide accurate position fixing and integrity information without interruption.  Interruptions to eLORAN deny vital information to the users and, if frequent, erodes user confidence in the ability of the service to provide that information.  The numbers of unusable events, not the length of the usable periods, determines continuity performance.

In the event that a healthy and monitored eLORAN site begins to experience intermittent failures (i.e. failures separated in time by a period less than one continuity time interval (CTI)), the period of intermittent operation would be counted as a single failure event for continuity purposes.

Continuity in the coverage area can be measured:

- at the broadcast site;

- by the use of a far field monitor; or

- by using a combination of these methods.

Continuity is based upon the mean time between failures as measured over a two year period and a 3 hour continuity time interval.  The period of performance is not adjusted for inadequate GPS constellation, nor do GPS constellation deficiencies count as failures against continuity.

Continuity events  All unscheduled non-momentary unusable events described in the availability section are considered failures.  Unlike availability, continuity does not count scheduled maintenance events as failures.  Since public notices are provided for all scheduled maintenance events, user should be aware of such planned outages and plan the voyage accordingly.

## Control Station

A Control Station is a station to which unmanned eLORAN transmitter stations report by data communications lines.  The control station is normally manned at all times, and may control activities in more than one Administration, by mutual agreement.  Control station personnel take the actions required when initiated by error reports from the reference stations/integrity monitors.  No standards or recommendations apply to the control station at this time.

The use of remote monitoring of signal performance in the far field, and the duplication of often vulnerable communications links are recommended.

# Publication of Information

Individual Administrations are encouraged to publish service descriptions, including coverage predictions and system performance statistics; examples are given in references 18 to 23.

In addition to the information contained in the standard message types, described in Section 1, notice of current or planned signal unavailability should be provided to users through the appropriate service (e.g. coastal radio station, VTS, Navtex, Safetynet etc.)

Wherever practicable, information on scheduled and unscheduled off-air periods should be promulgated to users as follows:

- Notification of System downtime

| Scheduled maintenance | Date and expected downtime | One month in advance is recommended, but at least 1 week in advance is mandatory |
| --- | --- | --- |
| Unscheduled outages | Expected downtime | As soon as practicable and not more than 1 hour after the occurrence |

IALA Guidelines on Bilateral Agreements and Inter-Agency MOU’s on the Provision of DGNSS services in the frequency band 283.5 kHz– 325 kHz contains examples of such documents.  The examples include guidance on the information that should be exchanged between co-operating agencies and, where appropriate, the circumstances and timing for it to be exchanged.

IALA will maintain a master list of eLORAN stations on the Internet.  Input to the master list will be prepared by each Administration. IALA will provide an electronic template that should be used for initial entry of station data and assembly of the complete Administration submission.  After initial submission, Administrations will be responsible for updating their own sections of the IALA master list.  The process to incorporate changes will require each Administration to provide a complete, updated section of the list for all the eLORAN sites that they operate.  Each complete submission will appear exactly as submitted.

The file contains the following information:

- Header information:

Country, Organisation, File Name, Date last updated, and Total number of sites in the file.

- Site Information:

Each site will have the following fields of information with commas to separate each field.

# References

- International Telecommunications Union - Radiocommunication Sector (ITU-R),  Recommendation M.589-2.

- IMO Resolution A.953(23), “World-Wide Radionavigation System”, Publications Section, International Maritime Organization.

- ITU Radio Regulations Volume 1, Geneva 1998.

- ITU, “Groundwave Propagation Curves for the frequencies between 10kHz and 30MHz”. ITU-R P.368-7, Geneva 1992.

- ITU, “World Atlas of Ground Conductivities,” ITU-RP.832-2, Geneva 1999.

- CCR, “Characteristics and Applications of Atmospheric Noise Data,” ITU Report 322-3, Geneva 1990.

- ITU, “Radio Noise”, ITU-R P.372-6, Geneva 1996.

- Specification of the Transmitted Loran-C Signal, COMDTINST M 18582.4A, 1994. United States Coast Guard.

- eLORAN; Securing Positioning, Navigation and Timing for Europe’s Future, Prepared by the European eLORAN Forum, April 2008.

- Enhanced Loran (eLORAN) Definition Document, International Loran Association, 2007.

- Independent Assessment Team (IAT) Summary of Initial Findings on eLORAN, Institute for Defense Analyses, January 2009.

- Minimum Performance Standards for Marine Loran Receiving Equipment, Revision 0.6 DRAFT, April 20 2009, RTCM Special Committee 127.

- LORIPP & LORAPP reports.

- US/Canada Loran agreement.

- New Potential of Low-Frequency Radionavigation in the 21st Century, Wouter J Pelgrum, 2006.

- Definitions and Glossary of Terms used in eLORAN

Additional Secondary Factors (ASFs)  Land path factors, essentially variations in the conductivity of the earth’s surface, which cause eLORAN signals to travel more slowly over a land mass than over seawater.  The degree of retardation varies with the terrain and the type and concentration of ground cover, and cannot be predicted.  Local ASFs must therefore be determined empirically and mapped within receiver memory.

Envelope to Cycle Difference (ECD)  A measure of eLORAN signal distortion.

Group Repetition Interval (GRI)  The time inval in microseconds between the start of one transmission from the master station in an eLORAN chain and the start of the next.  Between two master transmissions, each secondary transmitter in its turn sens its signal, after waiting the period of the secondary coding delay.  The GRI also serves to identify the chain.

Accuracy  The degree of conformance between the estimated or measured parameter of a craft at a given time and its true parameter at that time.  (Parameters in this context may be position coordinates, velocity, time, angle, etc.)

- Absolute accuracy (Geodetic or Geographic accuracy)  The accuracy of a position estimate with respect to the geographic or geodetic coordinates of the Earth.

- Relative accuracy  The accuracy with which a user can determine position relative to that of another user of the same navigation system at the same time.

- Repeatable accuracy  The accuracy with which a user can return to a position whose co-ordinates have been measured at a previous time using uncorrelated measurements from the same navigation system.

Availability  The percentage of time that an aid, or system of aids, is performing a required function under stated conditions.  The non-availability can be caused by scheduled and/or unscheduled interruptions

- Signal availability  The availability of a radio signal in a specified coverage area.

- System availability  The availability of a system to a user, including signal availability and the performance of the user's receiver.

Continuity  The probability that, assuming a fault free receiver, a user will be able to determine position with specified accuracy and is able to monitor the integrity of the determined position over the (short) time interval applicable for a particular operation within a limited part of the coverage area.

Continuity Time Interval  The period of time that it normally takes to complete a manoeuvre or seek safe anchorage.  When determining Continuity, this time period is 3 hours.

Correction  The numerical value of a correction is the best estimate that can be made of the difference between the true and the measured value of a parameter. The sign is such that a correction that is to be added to an observed reading is taken as positive.

Coverage  The coverage provided by a radionavigation system is that surface area or space volume in which the signals are adequate to permit the user to determine position to a specified level of performance.

Differential system  An augmentation system whereby radionavigation signals are monitored at a known position and the corrections so determined are transmitted to users in the coverage area.

Dilution of precision  The factor by which the accuracy of the GNSS position and time co-ordinates are degraded by geometrical considerations of the constellation of GNSS satellites used by the receiver.

- Horizontal dilution of precision (HDOP).  The factor for the horizontal position accuracy.

- Time dilution of precision (TDOP).  The factor for the time accuracy.

Distance root mean square (dRMS)  The root mean square of the radial distances from the true position to the observed positions obtained from a number of trials.

Failure  The unintended termination of the ability of a system, or part of a system, to perform its required function.

Fix  A position determined by processing information from a number of navigation observations.

Fix rate  The number of fixes per unit time.

Global Navigation Satellite System (GNSS)  A worldwide position, time and velocity radio determination system comprising space, ground and user segments.  The system relates to the properties of the GNSS service plus the receiver.

Integrated Navigation System (INS)  A system in which the information from two or more navigation aids is combined in a symbiotic manner to provide an output that is superior to any one of the component aids.

Integrity  The ability to provide users with warnings within a specified time when the system should not be used for navigation.

Integrity monitoring  The process of the determination whether the system performance (or individual observations) allow use for navigation purposes.  Overall system integrity is described by three parameters: the threshold value or alert limit, the time to alarm and the integrity risk.  The output of integrity monitoring is that individual (erroneous) observations or the overall system cannot be used for navigation.

- Internal integrity monitoring is performed aboard a craft

- External integrity monitoring is provided by external stations

IMO	International Maritime Organization

ITU	International Telecommunication Union

ITU-R	ITU Radiocommunication Sector

IEC	International Electrotechnical Commission

Latency  The time lag between the navigation observations and the presented navigation solution.

MTBF	Mean time between failures.  The average time between two successive failures of a system, or part of a system.

MTBO	Mean time between outages.

MTRS	Mean time to restore system to full operation.

Navigation  The process of planning, recording and controlling the movement of a craft from one place to another.

Radionavigation  The use of radio signals to support navigation for the determination of position or direction, or for obstruction warning.

Redundancy  The existence of multiple equipment or means for accomplishing a given function in order to increase the reliability of the total system.

Repeatability  The accuracy of a positioning system, taking into account only the random errors.  The repeatability is normally expressed in a 95% probability circle.

Root mean square error (RMS)  RMS error refers to the variability of a measurement in one dimension.  In this one-dimensional case, the RMS error is also an estimate of the standard deviation of the errors.

RTCM	Radio Technical Commission for Maritime services

Threshold value (or alert limit) is the maximum allowable error in the measured position –during integrity monitoring - before an alarm is triggered.

Time to alarm  The time elapsed between the occurrence of a failure in the system and its presentation on the bridge.

True position (2D)  The error-free latitude and longitude co-ordinates in a specified geodetic datum.

World Geodetic System (WGS)  A consistent set of parameters describing the size and shape of the Earth, positions of a network of points with respect to the centre of mass of the Earth, transformations from major geodetic datums and the potential of the Earth.

- eLORAN Data Message Format

- Differential eLORAN Type 10 - Phase Corrections

| Field | Bits | # bits | Units | Range | Remark |
| --- | --- | --- | --- | --- | --- |
| Eurofix ID | 1 - 4 | 4 |  | 16 | 1010 |
| Reference Station | 5 - 14 | 10 |  | 0 - 1023 | Correction Number Identification Code 000 = Cor 1,2 001 = Cor 3,4 010 = Cor 5,6 011 = Cor 7,8 100 = Cor 9,10 101 = Cor 11,12 110 = Undefined 111 = Undefined |
| Correction Number | 15 - 17 | 3 |  | 0 - 7 | 0 = No warning 1 = Warning |
| Sky Wave Warning | 18 | 1 |  | 0 - 1 | Quality of Time Reference at Differential eLORAN 00 = Level 1: <15ns (rms) 01 = Level 2: <15-40ns (rms) 10 = Level 3: 40-200ns (rms) 11 = Level 4: >200ns |
| Time Base Quality | 19 - 20 | 2 |  | 0 - 3 | All ones indicates do not use |
| Age of Correction | 21 - 23 | 3 |  | 0 - 7 | All ones indicates do not use |
| Correction 1 | 24 - 34 | 11 | 2 ns | -2.046 – 2.046 µs |  |
| Correction 2 | 35 - 45 | 11 | 2 ns | -2.046 – 2.046 µs |  |
| Spare | 46 - 56 | 11 |  |  |  |
| CRS | 57 - 70 | 14 |  |  | Eurofix CRC |
| Total |  | 70 |  |  |  |

Administrations typically set power levels to provide greater coverage than prescribed minima, thus providing some safety margin in power output.