|  | e-NAV7/10/9 |
| --- | --- |
| Radiocommunication Study Groups |  |
| Source: Document 5B/TEMP/148 Subject: WRC-11 Agenda item 1.10 Resolution 357 (WRC-07) | Annex 25 to Document 5B/296-E |
|  | 2 June 2009 |
|  | English only |
| Annex 25 to Working Party 5B Chairman’s Report |  |
| Working document TOWARDS DRAFT CPM text on wrc-11 AGENDA ITEM 1.10 |  |

CHAPTER 1

Maritime and aeronautical issues

(Agenda item 1.10)

AGENDA ITEM 1.10

(WP 5B / (WP 5A), (WP 5C), (WP 7B), (WP 7C), (WP 7D))

1.10	to examine the frequency allocation requirements with regard to operation of safety systems for ships and ports and associated regulatory provisions, in accordance with Resolution 357 (WRC-07);

Resolution 357 (WRC-07): Consideration of regulatory provisions and spectrum allocations for use by enhanced maritime safety systems for ships and ports

### 1/1.10/1		Executive summary

[Text of the executive summary, not more that half a page]

### 1/1.10/2		Background

The global maritime community has agreed on special measures to enhance ship and cargo identification and tracking, as well as ship and port security and safety. Related ITU-R studies are:

1)	Satellite-AIS:  Additional AIS channel or channels may be required to enhance and accommodate global ship-tracking capabilities.

2)	HF-data: Advanced maritime HF data systems may be used to deliver security alerts and safety information

[3)	Cargo identification and tracking: Matters to be considered are management and identification of cargo; expedited transfer of pre-screened shipments; coordination of sensors and monitors; rapid detection of dangerous, unauthorized, or compromised shipments; and, enhanced interaction with both local and national public protection resources].

[There is a need for improved identification, tracking, and surveillance of international shipping and its cargo. Some administrations as well as the International Standards Organization (ISO) are studying the spectrum and standardization requirements for electronic seals used on freight containers to provide a more secure international transportation system, which include use of such technologies as a frequency hopping spread spectrum (FHSS), passive frequency sensors and narrow band, active frequency devices.

4)	Broadcasts of safety and security information to and from ships and ports;

5)	Monitoring the evolution of the concept of e-navigation and consequent consideration of related radio regulatory issues]

6)	VHF Data (Res. 342)

### 1/1.10/3		Summary of technical and operational studies and relevant ITU-R Recommendations

[Text of the summary of the studies]

[ITU-R studies should provide a basis for considering regulatory changes, including additional possible allocations and recommendations, designed to accommodate spectrum requirements of ship and port security, consistent with the protection of incumbent services. Additionally related studies on spectrum-efficient technology should help define applications within the maritime environment.]

[Improved satellite detection of AIS

Satellite detection of AIS messages has been requested by administrations.

Separate frequencies for satellite detection of AIS should be selected from within Appendix 18 because the tuning range of the shipborne AIS Class A is limited to these frequencies. Also, with respect to possible additional AIS frequencies, Report ITU-R M.2084 indicated that the interference environment resulting from the existing services in those bands must be taken into account in determining the feasibility of accommodating satellite AIS in any given band or channel, due to the large satellite antenna footprint that overlaps both land and sea]. [Separate operating frequencies in addition to AIS 1 and AIS 2 are needed that are not subject to terrestrial use.

Frequencies should be considered only from Appendix 18 due to the limited tuning range of the shipborne AIS.

Appendix 18 contains only 3 frequencies (channels 16, 75 and 76) that are exclusively dedicated to maritime use (channels 75 and 76 are proposed to be shared with this service). This proposal meets the intent of footnote n) to Appendix 18 for interference mitigation.

Satellite detection of the shipborne AIS should be limited to the AIS Class A (SOLAS Class) because the AIS Class B population is too large to be included.

Recommendation ITU-R M.1371-3 should be revised to add the proposed Message 27 along with its transmissions on the designated channels 75 and 76, and the AIS Class A equipment should be updated to add this message to facilitate improved satellite AIS detection].

[Editorial note: Most part of the text above should be moved from the Summary of Studies into Analysis of the Results of Studies.]

[HF-Data: If HF-data is to be considered in the background-section, then is should have a sub-heading also in this section.]

Cargo identification and tracking [Working document towards a preliminary draft new Report ITU-R M.[CARGO-IDS]*

–	ISO 10374:1991, Freight containers – Automatic identification, which provides an RF tag that encodes only the unique identification of the freight container

–	ISO 10891:CD2008, Freight containers – RF automatic identification, a revision and re-designation of ISO 10374, above, which also provides an RF tag that encodes only the unique identification of the freight container

–	ISO 17363:2007, Supply chain applications of RFID – Freight containers, which provides a data-rich RF tag for supply chain data, e.g. a manifest of contents

–	ISO 18185:2007, Freight containers – Electronic seals, which provides an electronic container seal with only the identity of the electronic seal and whether the seal has been compromised.  ISO 18185 is comprised of the following parts

•	ISO 18185-1:2007, Freight containers – Electronic seals – Part 1: Communication protocol

•	ISO 18185-2:2007, Freight containers – Electronic seals – Part 2: Application requirements

•	ISO 18185-3:2006, Freight containers – Electronic seals – Part 3: Environmental characteristics

•	ISO 18185-4:2007, Freight containers – Electronic seals – Part 4: Data protection

•	ISO 18185-5:2007, Freight containers – Electronic seals – Part 5: Physical layer

It is the position of the ocean carrier community that each freight container should be equipped with three unique tags.

1)	The ISO 10891 tag should not require battery power and should be affixed to the freight container for the life of the container.

2)	The ISO 17363 tag should prevent unauthorized persons from viewing the data content of the tag. The responsibility of the 17363 tag should be a matter between the shipper and the consignee and should not involve the carrier.

3)	The ISO 18185 tag should be a single-use tag, discarded at the conclusion of the trip and should support two unique air interfaces.]

Recommendation ITU-R M.1842

Broadcasts of safety and security information to and from ships and ports;

[If this matter is considered in the background section, then a heading is required also here.]

Monitoring the evolution of the concept of e-navigation and consequent consideration of related radio regulatory issues

[If this matter is considered in the background section, then a heading is required also here.]

### 1/1.10/4		Analysis of the results of studies

[Text of the analysis of the studies]

[NOTE – Use identical sub-headings as in previous sections]

[ITU-R Radio Regulations recognize the AIS-SART operation as having a safety function on the two AIS frequencies. Consideration should be given to the appropriate RR designation of the AIS radio service, including the AIS ship-to-ship collision avoidance function, AIS use in Vessel Traffic Services (VTS), and AIS general use as a navigational technology. Other matters which could be studied include whether additional channels for satellite detection of AIS allocated exclusively to the maritime mobile satellite service is required including assessing the impact of any changes to other systems and services.

]

Satellite-AIS

HF-data

Cargo identification and tracking

[The challenges of supporting multiple frequencies and multiple devices are those of both cost and regulation.  The cost factor is either that of providing complex RF tags or of providing a complex infrastructure of readers, since all three tags (Container Identity, eSeal, and Supply Chain Tag) must be read at the same high density transit points. A challenge exists since there is no common frequency on a worldwide basis that would permit power levels to support the application requirements of freight containers.

An ideal solution would be one where permanently installed battery-less tags could be read at distances and speeds required by the application requirements, while within the same infrastructure battery-powered tags could meet the longer distance and localization requirements of eSeals and Supply Chain Tags. This same air interface could be used for integrated Container Security Devices (CSDs) that would incorporate sensor input to a wireless infrastructure.

In May 2003 ISO TC 104 requested the ITU assist with a frequency band that would provide

–	a frequency hopping spread spectrum (FHSS), passive frequency; and,

–	a narrow band, active frequency.

The following currently in-place air interfaces could be taken into consideration:

–	433 MHz (ISO/IEC 18000-7) is an ISM band in various regions,

–	860-960 MHz (ISO/IEC 18000-6) is an ISM band in various regions, and

–	2 450 MHz (ISO/IEC 24730-2) is an ISM band in all regions.

Out of these three only the band around 2 450 MHz seems to offer some realistic possibility because of RR No. 5.150. On the other two bands the difficulties are much bigger due to lack of global availability or due to other usages of the band.

These technologies could provide for enhanced safety, security, and efficient movement of commerce.  They would also increase the utility for the handling of containers. It would be beneficial to harmonize an air-interface that is available for any radio frequency identification purpose, not only maritime.]

[Editors’ note: Member states are requested to coordinate within their respective agencies and organizations responsible for intermodal transportation, border inspection, harbour safety, port operations, and cargo tracking. International organizations are invited to identify relevant impacts of this effort.]

Broadcasts of safety and security information to and from ships and ports

Consider whether a future requirement exists for provision of information “pushed” to mariners by responsible authorities in an urgent situation, requiring more capacity than NAVTEX, SafetyNET or a voice announcement can provide, and if so, what the specifics of that requirement would likely be.

Monitoring the evolution of the concept of e-navigation and consequent consideration of related radio regulatory issues [Note: If this matter is discussed in the previous sections, then it should be discussed also here.]

### 1/1.10/5		Methods to satisfy the Agenda item

[Text of the method(s) to satisfy the Agenda item]

### 1/1.10/6		Regulatory and procedural considerations

[Example(s) of regulatory text relating to the method(s) to satisfy the Agenda item]

[NOTE – This part should include identical sub-headings as the previous sections, i.e. satellite-AIS, HF-data, cargo identification and tracking, safety and security communications and e-navigation.]

[Freight container standards and associated frequencies

|  | 433 MHz (18000-7) | 850–950 MHz* | 860–960 MHz (18000-6C) | 2 450 MHz (24730-2) | 2 400–2 500 MHz* |
| --- | --- | --- | --- | --- | --- |
| ISO 10374 |  |  |  |  |  |
| ISO 10891 |  |  |  |  |  |
| ISO 17363 |  |  |  |  |  |
| ISO 18185 |  |  |  |  |  |
| *NOTE – Columns without a parenthetical reference standard have no published or in process air interface standard and may be considered proprietary. ISO/IEC 18000-7 and ISO/IEC 24730-2 |  |  |  |  |  |

] [Article 33 of the Radio Regulations describes the operational procedures for maritime urgency and safety communications, including the transmission of maritime safety information.  Security communications and transmissions of maritime security information are not explicitly included in these operational procedures, but neither are they explicitly prohibited.

Since it can be argued that security relates to the movements and needs of ships and since certain security information may be classified as urgent, most transmissions may already meet the requirements of Article 33 and may be made over GMDSS systems.  The U.S. considers that GMDSS systems should be sufficient to handle security communications and broadcasts and that additional spectrum should not be required for that purpose, but modifications to Article 33 may be necessary.]

[Assignment of spectrum in ports necessary for the identification and security of cargo containers may improve the effectiveness and international interoperability of such systems, without causing interference to other uses of that spectrum outside of ports. Assignment of spectrum in ports necessary for the identification and security of cargo containers may improve the effectiveness and international interoperability of such systems, without causing interference to other uses of that spectrum outside of ports. Additional studies may be required to assess the current and future RFID technology for cargo container systems.  General study of emissions from RFID devices are being considered under WRC-11 Agenda item 1.22.]

[Key technical/regulatory issues to be resolved

Technical problems regarding satellite detection of AIS and the possible need for a third channel dedicated to satellite detection need to be addressed.  If the need for a third channel is required, an amendment to Recommendation ITU-R M.1371-3 will be needed to enable its operation, and studies completed at Working Party 5B to select a feasible channel.

The International Organization for Standardization Technical Committee 104 Freight Containers Subcommittee 4 Identification and Communication (ISO TC 104 SC 4) is currently working on standards for RFID devices to be attached to or associated with freight containers.  This ISO committee has requested assistance in the technical analysis of standards related to spectrum characteristics related to this technology.  Studies are required into the technical characteristics and spectrum requirements of the existing RFID container systems.

A determination through Working Party 5B studies would likely be necessary to show whether narrow-banding the VHF maritime band (RR Appendix 18) is necessary.

Further identify appropriate (RR Appendix 18) channels to support wideband VHF data transmission that are available on a global basis.]

[Annex 1

Aspects of regulatory status of AIS frequencies

IMO notes that IMO resolution MSC.74(69) requires that AIS improve the safety of navigation by assisting in the efficient navigation of ships, protection of the environment, and operation of Vessel Traffic Services (VTS), by satisfying the following functional requirements:

1)	in a ship-to-ship mode for collision avoidance;

2)	as a means for littoral States to obtain information about a ship and its cargo; and

3)	as a VTS tool, i.e. ship-to-shore (traffic management).]

[Editor’s note

IMO remarks that although these functional requirements clearly specify safety functions, the ITU Radio Regulations only recognize the AIS-SART operation as having a safety function on the two AIS frequencies (see ITU Radio Regulations, Appendix 15 (Rev.WRC-07).

IMO kindly invites ITU to study the appropriate Radio Regulation designation of the AIS frequencies (AIS1 and AIS2), considering the AIS ship-to-ship collision avoidance function, 
AIS use in Vessel Traffic Services (VTS) and AIS general use as a navigational technology.]

[Annex 2

Studies related to e-navigation

IALA suggested study topics

IALA believes that “e-Navigation” as defined by IMO and envisioned by IALA requires appropriate provisions within the Radio Regulations, and thus IALA suggests that the following topics require study by ITU-R WP 5B:

- the protection of AIS1 and AIS2 channels as distress and safety communications;

- the spectrum resources for application of the VHF data service as described in Recommendation ITU-R M.1842;

- the review of RR Appendix 18 to support evolving digital technologies in maritime communication and navigation systems. ]

[Editor’s note:

IALA noted it is considering current and future e-Navigation requirements for maritime navigation and communication systems.  IALA projected a significant shift from analog to digital communications in the VHF maritime band (Radio Regulations Appendix 18), as well as advanced AIS technologies, and modernization of the GMDSS.  IALA stated it continues to review the spectrum needs for e-Navigation which cannot be met by Appendix 18 and will provide further comments in due course; however, it is suggested that the topics listed below will need to be studied, as a matter of urgency, under WRC-11 Agenda item 1.10 by ITU-R WP 5B for consideration at WRC-11.]

[Annex 3

Modernisation of VHF in the maritime mobile service

# Discussion

Regarding the improvement of VHF in the maritime mobile service, the introduction of new data transmission would be certainly one of the main interesting topics.

Since WRC-2000 a certain number of channels have been identified inside Appendix 18 for a possible use for new digital technology. This identification was the first step of a long process, which needs to come to an end if we want the introduction of these new technologies to be successful. Recommendations ITU-R M.1842 describes various systems working as narrow band or wide band ones.

In fact the problem is not to find a new data digital system because there is more than one, all interoperable and compatible through the IP level; the real problem is to find where they could be implemented within the VHF maritime mobile service. As it is, the identification of some channels inside Appendix 18 allows the usage of narrow band systems, while the wide band systems need the combination of more than two, three or four channels. If we want to mitigate the effect of the already existing congestion of the maritime mobile service it would be more than useful that these channels were harmonised worldwide in order to avoid that each Administration may decide where it would implement such a technology.

Currently the maritime mobile service for the VHF band is comprised between 150.05 MHz and 174 MHz and is included in the mobile service which is worldwide and primary in this band. This is not completely exact because there is more generic mobile service above and under this band but there are not really relevant for the question that we study here.

It has to be recalled that the maritime mobile service is a non exclusive service except for the distress channels and its guard band. It is a fact that is quite often forgotten. The maritime community easily forgets that the maritime mobile service frequencies are shared inside the mobile service.

Presently, Appendix 18 describes how some portions of the mobile service band could be used by the maritime mobile service. It is interesting to note that Appendix 18 covers only a portion of the maritime mobile service and not in continuous way. The maritime community widely restricted the maritime mobile service by mistake to the Appendix 18.

Appendix 18 covers the bands 156, 025-162, 050 MHz despite the fact that the maritime mobile band is comprised between 150, 05-174 MHz.

The following scale graphical presentation illustrates that situation.

Figure 1

We know that Appendix 18 is congested and that any attempt to introduce a new system which will pre-empted the current analogue voice channel would create an even more congested Appendix.

For this reason it is interesting to look carefully how Appendix 18 is built. The following scale graphic gives some useful indications.

Figure 2

As we could see the Appendix 18 is formed with 3 bands: 156-157.45 MHz, 160.6-160.975 MHz and 161.475-162.05 MHz.

It is interesting to note the two gap bands (157.45-160.6 MHz and 160.975-161.475 MHz). These two bands belong to the maritime mobile service and they are within the range of Appendix 18 which means that the current equipment is fully tuneable on these two bands. The existence of these two gaps is widely forgotten.

The Appendix 18 described a list of frequencies which are worldwide harmonized for their usage. The Appendix 18 is not bounded in any way and could be extended easily (by a WRC).

These two bands could be perfect candidates for new usage of VHF for the maritime community (e.g. new digital data transmission or even for AIS satellite detection).

Example of data transmission?

A narrow band system uses a 25 kHz channel spacing, while a wide band system needs an average of 100 kHz channel spacing.

If the WRC identify, for example, a band of 400 kHz in duplex mode we could ensure that a digital system could work. For a duplex operation a spacing of 4, 6 MHz between the two legs of the frequencies is generally recognized as optimum. For this reason the two gaps but also the band just under 156 MHz and the band just over 162, 05 MHz give a lot of possibility.

It is noted that due to a lot of additional allocations to the broadcasting service over 162 MHz in some countries, the band under 156 MHz seems more practicable.

The recurrent problem in any WRC is to identify new allocations for new technologies. The problem here is completely different: we have the allocation but we do not optimize its use because it is not harmonized.

If the WRC decided to identify new channels in order to coordinate the maritime usage we would solely need to modify Appendix 18 but not the table of allocation!!!

Difficulties and hopes

The harmonization of exclusive channels, for a worldwide usage of data communications, is one of the most important elements.

Two major difficulties are expected for this harmonization:

1)	Already some Administrations did not follow RR No. 5.226 (In the bands 156-156.4875 MHz, 156.5625-156.7625 MHz, 156.8375-157.45 MHz, 160.6-160.975 MHz and 
161.475-162.05 MHz, each administration shall give priority to the maritime mobile service on only such frequencies as are assigned to stations of the maritime mobile service by the administration (see Articles 31 and 52, and Appendix 18). Together with the existing congestion of Appendix 18, it seems difficult to find a compromise in order to identify exclusive channels, within existing Appendix 18, for data transmission systems.

2)	The great utilisation of the land mobile service could not be denied and will be certainly a brake of the expansion of the Appendix 18. In the same time the Administrations have to recognize that the maritime mobile service is fully part of the mobile service and the optimum claim (around 2 X 400 kHz) seems reasonable enough to be examined. If this will be only all along the shore for a maximum distance of 40 km inside the national territory this will certainly mitigate the impact over the land mobile service.

The exclusivity for the maritime mobile service in a 40 km area along the coast inside one Administration territory for the current channels of the Appendix 18 is done every day and works satisfactorily. This solution allowed the reutilization of all the maritime channels (except distress channels) for the benefit of the land mobile community. ]

International Organization for Standardization Technical Committee 104 – Freight Containers 
(ISO TC 104) letter to ITU-R dated 21 May, 2003.

Communications and information  related to IMO SOLAS Chapter V, XI-1(Special measures to enhance maritime safety), and XI-2 (Special measures to enhance maritime security) – ISIP Code.

*	Note by the Secretariat: This subsection is not part of the draft CPM text.

See ITU-R Radio Regulations Appendix 15 (Rev.WRC-07)

Report ITU-R M.2084 “Satellite detection of automatic identification system messages”

Letter dated 21 May 2003 from International Organization for Standards to BR Director