Executive Summary

IALA World Wide Radio Navigation Plan

International Association of Marine Aids to Navigation and Lighthouse Authorities

The International Association of Marine Aids to Navigation and Lighthouse Authorities (IALA) exists to:

- harmonise standards for aids to navigation systems worldwide;

- facilitate the safe and efficient movement of shipping, and;

- enhance the protection of the maritime environment.

This IALA World Wide Radio Navigation Plan aims to build on individual National and Regional plans and identify the Radio Navigation components which will be key to the successful implementation of e-Navigation. One of the cornerstones of e-Navigation is the universal availability of robust position-fixing, navigation and timing services.

The plan provides guidance to IALA members regarding potential future developments, which will enable members to identify areas requiring resource allocation and research activity.

Contents

|  | Executive Summary | 1 |
| --- | --- | --- |
|  | Glossary of Terms | 2 |
| 1 | Introduction | 4 |
| 1.1 1.2 | General Scope and Objectives | 4 5 |
| 2 | Background to Service Provision | 6 |
| 2.1 2.2 2.3 2.4 2.5 2.6 | General Institutional Regulatory Commercial Operational Technical | 6 6 6 6 7 8 |
| 3 | The IALA World Wide Radio Navigation Plan | 9 |
| 3.1 3.2 3.3 3.4 3.5 3.6 3.7 3.8 3.9 | Overview GNSS GNSS Augmentation The Future of GNSS Terrestrial Radionavigation VTS radar Radar and racons Automatic Identification System Future Systems | 9 11 13 15 16 16 16 20 20 |
| 4 | Non-radio Position Fixing | 19 |
| 4.1 | Non-radio Navigation System | 19 |
| 5 | Delivering the Plan | 20 |
| 5.1 | Implementation | 20 |

- Introduction

  - general

The International Association of Marine Aids to Navigation and Lighthouse Authorities (IALA) was formed in 1957 as a non-government, non-profit making, technical association that provides a framework for aids to navigation authorities, manufacturers and consultants from all parts of the world to work with a common effort to:

      - harmonise standards for aids to navigation systems worldwide;

        - facilitate the safe and efficient movement of shipping, and;

        - enhance the protection of the marine environment.

The functions of IALA include, among other things:

- developing international cooperation by promoting close working relationships and assistance between members;

- collecting and circulating information on recent developments and matters of common interest;

- liaison with relevant inter-governmental, international and other organisations.  For example, the International Maritime Organization (IMO), the International Hydrographic Organisation (IHO), the Commission on Illumination (CIE), and the International Telecommunication Union (ITU);

- liaison with organisations representing the aids to navigation users;

- addressing emerging navigational technologies, hydrographic matters and vessel traffic management;

e-Navigation

e-Navigation is an International Maritime Organization (IMO) led concept based on the harmonisation of marine navigation systems and supporting shore services driven by user needs.

The working definition of e-Navigation as adopted by IMO is:

“e-Navigation is the harmonised collection, integration, exchange, presentation and analysis of maritime information onboard and ashore by electronic means to enhance berth to berth navigation and related services, for safety and security at sea and protection of the marine environment”

There are 3 key elements or strands that must first be in place before e-Navigation can be realized:

- Electronic Navigation Chart (ENC) coverage of navigational areas,

- a robust electronic position, navigation and timing system (with redundancy), and

- an agreed infrastructure of communications to link ship and shore.

Vision

The aim of IALA is:

Fostering the safe, economic and efficient movement of vessels by improvements and harmonisation of aids to navigation worldwide

while the Vision of IALA is

Safe marine navigation in a world of:

- Larger and faster ships

- Changing economy & technology

- Stringent standards

- Holistic approach (e-Navigation)

- Changing waterway use

With this in mind, IALA has taken an initiative as part of the strategy for the future of e-Navigation by developing a World Wide Radio Navigation Plan.

  - scope and objectives

The 3 key elements for e-Navigation are identified above. However, this document focuses solely on the need to provide robust electronic position, navigation and timing (PNT) information, encompassing radio navigation systems. It presents the IALA position on current, developing and future PNT systems within the maritime environment.

This document is aimed at assisting in the formulation of policy for National and Regional Radio Navigation Plans.

It is also provided for IALA members and other administrations to assist them in offering their radio navigation systems to IMO as part of the WWRNS.

The core objective of this document is to state the IALA vision for robust position, navigation and timing (PNT) within e-Navigation.

IALA will publish a separate World Wide Radio Communications Plan (WWRCP).

- Background to Service Provision

  - general

The IALA Aids to Navigation service provision mix is coming under increasing pressure. User requirements are becoming more demanding, whilst at the same time IALA members endeavour to deliver safety critical services at maximum value for money. This is in the context of growth in shipping traffic and an increasingly litigious legal environment. Radionavigation itself is also adding new levels of complexity to the service provision environment and significant change is anticipated over the next two decades.

  - institutional

Significant changes have occurred in the world wide marine institutional environment during the last five years.

In the near future, there will be a need for even closer co-operation between the International Maritime Organization (IMO), the International Hydrographic Organisation (IHO) and the International Association of Marine Aids to Navigation and Lighthouse Authorities (IALA) in pursuit of the emerging e-Navigation strategy.

- 2.3	REGULATORY

Globally, there are growing signs that an increasing number of National Administrations are ready to support further legislation to ensure maritime safety, environmental protection and security, for example:

- at IMO NAV 54 in July 2008, IMO agreed a schedule (2012-2018) for mandatory carriage of ECDIS on SOLAS vessels;

- IMO have defined an increasing number of Traffic Separation Schemes, with an IHO requirement to regularly resurvey these areas with a high degree of accuracy;

- The increasing number of multi-national data-sharing agreements, such as IMO’s LRIT, the European Union’s SafeSeaNet and the Baltic HELCOM Agreement and

- the development of Marine Electronic Highways and Motorways of the Sea in high risk areas.

This is likely to result in greater commonality in service provision between different National Administrations. It may lead to a strengthening of IMO’s role as regulatory requirements are implemented in a more stringent manner.

There are moves towards charging “market rates” for the use of radio spectrum in some countries and proposals may be put forward for adoption of this approach internationally.

- 2.4	commercial

There are key trends in the global shipping industry that are already having, and will continue to have, a large effect on the service to be delivered:

- the continued importance of the maritime sector in supporting global economic growth particularly on the Asia routes - in 2004 the world seaborne trade reached a record high of 6.76 billion tonnes and the world fleet expanded by 4.5%;

- ships are becoming larger and faster – on 1st September 2006, Maersk took delivery of the Emma Maersk.  This is 397 metres long, 56 metres wide, can carry 11000 twenty-foot containers and is capable of more than 25 knots; and

- the age profile of the world fleet – over the last decade, the average age of container ships and tankers has decreased markedly although more than 30% of the remainder of the world fleet is more than 20 years old.

- A move towards increasingly sophisticated integrated bridge equipment, relying on electronic equipment to compensate for much reduced manning levels. The cost of such equipment is reducing in comparison to the overall cost of building and operating vessels.

The pressures on IALA members’ service provision will increase as a result of these trends. The size, age and abilities of the international fleet is becoming more varied and this has a significant effect upon plans for introducing new AtoN services and equipment.

As service providers, IALA members must meet the demands of modern, state-of-the-art ships with fully integrated bridge systems as well as 30 year-old cargo ships with basic bridge and navigation equipment.

- 2.5	OPERATIONAL

There are many changes taking place in the operational environment that present new challenges including:

- IMO’s and IALA’s support of e-Navigation

- the widespread and growing reliance upon GNSS and its role underpinning navigation, situational awareness and communications for e-Navigation;

- Growing deployment of local and specific Traffic Management Schemes to meet ever more stringent requirements at higher capacity levels; and

- the balance between traditional navigation skills and the role of new technological advances such as ECDIS and Integrated Bridge Systems.

The introduction of GNSS has encouraged mariners to navigate in areas and conditions in which they would not previously have done and the introduction of e-Navigation will further change the way that ships operate.  As part of its introduction, it is essential to understand what happens when key e-Navigation components (e.g. GNSS) fail or are denied.  Getting the human factors part of this right is also critical; before adopting the technology - safety, liability, on-board training and duty of care must all be considered.

e-Navigation means that international bodies (IMO, IHO, IALA etc) must work more closely together as the concept encompasses all their areas of responsibility.

In 2008 IMO Sub-Committee on the Safety of Navigation (NAV), IMO agreed that there was a need to provide an internationally agreed alternative system for complementing the existing satellite navigation, positioning and timing services to support e-navigation and recognized that potential back up systems could be made available (IMO  NAV53 Report, para.13.24)

- 2.6	technical

Significant changes to underpinning services and systems are expected over the next two decades:

- the development and proliferation of GNSS services

- the development of additional GNSS augmentation services

- the deployment of AIS as an AtoN

- further developments of AIS or complementary technologies

- new technology radar that does not trigger existing racons

- the prospect of the international provision of eLoran.

- The evolution of e-Navigation with improved communications between stakeholders

These new systems and services provide us with an ever-increasing array of options through which to optimise service level, and minimise risk and cost.  At the same time, the need for co-ordination between IALA members, and with key stakeholders such as IMO has never been more important as we endeavour to ensure consistent levels of service provision on an international basis.

It must also be remembered that there will be a continuing need for a supporting backbone of physical AtoNs, many of which are platforms supporting multiple systems, mixing radio and visual AtoNs.

- The IALA World Wide Radio Navigation Plan

  - OVERVIEW

The aim of this section is to list the existing and emerging radio-navigation systems as well as the augmentation systems required to provide local, regional and worldwide distribution. These systems or a combination of them can be used as a source of position, navigation and timing (PNT).

Potential components of a World Wide Radio Navigation System are depicted below:

Figure 1:	 Candidate systems to provide a robust positioning, navigation and timing; The grey shadowed Systems are already identified as part of a WWRNS. It should be noted that:

- Only terrestrial/ground-based components are operated by IALA Member Administrations

- Space Based Augmentation Systems transmit via satellites, but require ground station infrastructure including reference stations

The only PNT systems that currently meet the IMO requirements as a WWRNS on a global basis are the GNSS systems. Therefore marine navigation is highly dependent on GNSS for PNT information. This provides current information regarding the position of the vessel, and also a direct input into such onboard systems as:

- Integrated Bridge Systems

- ECDIS

- ARPA / radar

- GMDSS

- AIS

- LRIT

- VDR

These inputs influence both the onboard decision-making process on each vessel and also that on other vessels and ashore by relaying the vessel information by AIS.

The shore-based marine infrastructure is also dependent on PNT information:

- position reports to VTS

- LRIT

- synchronising time slots in AIS

- synchronising flashing lights

- synchronising communication systems

Robust PNT information will become an essential foundation of e-Navigation, and requires three complementary components:

- A Core Global Navigation Satellite System (GNSS)

- Augmentation of GNSS to ensure that GNSS system performance is fit for purpose

- Adequate backup in the event of GNSS system failure

Requirements

Requirements for navigation systems are generally described as their ability to meet five core functions:

- Accuracy

- Availability

- Continuity

- Integrity

- Coverage

These functions are well described in other publications, such as the IALA Navguide. Marine requirements standards are summarised in IMO Resolutions A.953(23) World-Wide Radio-Navigation System (see Annex A), and A.915(22) Revised Maritime Policy and Requirements for a Future Global Navigation Satellite System.

A guarantee of service provenance and security, in the form of Signal Authentication, may also be a future requirement.

GNSS receiver technology is defined in the IEC 61108 series of standards and incorporates Receiver Autonomous Integrity Monitoring (RAIM).

It is anticipated that a future ‘e-Navigation’ receiver will incorporate core GNSS, augmentation and backup capability within a single unit (i.e. a multi-system receiver).

Figure 2:	Radio navigation systems regarding distribution and range

(MH to edit)

High accuracy proprietary positioning systems include developments such as Real Time Kinematic GNSS and pseudolite technology, which generally rely on line-of-sight communications links.

Vulnerability

GNSS systems are known to be very reliable when it comes to Quality of Service (QoS), but this is dependent in the longer term on the Operator’s continued investment in maintaining an adequate constellation of operational satellites. In 2000 the GLONASS system was reduced to a minimum constellation, however this has since recovered with an increase in replenishment launches and improved satellite reliability. In April 2009 the United States General Audit Office warned that the probability of having the core 24-satellite GPS constellation may fall to 0.8 (80%) over the next few years and hence have a significant impact on GPS services (Ref).

The user should also be aware of these systems’ short-term vulnerability, mainly a result of the very low received signal strength. Since the Volpe Report (Ref) focusing on GPS vulnerability, was published in 2001, the awareness of the vulnerability of all GNSS systems to both intentional and non-intentional interference has increased (Ref: IALA R-129). Temporary interruptions of GNSS are not expected to be frequent, but would have a high impact on Users.

  - GLOBAL & Regional NAVIGATION SATELLITE SYSTEMS

### GPS

The Global Positioning System (GPS) is a multi-use, space-based radio navigation system owned by the U.S. Government, and operated by the U.S. Department of Defense. GPS was put into full operation in mid 1995. There are 31 usable satellites in the constellation at present. Accuracies are well within the declared (7.8m 95%) target (Ref) and monitoring has been improved by an enhanced ground control segment. A modernization program aims to improve the accuracy and availability for all users and involves new ground stations, new satellites, and four additional navigation signals: three new civilian signals known as L2C, L5 and L1C and a new military code called M-Code. Initial Operational Capability (IOC) of the L2C code is expected in 2008, but operations with L2C cannot be expected before 2014 and Full Operational Capability for L5 is not expected before 2016. A goal of 2013 has been established for the entire program, with incentives offered to the contractors if they can complete it by 2011.

The next generation of satellites, GPS III, is already in its definition phase (timeframe to 2021).

### GLONASS

GLONASS is a former Soviet Union developed GNSS system, now owned and operated by the Russian Space Forces. Development of GLONASS began in 1976, with a goal of global coverage by 1991. Beginning in 1982, numerous launches added satellites to the system until the constellation was completed in 1995. Following the dissolution of the USSR the system was not replenished for several years and performance became degraded however this situation has now reversed and the Russian Government is committed to restoring a full system by late 2009. Recent launches have included the improved GLONASS M satellites with a second civil signal. The new GLONASS-K with a third civil signal on L3, should start launching in 2009. It will also carry differential corrections, integrity information and search and rescue functions. The future GLONASS K-M system is at the requirement definition stage and it has been announced that a code division multiple access (CDMA) signal, inter-operable with GPS will be provided.

### GALILEO

Galileo is the European civil GNSS currently under development, and is due to reach full operational capability by 2013. Galileo will be funded by the European Union. The first test-bed satellite, GIOVE A is in place and continues to broadcast signals, including navigation signals for test purposes. The second, GIOVE B, was launched in April 2008. The first launch of an operational Galileo satellite is scheduled for 2010.

The first generation of Galileo will incorporate a number of features which were not part of previous GNSS systems. These include a signal in the E1 band with the advanced Multiplex Binary Offset Carrier (MBOC) modulation, and the safety-of-life signal in the E5b band with a function similar to the L5 signal to be introduced on GPS.

### BEIDOU/COMPASS

China is currently engaged in the development of a global system complemented by a regional component. The first four BEIDOU/COMPASS satellites were GEOs (Geo-stationary Orbit); then in April 2007 the first MEO (Medium Earth Orbit) satellite was launched.

It is not yet known whether the intention is to develop a full 30 MEO constellation, or 4 GEOs + 12 MEOs, or some other combination of MEOs, GEOs and IGSOs (Inclined Geosynchronous Orbit), the GEOs and IGSOs forming the regional component. The complete system is planned to be operational in 2015.

### QZSS

Japan is developing a Quasi-Zenith Satellite System (QZSS). QZSS is based on 3 satellites in eccentric, highly inclined orbits guaranteeing always one satellite in visibility of Japan with a minimum elevation angle of 60 degrees. Each satellite will transmit 6 signals in the L-band: 3 in L1, one in E6, one in L2 and one in L5.

One of the signals aims to provide sub-meter accuracy and integrity while maintaining compatibility with SBAS. The signal in E6 aims to support a commercial service with high data rate (2 kbps). The other signals are GPS-like signals, including L2C and L1C standards. The QZSS programme currently plans the launch of one satellite, for validation purposes, by 2009.

### IRNSS

The Indian Regional Navigational Satellite System (IRNSS) will as an independent navigation system cover the Indian region with a space segment of 3 GEO satellites and 4 IGSO satellites. The inclination of the orbital plane of the IGSO satellites is low, so that all the satellites can be seen simultaneously over India. It is planned to launch the first IRNSS satellite in 2009, with system completion by 2011.

Three IRNSS services are anticipated:

• Open Service using signals in the L5 and S bands.

• Precise Positioning Service using signals in the L5 and S bands.

• Restricted Access Service using signals in the L5 band only.

The Open and Precise services target dual frequency users but it is also intended to compute and broadcast ionosphere-corrections to support single frequency users. Because of the limited coverage of the IRNSS network of reference stations the satellites will, apart from the navigation payload, also include a dedicated C-band uplink/down-link ranging payload to support precise satellite orbit determination.

Note: the respective service providers (US, Russia, EC, China, Japan and India) should be invited to provide input or comments. A table showing the bands, signal strengths and other characteristics of the different systems might be helpful.

  - GNSS AUGMENTATION

### GROUND BASED AUGMENTATION SYSTEMS (GBAS)

IALA Radiobeacon DGNSS

IALA radiobeacon DGNSS remains the internationally accepted means of providing DGNSS corrections and integrity information to maritime users. It is defined by international global standards, with regional harmonisation of frequencies via IALA and ITU.

IALA members will continue to provide radiobeacon differential GNSS services. This service will be developed in line with GNSS evolution to monitor the performance of GNSS, to provide timely integrity warnings of service degradation and provide comprehensive but cost effective augmentation in terms of accuracy.

IALA members will:

- continue to provide the IALA radiobeacon DGNSS service meeting the needs of current users;

- recapitalise the existing DGNSS infrastructure, where necessary, to avoid system obsolescence;

- review their service provision to take account of GNSS developments;

- continue to encourage and support the IMO’s acceptance of IALA radiobeacon DGNSS into the World Wide Radionavigation System (WWRNS); and

- investigate the standardisation and delivery of high precision DGNSS to support aids to navigation management, harbour approach and docking.

AIS as a GBAS (Message 17)

Automatic Identification System (AIS) is a ship to ship and ship to shore data broadcast system, operating in the VHF maritime band.  Its characteristics and capability make it a powerful tool for improving the safety of navigation and efficiency of shipping traffic management.

An AIS unit is a VHF radio transceiver capable of exchanging information such as station identity, position, course, speed, length, ship type and cargo information etc., with other ships and suitable receivers ashore.  AIS uses self-organising TDMA, with synchronisation usually provided by GPS, though other timing sources are possible and base stations can act as the timing reference.

Under the SOLAS Convention AIS became mandatory for ships on international voyages since 2004. Also many administrations established regional networks to improve marine traffic control. AIS Base Stations may have the capability of providing DGNSS corrections to onboard AIS equipment using standardised transmissions (Message 17) as described in IALA Recommendation A.124, however it is worth noting that most vessel installations do not distribute AIS-derived DGNSS information to other onboard systems.

The use of AIS Message 17 increases the number of vessels which benefit from DGNSS transmissions with respect to better accuracy and integrity in their AIS position reports.

### SPACE-BASED AUGMENTATION SYSTEMS (SBAS)

The Wide Area Augmentation System (WAAS) has been implemented by the US FAA to support the use of GPS for general and commercial aviation over the Continental United States. It was recently extended to cover parts of Mexico and Canada. At present the WAAS architecture includes 38 reference stations, 3 master stations, 4 up-link stations, 2 geostationary satellite links and 2 operational control centres. WAAS provides both differential correction services and an additional GPS-type ranging signal from each satellite.

The present European GNSS infrastructure is EGNOS, an augmentation to GPS L1 designed for safety-of-life applications (i.e. civil aviation) and which is already providing initial operational services. EGNOS is currently in its initial operational phase with discussions on-going to secure the long-term operation of the system. The system provides currently an SBAS service in the L1 band through two GEO transponders on INMARSAT satellites. A third GEO transponder on ARTEMIS is used to support system upgrades. The required replacement of the transponders during the period 2008-2011 must be planned as an integral part of the evolution of EGNOS.

EGNOS is presently fully interoperable with the current generation of WAAS and MSAS. However, these systems are already planning an evolution towards a dual-frequency capability in the L1/L5 bands.

In Japan, the Multi-Satellite Augmentation System (MSAS) is an SBAS similar to EGNOS and WAAS. MSAS is already in its initial operational capability phase with 2 GEO-links using the L1 band via dedicated satellites shared with communications and meteorological missions.

India is developing a GPS-Aided Geo Augmented Navigation system (GAGAN), which is an SBAS similar to WAAS and EGNOS. It is currently at the end of its Technology Demonstration Phase. GAGAN includes 8 reference stations, 1 mission control centre, 1 up-link station and 1 Geo link through the L1/L5 transponder on the INMARSAT 4-F1 satellite. The full operational capability has been announced for 2010 by when the system would have been extended with an additional master control centre, an additional up-link station, 2 additional GEO links (L1/L5) plus one in-orbit spare and more reference stations.

Russia is also considering an augmentation to GLONASS called SDCM (System for Differential Corrections and Monitoring), which appears to be in a preliminary phase. The concept originally diverged substantially from SBAS standards but it seems that there is an interest in converging towards a more interoperable system.

- The future of gnss

Current and Planned Evolution of Space-based GNSS Capabilities

Evolution of the GNSS infrastructure needs to be considered, because there are several opportunities for fulfilling emerging user needs.

- Provision of augmentation services to all constellations in view to improve the performance obtained with the GPS-only signals in terms of accuracy, integrity, availability or robustness.

- Provision of additional channels for supporting the dissemination of augmentation data at regional level, both for safety-of-life and mass-market applications and to service users in the civil aviation, maritime and land domains.

- Provision of additional navigation signals at higher frequencies (S, C-band) for better performance (accuracy, robustness) and better interoperability with the evolution of spectrum utilization for mobile communications.

GNSS Strategy

At some stage (post 2013) there is likely to be three or four GNSS available, providing a total of more than 100 satellites, with multiple frequencies, inherent integrity monitoring and warning. This number of satellites will make Receiver Autonomous Integrity Monitoring (RAIM) a reliable proposition. The requirement for separate (satellite or ground-based) augmentation may need to be justified at that stage and it may be appropriate to consider phasing out existing augmentation systems with those developments in mind. However, the concerns about the vulnerability of GNSS will remain and there will still be a case for alternative electronic position fixing systems and for independent verification and warning systems.

It should also be noted that current GNSS providers do not accept liability for the service they provide.

| System |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| GPS | IOC L2C |  |  |  |  | FOC L2C |  | FOC L5 |  |  |
| GLONASS | FOC |  |  |  |  |  |  |  |  |  |
| GALILEO |  |  |  |  | FOC |  |  |  |  |  |
| COMPASS |  |  |  | FOC |  |  |  |  |  |  |
| WAAS |  |  |  |  |  |  |  |  |  |  |
| EGNOS |  |  |  |  |  |  |  |  |  |  |
| eLoran |  |  |  |  |  |  |  |  |  |  |
|  | 2009 | 2010 | 2011 | 2012 | 2013 | 2014 | 2015 | 2016 | 2017 | 2018 |

  - TERRESTRIAL RADIONAVIGATION SYSTEMS

3.5.1	eLORAN/CHAYKA

GNSS (in particular GPS) has become the primary means of navigation in many maritime applications. However, the vulnerability of GNSS to accidental or deliberate interference is well known and the need for more than one position input to e-Navigation is recognised.

It is noted that Loran/Chayka is the only wide area terrestrial radio-navigation system currently available. This system generally provides coverage including and exceeding the coastal phase of navigation and is usually provided by more than one administration working in partnership.

It is also noted that most countries with Loran facilities are now committed to retaining them for the foreseeable future as an alternative, backup or complement to GNSS. In particular, the US has recently announced its intention to ’undertake development of enhanced Long Range Navigation .……as a backup for the Global Positioning System (GPS) ....as a first step toward providing back-up capability for critical infrastructure that depends on GPS for position, navigation and timing.’ (Ref)

Members of IALA with Loran/Chayka facilities within their jurisdiction are encouraged to retain them in operation and make plans to upgrade them to eLoran capability, so that they can form part of the WWRNP.  IALA also encourages its Members to give full support to the development and standardisation of eLoran, so that the system can be recognised as a component of e-Navigation. The necessary performance and technical standards are currently under development by RTCM Special Committee 127.

The key differences between existing systems and eLoran can be shown in tabular form:

| Feature | Existing Loran-C / Chayka | eLoran |
| --- | --- | --- |
| Transmitter | Tube or solid state | Solid state |
| Timing control | System Area Monitoring or Time of Emission control | Time of Emission control |
| Chain Synchronisation | Chain controlled by Master Station | Each transmitter linked to UTC |
| Data transmission | None or Eurofix | Loran data channel |
| Receiver Mode | Hyperbolic; Chain selection, manual or automatic | Ranging; All-in-view |
| Use of Additional Secondary Factor (ASF) corrections | None or internal database | Database mapped within receiver |
| Differential Loran usage in port and port approach areas | No | Yes |
| Accuracy | Nominal 400m | 8-20m where differential Loran corrections provided; <100m elsewhere |

  - VTS RADAR

VTS and Coastal Surveillance (CS) radars are of potential use, as they deliver non-cooperative target detection and tracking and do not rely on an external position fixing system. In contrast, AIS reports rely on the ship-borne AIS unit being turned on and configured correctly as well as on an external position fixing system.

In case of primary position fixing system (GNSS) failure, VTS/CS radar can provide navigational information to a vessel, for example track information on radar targets or even a complete situation image for a certain area. However, VTS does not provide an alternative timing reference.

VTS/CS centres can monitor agreement between radar and AIS data and may communicate relevant alarms (manual/automatic) when disagreement occurs in critical areas. This also includes radar targets with no corresponding AIS reports.

  - raDAR aND RAcons

Ship-borne radars: The IALA WWRNP will not address ship-borne radars but the characteristics of the radars used onboard affect the use of radar AtoNs (radar beacons or racons). Characteristics to be taken into account include the use of pulsed magnetron transmitters vs. solid state modulated pulse transmitters (part of ’New Technology’ radars), pulse compression, and use of “raw” video vs. video processing to enhance small targets in clutter. The operation of passive radar target enhancers i.e. reflectors will not be affected by a change in transmission and video processing technology.

Radar map-matching / tagging:  More sophisticated shipborne radars may incorporate a facility to locate the vessel by means of tagging of radar-conspicuous targets at known locations.  Correlation of a shoreline on radar can also be overlaid on an electronic chart display to give confidence in the position displayed on the latter, although the value of this technique varies with the radar conspicuity of the area.

Racons: Uniquely identifiable racons are highly important when navigating in low visibility conditions, in the night or under adverse weather conditions.  Racons only rely on the excitation of the ship-borne radar, while physical AIS AtoNs rely on an external position fixing system, and synthetic and virtual AIS AtoNs rely on external communication lines. However, racons are not PNT systems, since they do not provide a timing reference.

When applying solidstate transmitters, excitation range of traditional racons will diminish because of lower maximum transmitted power. Racon response will disappear (or at least be attenuated) in the pulse compression process after response reception. The development of racons with responses focussed towards NT radars would be beneficial.

  - automatic identification system

AIS can also be applied to Aids to Navigation (AtoN) to improve and enhance services provided to mariners.

A special type of Automatic Identification System (AIS) station fitted to an Aid to Navigation (AtoN AIS) can provide a positive identification of the aid. Furthermore it can:

- Provide the position and identity of floating AtoN (i.e. buoys) by transmitting current position and monitoring whether they are on station (Message 21)

- Provide real- time information for performance monitoring, including state of ‘health’ (Message 6)

- Provide additional information such as actual tidal height and local weather to surrounding ships or to a shore authority (Message 8)

The AIS service may also enhance and complement Aids to Navigation functionality by providing:

- Synthetic AIS AtoN

- Virtual AIS AtoN;

It should be noted that AIS is essentially a communications system, not a PNT system, and only provides position information obtained from another source (e.g. GPS).

  - Future systems (inc. New functionalitY of existing systems )

Future systems (R-Mode on MF beacon and AIS)

At present the two maritime systems with widespread distribution, namely the IALA MF beacon system and the AIS Services, would be candidates for modification to add R-mode functionality.

The new functionality of the Ranging-Mode is the provision of timing information from shore to ship. The shipboard radio receiver may then calculate a distance (range) to the transmitter. Using several such calculations from a number of different transmissions, the shipboard equipment is able to calculate the ship position. Coverage, geometry and interference questions would need to be investigated.

The provision of R-Mode services via MF or VHF transmissions would require the availability of an accurate non-GNSS timing source at the transmitter. This could be provided by high stability clocks at each station, however this would be expensive and it is more likely that this would be sourced from a low frequency radio time clock or eLoran.

- 4   Non-radio Position-Fixing

4.1  Non-radio navigation system (Inertial Navigation Systems)

Loss of GNSS position and time may render AIS and ECDIS unusable. Therefore it could be essential to use a backup system based on INS which is able to continue providing an electronic position with a similar level of accuracy for a specified time period.

Inertial navigation is a self-contained navigation technique in which measurements provided by accelerometers and gyroscopes are used to track the position and attitude of an object relative to a known starting point.

The performance and costs of the INS heavily depends on the different available technologies.

At the current stage even high grade INS cannot be considered as a primary backup to GNSS positioning sensors.

It is necessary to integrate INS with other sensors like speed log, magnetic sensors or with GNSS to compensate for the drift in positioning errors.

However, low-cost inertial systems are developing and it is possible that in future their performance may make them useful as a short-term backup and they may also have a role in improving integrity and position stability.

Inertial systems are already used in combination with GNSS on some vessels for specialised applications, such as hydrographic survey, where the short-term stability of position is valuable.

4.2 Terrain Referenced Navigation

Another potential source of position information is the use of an ECDIS map together with echo sounder measurements, using a map matching algorithm based on a digital terrain model (DTM) together with measurements taken from a multibeam echo sounder (MBE) or a single beam echo sounder (SBE).

The terrain navigation system (TNS) can be viewed as an independent component of the navigation system whose primary function is to provide position measurements. Such a TNS could be integrated together with the inertial navigation system (INS) and the connected GNSS receiver. In other systems it might integrate with a dead-reckoning system or serve as an independent position source. Terrain navigation can also enhance the total navigational integrity by providing a position estimate supplementary to INS and GNSS. This is of interest for instance for vessels when they operate in an area where GNSS can be subject to jamming or spoofing. TNS are currently tested and used for navigation in autonomous underwater vehicles (AUVs) and on submarines.

The terrain correlation processor can be connected to INS to run the terrain correlation. Calculated position fix and associated uncertainty is sent back to the navigation system to limit the position error drift. The terrain correlation processor runs with any sensor providing bathymetric data, for instance multibeam echo sounder, single beam echo sounder (SBE) and/or DVL.

One essential problem for the use of the TNS is that the available digital terrain models (DTM) often have poor resolution and accuracy, except for dedicated areas where accurate high resolution (1 – 10 m horizontal resolution) DTMs have been surveyed. Terrain navigation accuracy depends on algorithm characteristics, sensor accuracy, map accuracy, map resolution and not least terrain suitability. All terrain navigation algorithms need terrain variation to work at all.

Within the context of e-Navigation, it is feasible for each vessel to continuously log echo sounder information against current position information, and relay this information to a shore station. Shore authorities may then conduct ongoing automatic checks of this information in comparison with the electronic chart, with the aim of:

- Identifying erroneous information, indicative of PNT system failure or ‘spoofed’ data

- Identifying erroneous hydrographic data, or indicating areas requiring new survey.

4.3 Integration of Visual Information

An “electronic pelorus” is proposed, which would enhance the functionality of the traditional shipboard compass for taking bearings of fixed and floating objects, by capturing bearings that would then be automatically recorded for use within the electronic navigation system.

An integrated electronic pelorus must be capable of visually sighting an object and transferring a line of position (LOP) to a navigation system display.  Further functionality may include:

- The ability to transfer a LOP to an electronic navigation system display, possibly by pulling a ‘trigger’ or pressing a button.

- The ability, once an object has been sighted, to automatically cross-reference it through a graphical means with charted features, or using databases such as lists of lights, sailing directions or a world fleet database.  This might result in the cross reference being made through AIS technology so that once an object, such as a lighthouse or another vessel, has been sighted a photographic image and characteristics of the object is simultaneously displayed.

- Bearings of other vessels to assess risk of collision may be able to associate themselves” with other target tracking features from radar or AIS to offer more effective tracking and to provide information redundancy and internal confirmation and checking.

- Visual images from the pelorus may be able to be enhanced with low-light or infrared technology.

- LOPs taken with the pelorus may be recorded within the Voyage Data Recorder (VDR) or other equipment so that a master can assess the level of activity of watchkeepers.

The e-Navigation ‘Integrated Radio Navigation Device’

an integrated radio navigation device: This is a device using all available components of IMO’s World Wide Radionavigation System (WWRNS) simultaneously to provide the best electronic position fix for the ship. Those components of the WWRNS are the GNSS, satellite- and terrestrial augmentation systems as well as terrestrial backup systems. The shipboard receiver “integrates” signals from the different available WWRNS components by combining the pseudo ranges or TOE/TOA measurements of all received signals across the individual WWRNS component boundaries. Such a device would render the different existing individual shipboard radio navigation devices obsolete. Again, such a device also would not require a user interface, because the interaction with the user again is done completely at the “Data Presentation and User Interaction” level by default.

e-Navigation requires a transition from existing discrete receivers to an integrated onboard system. This could take the form of a combined receiver, a separate unit to combine a number of sensors, or an integral part of the IBS.

New IEC data sentences may be required to support e-Navigation functionality.

To derive the full benefit of the e-Navigation communications links, it is not sufficient to transmit only latitude and longitude as calculated by the onboard receiver. Each vessel can be utilised as a mobile Integrity Monitoring station, by passing data to the shore station, which may include:

- Sources of PNT in use, and weighting applied to each within the onboard PNT solution

- Pseudorange information from each SV in use

- TOA information from terrestrial navigation systems

- Differential correction data received and used

- Depth sounder data

The shore station may then use this information to cross check the lat/long information supplied by the vessel, and also identify failures  within the PNT infrastructure.

- 5	Delivering the Plan

- 5.1 	CONCLUSIONS

IALA Members are encouraged to participate in realising the robust position-fixing element of e-Navigation by:

- Contributing to the development of e-Navigation;

- Contributing to the development of GNSS and Space Based Augmentation Systems – by representing marine interests to the suppliers of these systems;

- Continuing to provide Internationally recognised DGNSS Services, by:

  - recapitalising the existing DGNSS infrastructure as required to avoid system obsolescence;

  - reviewing their service provision to take account of GNSS developments;

  - continuing to encourage and support the IMO’s acceptance of IALA radiobeacon DGNSS into the World Wide Radio Navigation System (WWRNS); and by

  - investigating the standardisation and delivery of high precision DGNSS to support aids to navigation management, harbour approach and docking;

- Seeking to provide sufficient appropriate levels of contingency support  for GNSS, including:

  - where Loran/Chayka facilities exist within their jurisdiction, to retain them in operation and make plans to upgrade them to eLoran capability;

  - giving full support to the development and standardisation of eLoran, so that the system can be recognised as a component of e-Navigation;

- Responding to Developments in Marine Radar Technology – which may require the replacement of existing radar beacon technology with units adapted to ‘New Technology’ low power radars;

- Developing AIS as an Aid to Navigation – including synthetic and virtual aids to navigation where appropriate;

- Continuing to trial and develop new systems to enhance marine navigation

- Continuing to provide short range aids to navigation as the ultimate fallback position, based on risk analysis - taking into account all potential changes in the service provision environment. This includes trends, types, volume and mix of traffic, local hazards, areas of traffic convergence/separation, environmental considerations and changes to other risk mitigation measures;

Glossary of terms

| AIS | Automatic Identification System |
| --- | --- |
| AtoN(s) | Aid(s) to Navigation |
| COSPAS | Space System for Search of Distress Vessels (Russian Federation) |
| DGNSS | Differential Global Navigation Satellite System |
| DGPS | Differential Global Positioning System |
| EC | European Commission |
| ECDIS | Electronic Chart Display Information System |
| EGNOS | European Geostationary Navigation Overlay System |
| eLoran | Enhanced Loran |
| ERNP | European Radio Navigation Plan |
| EU | European Union |
| GALILEO | European GNSS (not an acronym) |
| GBAS | Ground Based Augmentation System |
| GLONASS | Global Navigation Satellite System (Russian Federation) |
| GNSS | Global Navigation Satellite System |
| GPS | Global Positioning System (US) |
| GSA | GNSS Supervisory Authority |
| IALA | International Association of Marine Aids to Navigation and Lighthouse Authorities |
| IBS | Integrated Bridge Systems |
| IEC | International Electrotechnical Commission |
| IHO | International Hydrographic Organisation |
| IMO | International Maritime Organisation |
| ITU | International Telecommunication Union |
| LORAN | Long Range Navigation system |
| MEHRA | Marine Environmental High Risk Area |
| NELS | North-west European Loran System |
| PSSA | Particularly Sensitive Sea Area |
| RACON | RAdar BeaCON |
| RNAV | Radionavigation |
| RTE | Radar Target Enhancer |
| SARSAT | Search and Rescue Satellite-Aided Tracking |
| SBAS | Satellite Based Augmentation System |
| SOLAS | Safety of Life at Sea (IMO Convention) |
| US | United States of America |
| WWRNS | World Wide Radionavigation System |

References

IALA Navguide.

IMO Resolutions A.953(23) World-Wide Radio-Navigation System

A.915(22) Revised Maritime Policy and Requirements for a Future Global Navigation Satellite System.

IEC 61108 series

Volpe Report

R-129

European GNSS summary

GPS Signal in Space specification

IMO NAV 53 & NAV 54

Annexes:

GNSS frequency bands etc.

IMO User requirements ???

Table of systems/timescales

“Review of Maritime Transport, 2005”. United Nations Conference on Trade and Development (UNCTAD) Secretariat, United Nations, New York and Geneva, 2005. (there should be a more up to date version available)

TOE: Time of Emission (of signal); TOA: Time of Arrival (of signal)