### Maritime Information System (MIS)

### Shipborne Information Tree

### The following identification is based on the primary tasks of present and future shipborne navigation systems.

1. Route Planning/Grounding avoidance

2. Route Execution

3. Maintaining Situational Awareness

4. Operational Mode Awareness

5. Collision Avoidance

6. Emergency & SAR

7. Reporting

8. Special conditions (berthing, anchoring)

### Reference documents

### IMO NAV54/WP.2, Annex 1

IMO Res A.893(21)

SOLAS V/19

MSC Circ. 1061 Guidance For The Operational Use Of  IBS

MSC 78/11/3	Bridge Design, Equipment and Arrangements (IACS)

ISO 8468 Ship’s bridge layout and associated equipment. Requirements and guidelines

ISO 14612 Additional requirements and guidelines for centralized and integrated functions

STCW

ISM

| Task/Functions | Information Needed | Information Type | Shipborne Source | External Source |
| --- | --- | --- | --- | --- |
| 1. Route planning/Grounding avoidance [IMO Res A.893] | Route, Navigational hazards, Bathemetry, Traffic lanes, AtoN’s, Ports, etc. Under keel clearance Vertical clearance | Charted information | ECDIS/ECS Appropriate Charts/Chart database Chart updating/ECDS chart database update Routing charts Company orders | Chart provider Chart update service Proprietary Routing services (Weather, other) |
|  | Climate information Ephemeris Currents METOC forecast Ice forecast | Tide prediction Water level forecast Wind forecast Published information or databases Updates to published info Marine Information Overlays | Publications or databases - Climatology - Almanac - Tide Tables - Coastal Pilots & Sailing Directions - List of aids to navigation GMDSS | Notice to mariners Navigational warnings [NAVTEX, SAFETYNET, Radio] |
|  | Station and service ID | Published information or database | List of radio stations & services |  |
|  | Environmental/Species protection |  | GMDSS | Special notice to mariners |
|  | Ship static parameters |  | Onboard Ship design documentation |  |
|  | Ship dynamic parameters | Ship parameters | Onboard Ship design documentation |  |
|  | Ship stability condition | Visual observations Measurements Calculations |  |  |
|  | Voyage constraints | Passage schedule, shore service needs, special cargo characteristics, etc. |  |  |
| 2. Route execution & monitoring | Charted plan Update of progress relative to Charted plan. Sensor integrity/validity Time of day (HH:MM) Vessel param (squat) Ship stability update | ECDIS/ECS Appropriate Charts/Chart database Position sensors Radar fix, obs Visual fix, obs Course Speed Heading Time | Chart or ECDIS GNSS (e.g., GPS/DGPS), inertial sensors Radar Compass, binoculars, pelorus, ATON’s GNSS or calculated. SDME Gyrocompass, Mag compass GNSS, radio time broadcast, clocks Radio | ATON’s VTS instructions Coastal authorities instructions/orders |
|  | Heading/rudder & propulsion orders |  | Heading control system Heading control system (HSC) Track control system Rate of turn indicator |  |
|  | Update of set/drift (Wind, Currents) Updated under-keel clearance Updated Vertical clearance | Wind, Currents update Tide data Water level & prediction | Onboard Met instruments |  |
|  |  |  | GMDSS/NAVTEX | Navigational warnings [NAVTEX, SAFETYNET, Radio] |
|  |  |  | AIS (binary data msg) | Example: NOAA Ports info |
|  | Update of Sea state | Wind, Wave height | Visual obs |  |
|  | Bathymetry observation |  | Echo Sounder |  |
| 3. Maintaining Situational Awareness | Route monitoring Collision Avoidance |  | [See above] |  |
|  | Comparison of multiple nav aids & visual obs | Visual observation (traffic, AtonN’s, etc.) Radar Observation (traffic AtoN’s, etc. | Appropriate Charts/Chart database ECDIS/ECS Radar AIS GNSS Compass/Pelorus Electro-Optical (night vision, HSC) Speedlog Echo Sounder |  |
|  | Communication/Interaction of bridge team & pilot | Voice Visual observation | Fatigue management |  |
|  |  |  |  | Training |
| 4. Operational Mode Awareness | Integrated Navigation Systems (e.g., INS & IBS) |  | Integrated Bridge System Guidelines for |  |
|  |  |  | Integrated Navigation System |  |
|  | Personnel status |  |  |  |
| 5. Collision avoidance | Relative bearing rate Range rate Relative bearing Relative position (Course of ship) Relative speed (STW) Heading of other Heading, ownship | Visual observations | Compass/Pelorus Electro-Optical (night vision, HSC) |  |
|  |  | Radar display | Radar, Speed Log (water track) | AIS |
|  |  |  | AIS, |  |
|  |  |  | Compass, THD, TMHD |  |
|  | Visibility Condition | Visual/Radar | Radar Charts/ECDIS Bridge team |  |
|  | Condition of vessel | Visual observation | Lights/day-shapes. | AIS |
|  | Exceptions | Voice comm. | Voice VHF/ VHF-DSC (AIS safety message?) | (AIS?) |
|  | COLREG, geographical position |  | AIS(external), Radar, GNSS DGPS, ECDIS/ECS VHF radio |  |
| 6. Emergency & SAR |  |  | AIS SART |  |
|  |  |  | RADAR SART |  |
|  |  |  | Voice VHF/VHF-DSC |  |
|  |  |  | MMSC |  |
|  |  |  | Radio DF |  |
| 7. Reporting | Load Plan |  | VHF HF NAVTEX Satellite (e.g., INMARSAT) |  |
|  | Report Hazard Matl |  |  |  |
|  | Casualty |  |  |  |
|  | Port State control data |  |  |  |
|  | Ship Stability data |  |  |  |
|  | Passage Plan |  |  |  |
|  | Onboard logs |  | VDR ECDIS voyage log |  |
|  | Reporting dangers (ice, floating, hazards) | Visual observations Position Other published information | GMDSS/radio List of radio stations & services GNSS Coastal Pilots & Sailing Directions |  |
|  | Malfunctions of nav aids | Visual observations Radar observations | GMDSS /radio List of radio stations & services Radar AIS List of aids to navigation |  |
|  | Weather reporting | METOC | GMDSS /radio Shipboard weather instruments Visual observation |  |
|  | Pre-arrival notice | Ship’s MSI# Position Arrival schedule (passage plan) | GNSS |  |
| Berthing |  |  |  |  |
| Anchoring |  |  |  |  |