IALA Maritime Radio Communications Plan

Edition 1

December 2009

AISM Association Internationale de Signalisation Maritime      IALA

International Association of Marine Aids to Navigation and Lighthouse Authorities

20ter, rue Schnapper, 78100

Saint Germain en Laye, France

Telephone: +33 1 34 51 70 01  Fax:  +33 1 34 51 82 05

E-mail - iala-aism@wanadoo.fr       Internet -  www.iala-aism.org

Document Revisions

Revisions to the IALA Document are to be noted in the table prior to the issue of a revised document.

| Date | Page / Section Revised | Requirement for Revision |
| --- | --- | --- |

Table of Contents

Document Revisions	2

Table of Contents	3

Index of Tables	4

Index of Figures	4

Executive Summary	5

Glossary of Terms	6

1	Introduction	9

1.1	General	9

1.1.1	e-Navigation	9

1.1.2	Vision	9

1.2	Scope and Objectives	10

2	Background to Service Provision	10

2.1	General	10

2.1.1	Institutional	10

2.2	Regulatory	11

2.3	Commercial	11

2.4	Operational	11

2.5	Technical	11

3	The IALA Maritime Radio Communication Plan	12

3.1	Overview	12

3.2	Maritime Communication Requirements	12

3.3	General overview of existing communication system technologies	13

3.3.1	Low Frequency Band (LF)	13

3.3.2	Medium Frequency/ High Frequency Band (MF/HF)	13

3.3.3	Very High Frequency Band (VHF)	14

3.3.4	Ultra High Frequency Band / Super High Frequency Band (UHF/SHF)	16

3.4	Future development of maritime radio communications	17

3.4.1	Low Frequency Band (LF)	17

3.4.2	Medium Frequency/ High Frequency Band (MF/HF)	17

3.4.3	Very High Frequency Band (VHF)	18

3.4.4	Ultra High Frequency Band / Super High Frequency Band (UHF/SHF)	19

3.5	Overview of the current and future voice and data communication technologies	19

3.6	Vision: Automated selection process of available communication technologies	22

4	Securing the spectrum	22

4.1	Situation with respect to existing spectrum usage	22

4.2	WRC-12 agenda items	23

4.2.1	Agenda Item 1.9	23

4.2.2	Agenda Item 1.10	23

4.2.3	Agenda Item 1.14	25

4.2.4	Agenda Item 1.15	26

4.2.5	Agenda Item 1.23	26

4.3	Other WRC 2012 issues that may impact maritime communication	27

5	Summary	27

ANNEX 1	System Description	29

ANNEX 2	Maritime Frequency Allocations	35

ANNEX 3	Current and evolving wireless telecommunications technologies in the Maritime environment	40

ANNEX 4	Development of new characteristics	49

Index of Tables

Table 1	Geographical classification of e-Navigation data and voice communication techniques	20

Table 2	System Description Table	29

Table 3	Maritime Frequency Allocations	35

Table 4	Summary of spectrum requirements of new commercial data links	40

Table 5	Summary of maritime radio spectrum	42

Table 6	ITU recommendations for Maritime Radio Applications	50

Index of Figures

Figure 1	Overview of maritime communication applications by type	12

Executive Summary

The International Association of Marine Aids to Navigation and Lighthouse Authorities (IALA) exists to:

- harmonise standards for aids to navigation systems worldwide;

- facilitate the safe and efficient movement of shipping, and;

- enhance the protection of the maritime environment.

This Maritime Radio Communication Plan (MRCP) is being developed by IALA to assist in the selection of radio communication systems required to support e-Navigation.  The current status of radio communication in the Maritime Mobile Band is discussed and forms the basis of projections for future developments needed to support e-Navigation.  This MRCP could be the basis for a possible future submission to ITU as a contribution to the continuing efficiency improvements with respect to radio spectrum.  The plan provides guidance to IALA members regarding potential future developments, which will enable members to identify areas requiring resource allocation and research activity.

Glossary of Terms

AIS	Automatic Identification System

ARPA	Automatic Radar Plotting Aid

AtoN(s)	Aid(s) to Navigation

BPL	Broadband over Power Line

CDMA	Code Division Multiple Access

COMPASS	China GNSS (not an acronym)

CS	Coastal Surveillance (Radar)

DGNSS	Differential Global Navigation Satellite System

DGPS	Differential Global Positioning System

DRM	Digital Radio Mondiale

eANSI	Electronic Aids to Navigation System Information

ECDIS	Electronic Chart Display Information System

EGC	Enhanced Group Call

EGNOS	European Geostationary Navigation Overlay System

eLoran	Enhanced Loran

ELT	Emergency Locator Transmitters

ENC	Electronic Navigation Chart

EPIRB	Emergency Position-Indicating Radio Beacons

FAA	(US) Federal Aviation Authority

GAGAN	GPS-Aided Geo Augmented Navigation (System) (India)

GALILEO	European GNSS (not an acronym)

GBAS	Ground Based Augmentation System

GEO	Geo-stationary Orbit

GMDSS	Global Maritime Distress & Safety System

GLN	Global Link Network

GLONASS	Global Navigation Satellite System (Russian Federation)

GNSS	Global Navigation Satellite System

GPRS	General Packet Radio Service

GPS	Global Positioning System (US)

GSM	Global System for Mobile Communications

HF	High frequency (3 – 30 MHz)

HSDPA	High Speed Downlink Packet Access.

IALA	International Association of Marine Aids to Navigation and Lighthouse Authorities

IBS	Integrated Bridge Systems

IEC	International Electrotechnical Commission

IGSO	Inclined Geosynchronous Orbit

IHO	International Hydrographic Organisation

IMO	International Maritime Organization

INS	Inertial Navigation System

IP	Internet Protocol

IRNSS	Indian Regional Navigational Satellite System

ITU	International Telecommunication Union

LEO	Low Earth Orbit

LF	Low Frequency (30 – 300 kHz)

LORAN	Long Range Navigation system

LRIT	Long Range Identification & Tracking

MBOC	Multiplex Binary Offset Carrier

MEO	Medium Earth Orbit

MF	Medium Frequency (300 – 3 000 kHz)

MMS	Maritime Mobile Service

MRCP	Maritime Radio Communication Plan

MSAS	Multi-Satellite Augmentation System (Japan)

NBDP	Narrow Band Direct Printing

OFDM	Orthogonal Frequency Division Multiplexing

PLB	Personal Locator Beacons

PLT	Power Line Transmissions

PMR	Private Mobile Radio

PNT	Position , Navigation & Timing

QAM	Quadrature Amplitude Modulation

QPSK	Quaternary Phase Shift Keying

QZSS 	Quasi-Zenith Satellite System

RACON	RAdar BeaCON

RCC	Rescue Coordination Center

RAIM	Receiver Autonomous Integrity Monitoring

RNAV	Radionavigation

SBAS	Satellite Based Augmentation System

SDCM	System for Differential Corrections and Monitoring (Russian Federation)

SHF	Super High Frequency (3 – 30 GHz)

SMS	Short Message Service

SOLAS	Safety of Life at Sea (IMO Convention)

TDMA	Time Division Multiple Access

UHF	Ultra High Frequency (300 – 3 000 MHz)

VDR	Voyage Data Recorder

VHF	Very High Frequency (30 – 300 MHz)

VTS	Vessel Traffic Services

WAAS	Wide Area Augmentation System (US)

WiFi	Wireless Fidelity

WiMax	Worldwide Interoperability for Microwave Access

WWRNP	World Wide Radio Navigation Plan

WWRNS	World Wide Radio Navigation System

IALA MARITIME RADIO COMMUNICATIONS PLAN

# Introduction

## General

The International Association of Marine Aids to Navigation and Lighthouse Authorities (IALA) was formed in 1957 as a non-government, non-profit making, technical association that provides a framework for aids to navigation authorities, manufacturers and consultants from all parts of the world to work with a common effort to:

- harmonise standards for aids to navigation systems worldwide;

- facilitate the safe and efficient movement of shipping; and

- enhance the protection of the marine environment.

The functions of IALA include, among other things:

- developing international cooperation by promoting close working relationships and assistance between members;

- collecting and circulating information on recent developments and matters of common interest;

- liaison with relevant inter-governmental, international and other organisations.  For example, the International Maritime Organization (IMO), the International Hydrographic Organisation (IHO), the International Electrotechnical Commission (IEC), and the International Telecommunication Union (ITU);

- liaison with organisations representing the aids to navigation users;

- addressing emerging navigational technologies, hydrographic matters and vessel traffic management.

### e-Navigation

e-Navigation is an International Maritime Organization (IMO) led concept based on the harmonisation of marine navigation systems and supporting shore services driven by user needs.

The working definition of e-Navigation as adopted by IMO is :

“e-Navigation is the harmonised collection, integration, exchange, presentation and analysis of maritime information onboard and ashore by electronic means to enhance berth to berth navigation and related services, for safety and security at sea and protection of the marine environment”

There are 3 key elements or strands that must first be in place before e-Navigation can be realized:

- Electronic Navigation Chart (ENC) coverage of navigational areas;

- a robust electronic positioning, navigation and timing system (with redundancy); and

- an agreed infrastructure of communications to link ship and shore.

### Vision

The aim of IALA is:

Fostering the safe, economic and efficient movement of vessels by improvements and harmonisation of aids to navigation worldwide

while the Vision of IALA is

Safe marine navigation in a world of:

- Larger and faster ships

- Changing economy & technology

- Stringent standards

- Holistic approach (e-NAV)

- Changing waterway use

With this in mind, IALA has taken an initiative as part of the strategy for the future of e-Navigation by developing a World Wide Radio Navigation Plan supported by this IALA Maritime Radio Communication Plan.

## Scope and Objectives

The 3 key elements for e-Navigation are identified above.  However, this document focuses mainly on the need for an agreed infrastructure of communications between ships and ashore. It presents IALA’s view on current, developing and future Radio Communication Systems for the maritime sector.

e-Navigation will require appropriately designed radio communication systems for a robust and reliable service to the mariner.  This document is aimed at assisting in the formulation of policy for National and International spectrum allocation and usage.

It is also provided for IALA members and other administrations to assist them in offering their proposed use of radio spectrum as part of the on going studies at ITU.

The core objective of this document is to state the IALA vision for the efficient use of Radio Spectrum in the Maritime Mobile Service.

# Background to Service Provision

## General

The challenge facing radio communication in the maritime sector is pressure on the availability of spectrum from other users and the requirement to relay more and more information over fewer and fewer channels.

Spectrum as a finite resource is under increasing demand from all users globally. Recent trends from national licensing/regulatory authorities have been to use pricing/cost to drive efficiency improvements.

The moves from analogue to digital and from voice to data show the way forward for future radio communications. Time sharing of voice and data communications on a common channel is a recent technological advance that can be applied to e-Navigation.

Recent advances with TDMA techniques, AIS is the prime example, have shown the way to have several hundred users effectively and efficiently using only two channels to share vast amounts of data.

Selective of appropriate bandwidth given the constraints of required range, data rates and channel availability means that narrow band techniques should be considered.

### Institutional

Significant changes have occurred in the world wide marine institutional environment during the last five years:

In the near future, there will be a need for ever-closer co-operation between IMO, IHO and IALA in pursuit of the emerging e-Navigation concept.

Close co-ordination with ITU will be required to achieve the new approach in communications technology that e-Navigation will need. IALA national administrations are encouraged to get their national representatives at ITU to support the MRCP.

## Regulatory

The carriage requirement for radio communications equipment comes from IMO. This includes AIS, GMDSS, LRIT, DSC, voice communications and radar systems.

Radio Spectrum is regulated by ITU which includes not only the frequencies but the technologies and standards for the systems employed.

The MRCP is ITU and IMO compliant in meeting the needs of the maritime industry.

There are moves towards charging “market rates” for the use of radio spectrum in some countries and proposals may be put forward for adoption of this approach internationally.  GMDSS and other safety related communications should be free of charge.

## Commercial

Growth of the world fleet places greater stress on the ability of the existing radio communications systems to cope with the traffic. Voice systems that occupy a channel for the duration of the call can be viewed as an inefficient means of relaying information.  As the number of vessels increases, a saturation point will be reached where no more calls can be made.  Systems using TDMA, such as AIS, can cope with multiple simultaneous users and are much more immune to overloading due to growth in the number of participants.

The option of more spectrum in the Maritime Mobile Service is not readily available so alternatives have to be found to support future growth.

## Operational

There are many changes taking place in the operational environment that present new challenges including:

- the development and implementation of e-Navigation;

- the widespread and growing reliance upon GNSS and its role underpinning navigation, situational awareness and communications for e-Navigation;

- Growing deployment of local and specific Traffic Management Schemes to meet ever more stringent requirements at higher capacity levels; and

- the balance between traditional navigation skills and the role of new technological advances such as ECDIS and IBS.

The introduction of GNSS has enabled mariners to navigate in areas and conditions in which they would not previously have done and the introduction of e Navigation will further change the way that ships operate.  As part of its introduction, it is essential to understand what happens when key e-Navigation components (e.g. GNSS) fail or are denied.  Getting the human factors part of this right is also critical: before adopting the technology - safety, liability, on-board training and duty of care must all be considered.

e-Navigation means that international bodies (IMO, IHO, IALA etc) must work more closely together as the concept encompasses all their areas of responsibility.

## Technical

Significant changes to underpinning services and systems are expected over the next two decades:

- development of data communication systems by ITU;

- development of AIS satellite detection;

- regulatory constraints from ITU;

- spectrum sharing and efficiency;

- compatibility with existing systems and services;

- application of Wi-Fi and Wi-Max in short range (e.g. when in port);

- adding data services to current voice channels;

- Change from analogue voice to digital voice.

# The IALA Maritime Radio Communication Plan

## Overview

The maritime domain uses a wide range of communications technologies across the radio spectrum in order to support safe navigation, efficient operations and to provide commercial opportunities.  However, many of these technologies have been developed with a single application in mind.  As a result a vessel needs to carry many different forms of radio in order to be able to receive relevant data to allow it to undertake its voyage.

In the e-Navigation environment there is the opportunity to plan the maritime communications system architecture afresh.  This requires an assessment of the likely applications of communications as well as an understanding of the current radio spectrum available to the community.  The following sections consider the current radio spectrum and the use made of it and then move on to consider what applications are likely to run on top of the future communications infrastructure.  This section gives a brief summary of the existing and future Radio communication systems.

## Maritime Communication Requirements

The maritime domain uses communication links for a wide range of purposes, from essential safety of life applications through routine operational activities to commercial applications.

When establishing the communication requirements under e-Navigation it is essential to consider safety of life, operational, and commercial applications.  In order to achieve the benefits of e-Navigation as soon as possible the design of the communications architecture will need to focus on a small set of known applications with the flexibility to grow to encompass others in the future as necessary.

Safety

Safety

Operational

Operational

Commercial

Commercial



AIS position reports



AIS AtoN



Digital Selective Calling



Long Range Identification and

Tracking



Differential GNSS



NAVTEX



VTS coordination



Emergency SAR



AIS position reports



AIS AtoN



Digital Selective Calling



Long Range Identification and

Tracking



Differential GNSS



NAVTEX/Safety-Net



VTS coordination



Emergency SAR



Weather data



Ship reporting



Notifications to coastal States



Port arrival notification



IALA Maritime Information

Objects



Port & VTS surveillance feeds



Electronic chart updates



Access to vessel & equipment

manuals



Remote maintenance & service



Telemedicine



Weather data



Ship reporting



Notifications to coastal States



Port arrival notification



IALA Maritime Information

Objects



Port & VTS surveillance feeds



Electronic chart updates



Access to vessel & equipment

manuals



Remote maintenance & service



Telemedicine



Voyage orders



Commercial port services



Operational reports



Crew personal communications



Crew e

-

training



Cargo telemetry



Passenger Internet access



Point of Sale



Voyage orders



Commercial port services



Operational reports



Crew personal communications



Crew e

-

training



Cargo telemetry



Passenger Internet access



Point of Sale

Distress and Urgency alerting/calling

Distress and Urgency alerting/calling

- Overview of maritime communication applications by type

Figure 1 shows some example applications within e-Navigation broken down between the categories of essential communication application to ensure safety, important communication application for efficient operations and possible commercial communication applications

## General overview of existing communication system technologies

This section details the current technologies used for maritime communications by frequency band. Please refer to the table in Annex 1 for system details, Annex 2 for maritime spectrum allocations and Annex 4 for specific ITU technical characteristics associated with the systems described in section 3.3 and 3.4.

### Low Frequency Band (LF)

There is some use of the LF radio spectrum by the maritime community.  The Loran-C system broadcasts at 100 kHz and is used for radio navigation.

### Medium Frequency/ High Frequency Band (MF/HF)

Various uses are made of the MF/HF radio spectrum by the maritime community for communication of voice and data, in ship-ship, shore-ship and ship-shore modes of operation:

MF/HF transmissions support both general and distress related communications using DSC, NBDP, voice and data.  These communications take place across the maritime mobile service bands within 1.6-26.5 MHz as defined in Appendix 17 to the ITU Radio Regulations, whilst distress related communications are consigned to a small set of specific channels as indicated in Appendix 15 to the ITU Radio Regulations. Channel widths are typically 0.5 kHz (DSC and NBDP) and 3 kHz (voice and data).

#### Digital Selective Calling (DSC)

DSC is a technique using digital codes which enables a radio station to establish contact with, and transfer information to, another station or group of stations, for distress or general communications over medium to long range distances.  DSC is primarily used for distress alerting, urgency and safety calling within ship-to-ship, ship-to-shore and shore-to-ship prior to initiating distress, urgency and safety communications using radiotelephone or MF/HF radiotelex. DSC distress alerts, which consist of a preformatted distress message, are used to initiate emergency communications with ships and rescue co-ordination centres.  DSC is intended to eliminate the need for watch keeping on a ship's bridge or on shore to monitor continuously radio receivers on the distress and safety frequencies. Six specific MF/HF frequencies are also set aside for Digital Selective Calling (DSC) distress and safety communications, one in each communication sub-band.  DSC is an element of the Global Maritime Distress and Safety System (GMDSS).  DSC can also be used to call individual stations, groups of stations, or "all stations" in radio range.  Each DSC-equipped ship, shore station and group is assigned a unique 9-digit Maritime Mobile Service Identity (MMSI) as defined in the Recommendation ITU-R M.585.

#### Voice Communication

Various uses are made of the MF/HF radio spectrum by the maritime community for communication of voice in ship-ship, shore-ship and ship-shore modes of operation. General voice communication takes place across the band 1.6-26.5MHz. Channel widths are typically 3 kHz. Digital voice communication within the MF/HF bands is relatively new technology with limited use however growing with future technologies.

#### Data Communication

Current and emerging HF digital modulation schemes are breathing new life into data transmission in this frequency band (1.6 to 26.5 MHz). The relevant technologies are described in the Recommendation ITU-R M.1798.  The Recommendation includes two systems; System 1 and System 2.  System 1 is an HF data services modem protocol using orthogonal frequency division multiplexing (OFDM) by Globe Wireless, and uses 4/8-PSK modulation to 32 sub-carriers. System 2 is an Electronic mail system using the Pactor-III protocol, including system used by the Global Link Network (GLN) and uses QPSK modulation to 18 sub-carriers.  Both systems use 3 kHz channels for the data rate of 3 kbps or below and are IP level compatible making interoperability possible.

#### Narrowband Direct Printing (NBDP)

NBDP is a technique which automates radio signals to telegraphy.  NBDP (also known as radio telex) is FSK modulated onto HF channels of 0.5 kHz and supports low speed data transmissions (100 bps) in the maritime mobile service bands within 1.6-26.5 MHz. NBDP is an element of GMDSS and can be used as the text based distress follow-up communications and general communications between ship-to-ship, ship-to-shore and shore-to-ship especially to overcome the language difficulties.  The use of NBDP for general communication is declining and is now used for position reporting from ships and promulgation of meteorological warnings and forecasts from coast stations.

#### Telex over Radio (TOR / SITOR)

Telex over Radio (TOR) and Simplex TOR (SITOR) are used to describe a method of sending Maritime Safety Information using text communication techniques operating as FSK modulation in the MF/HF bands and having it automatically printed.  They provide for bi-directional communication using a 0.5 kHz bandwidth. This technology is nearly obsolete.

#### Navigational Telex (NAVTEX

NAVTEX is an international, automated system for instantly distributing Maritime Safety Information (MSI) such as maritime navigational warnings, weather forecasts and warnings, search and rescue notices and similar information to ships.  A small, low-cost and self-contained "smart" printing radio receiver (NAVTEX receiver) is an element of GMDSS and installed on the bridge.  Messages are broadcasted in English on 518 kHz, while 490 kHz and 4209.5 kHz are used to broadcast in English and/or local language.  The messages are coded with a header code identified by using alphabets to represent broadcasting stations, type of messages, and followed by two figures indicating the serial number of the message.  The time of broadcasting is internationally coordinated by areas (NAVAREA) to share the same frequency.

#### Differential Global Navigational Satellite System (DGNSS)

The IALA Differential GNSS coastal radio beacon network broadcasts corrections and integrity information to maritime users in the LF/MF bands (between 283.5 and 325 kHz).  Data rates can be 50, 100 or 200 bps.  This is a data broadcast system from shore to ship.

### Very High Frequency Band (VHF)

Voice communication using the maritime VHF band (156.025 to 162.025 MHz) is prevalent and the primary means of ship-shore and ship-ship communication in the domain.  It is used for distress, safety information and general communications. Channel spacing is currently 25 kHz although the use of 12.5 kHz channels is allowed by Appendix 18 of the Radio Regulations in accordance with Recommendation ITU-R M.1084 to improve spectrum efficiency.

#### Digital Selective Calling (DSC)

DSC is a technique using digital codes which enables a radio station to establish contact with, and transfer information to, another station or group of stations, for distress or general communications using channel 70 (156.525 MHz).  DSC is primarily for distress alerting, urgency and safety calling within ship-to-ship, ship-to-shore and shore-to-ship prior to initiating distress, urgency and safety communications using radiotelephone.  Additionally, DSC may be used for AIS channel management in specific geographic areas.  DSC distress alerts, which consist of a preformatted distress message, are used to initiate emergency communications with ships and rescue coordination centres.  DSC is intended to eliminate the need for watch keeping on a ship's bridge or on shore to monitor continuously radio receivers on the distress and safety channels.  DSC is an element of the Global Maritime Distress and Safety System (GMDSS). DSC can also be used to call individual stations, groups of stations, or "all stations" in radio range. Each DSC-equipped ship, shore station and group is assigned a unique 9-digit Maritime Mobile Service Identity (MMSI) as defined in the Recommendation ITU-R M.585.

#### Voice communication

Voice communication using the maritime VHF band (156.025 to 162.025 MHz) is prevalent and the primary means of ship-shore and ship-ship communication in the domain. It is used for distress, safety and general communications.  Primary channels used for distress and safety communications are Ch 6, Ch 13, Ch 16 and Ch 70.  Channel spacing is currently 25 kHz although there has been encouraged by Appendix 18 to the radio Regulations to use 12.5 kHz channels identified in the Recommendation ITU-R M.1084.

#### 121.5MHz Locating Beacon

The frequency 121.5 MHz is an aeronautical emergency frequency. 121.5 MHz radiobeacons were developed in the mid-seventies for installation on aircraft, as Emergency Locator Transmitters (ELTs). However, they can also be used on board ship as Emergency Position-Indicating Radio Beacons (EPIRBs) or as Personal Locator Beacons (PLBs).

#### Automatic Identification System (AIS)

##### Primary purpose of Automatic Identification System

AIS is a TDMA based data exchange system used by ships and shore authorities. The main purpose for ships is collision avoidance and to assist safety of navigation. Littoral states use AIS for the identification and locating of vessels, to obtain information about a ship and its cargo and as a VTS tool. AIS provides a means for ships to electronically exchange ship data including: identification, position, course, and speed, with other nearby ships and shore stations. This information can be displayed on a screen display.  AIS is intended to assist the vessel's watch standing officers and allow maritime authorities to track and monitor vessel movements. AIS uses VHF Channels AIS 1 (161.975 MHz) and AIS 2 (162.025 MHz) or regional channels in defined geographical areas. Additionally, AIS has the capability for data exchange by application specific messages for navigation and safety related purposes.  Due to capacity concerns, the use of application specific messages is currently limited.

##### Automatic Identification System Aids to Navigation (AIS AtoN)

AIS is also used on Aids to Navigation for the broadcast of navigation information, meteorological and hydrological data and binary messages.

##### Automatic Identification System Search and Rescue Transmitter (AIS-SART)

AIS-SART is a locating device (alternative to radar SART). As an element of GMDSS, AIS SART is used to locate survival craft and distressed vessels.  The AIS-SART has no receiver and operates up to 96 hours on a primary battery. The position and time synchronization of the AIS-SART is derived from a built in GNSS receiver.

#### Two-way VHF Radiotelephone Apparatus

Two-way VHF radiotelephone apparatus is an element of GMDSS.  The apparatus is either a portable or fixed transceiver for use in survival craft. It is used for on scene communications between survival craft, to other vessels and rescue units. Communication is on Ch16 (156.8 MHz) and at least one other channel. The transceiver can be used for onboard communications using a secondary battery; however the primary battery must be used for GMDSS purposes.

#### Regional Data Communication Systems

In various regions VHF data communication systems exists for data exchange between shore to ship and ship to shore. Those systems are commercial and used primarily for vessel tracking, search areas in SAR operation, etc.   An example of this technology is the Blue Box used in Italy for vessel monitoring. A further example is a Norwegian system that deploys a network of radio modems capable of switching between nine narrowband duplex VHF channels and is used for general data communication.

#### Satellite Data Communication

Satellite communications in the VHF band are commercially provided services used for SMS, weather and tracking. An example of such a satellite system is Orbcomm.

### Ultra High Frequency Band / Super High Frequency Band (UHF/SHF)

Various uses are made of the UHF radio spectrum by the maritime community for communication of voice and data, in ship-ship, shore-ship and ship-shore modes of operation:

#### Emergency Position Indicating Radio Beacon (EPIRB)

Distress radio beacons, also known as emergency beacons, are tracking transmitters which aid in the detection and location of boats, aircraft, and people in distress. They are radio beacons that interface with COSPAS/SARSAT, the international satellite system for search and rescue (SAR). When activated, such beacons send out a distress alert signal that, when detected by non-geostationary satellites, can be located by the combination of doppler shift measurement and triangulation.

An Emergency Position-Indicating Radio Beacon (EPIRB) is an element of GMDSS and used to alert distress signal to the COSPAS/SARSAT satellite system for the purposes of notifying Search and Rescue (SAR) organisations. EPIRBs transmit a 144-bit message including 49 bits of identification plus optionally position information periodically (approximately 50 seconds interval) to the satellites at 406 MHz on a 170 kHz wide channel (actually centred at 406.05 MHz).

The EIPB is also equipped with 121.5MHz beacon transmitter for homing by SAR aircraft.

#### On board Communication

UHF radios are commonly used onboard vessels for internal communications and for communications with workers on the dock or berth when alongside. These radios are typically constrained to radiating less than 2W on 450-470 MHz band and are for voice communication only.

#### Satellite Voice and Data Communication

Satellite communications in the UHF band are commonly deployed on vessels to fulfil a number of distress, safety and general communications purposes.

Satellite communication links are capable of supporting for example digital voice, e-mail, SMS, crew calling, telex, facsimile, remote monitoring, tracking (position reporting), chart and weather updates and FleetNET services.

Satellite solutions cover a wide range of capacity and coverage but also drive costs.  Satellite systems are commercially provided services which may have global or regional coverage. This systems may be geostationary or non geostationary.

Inmarsat, a geostationary satellite system, is an element of GMDSS for distress alerting, urgency and safety callings. An example of non geostationary satellite system is Iridium and Orbcomm.

#### Enhanced Group Call (EGC)

The Inmarsat-C maritime mobile satellite system is an element of GMDSS and has an inherent capability, known as Enhanced Group Call (EGC), which allows broadcast messages to be made to selected groups of ship stations located anywhere within a satellite's coverage. Four geostationary satellites provide near worldwide coverage for SafetyNET. SafetyNET, along with NAVTEX, is recognized by the GMDSS as the primary means for disseminating maritime safety information. Ships subject to the Safety of Life at Sea Convention travelling outside areas covered by NAVTEX must carry an Inmarsat-C SafetyNET receiver.

#### Long Range Identification and Tracking (LRIT)

LRIT is an IMO mandated scheme through which all passenger ships, cargo ships and mobile offshore drilling units engaged on international voyages must report their position on a regular basis (at least 4 times a day) to their flag administration. The data from the vessel can be augmented with additional information by the shore based authorities. Other States may be entitled to request this information from the flag administration.

#### Global Navigational Satellite System (GNSS)

Global Navigation Satellite Systems (GNSS) are used for positioning, navigation and timing (PNT) and as an essential input into other ship systems. GNSS may also be used for short message communication. Current signals are in the frequency-band around 1 GHz to 2.5 GHz.

#### X-band and S-band radar, radar beacon (RACON) and radar SART

Radar systems are commonly deployed and typically operate in two bands: X-band from 9.2 GHz to 9.5 GHz and S-band from 2.9 GHz to 3.1 GHz. The radars are used for target identification and for coastal and port navigation. These bands are also used by radar transponders, namely the RACON and radar Search and Rescue Transponder (SART) both create identifiable patterns when interrogated by vessel radars. Racons are used to highlight the location of a visual Aid to Navigation (AtoN) or hazards on a mariner’s radar screen and a radar SART is used to aid in locating a casualty in a SAR operation. RACONs operate in S- and X-band and SARTs are X-band.

The radar-SART is a locating device (alternative to AIS-SART) as an element of GMDSS and used to locate a survival craft or distressed vessel by creating a series of dots on a rescuing ship's radar display. A radar-SART will only respond to a 9 GHz X-band (3 cm wavelength) radar.

#### Other communication technologies using the UHF/SHF band

Various other communication technologies in the UHF to SHF bands are being used (or considered) for general maritime communications, namely GSM/GPRS, 3G, 4G, WiFi, WiMax and ZigBee links. These offer the possibility of data transfer up to Mbytes/second. However, it should be noted that the coverage of most of these systems is limited in range and they would therefore be confined to supporting data transfer within a port or harbour environment.

## Future development of maritime radio communications

This section details future technologies or technologies not widely used for maritime radiion communications by frequency band. Please refer to the table in Annex 3 for technical details.

### Low Frequency Band (LF)

#### Enhanced Long Range Aid to Navigation (eLORAN)

In the future the proposed enhanced Loran (eLoran) system might provide a data channel modulated onto the approximately 100 kHz signals. Two formats for this data channel are currently available, known as Eurofix and 9th pulse respectively. Both techniques offer data rates below 100 bps although higher rate concepts have been proposed.

### Medium Frequency/ High Frequency Band (MF/HF)

#### Digital data communication using MF/HF band

The likely increase in ship traffic in polar regions, as a result of receding ice fields may also create new potential for HF communications, since geostationary satellites do not cover these areas. Globe Wireless is studying increased data rates using 3 kHz channels which may produce data rates of 19.2 kbps.

Furthermore, a new system proposed by France is under consideration in ITU-R. In order to implement data communications in the maritime mobile service HF bands, the Appendix 17 to the Radio Regulations would have to be revised in World Radiocommunication Conference 2012.

Whilst not currently under consideration for maritime services, broadcasters have developed a digital HF (and MF/LF) technology known as Digital Radio Mondiale (DRM) which can offer data rates in excess of 20 kbps in a 10 kHz channel.  Such techniques may offer additional future capability.

### Very High Frequency Band (VHF)

#### Automatic Identification System (AIS)

It is recognized that by design AIS is not an ideal candidate for high speed and/or high volume data communications. High speed digital data communications for e-Navigation is better handled using multiple 25KHz channels. However, AIS is a proven maritime data system, with ships equipped and shore infrastructure established. Therefore AIS is the quickest path for handling the increasing data exchange needs that will be required to support e-Navigation.

The future AIS channel plan should consider at least the following:

- AIS channels 1 and 2 for Safety of Navigation purposes

- The AIS-1 and AIS-2 are internationally allocated with sharing basis.  These frequencies should be retained and protected for safety of navigation purposes.

- AIS channels 3 and 4 for Satellite detection of AIS and future GMDSS system

- Since the satellites have very wide footprint, the frequencies for satellite detection of AIS should be exclusive for maritime mobile service.  The preliminary draft report ITU-R M.[SAT-AIS] proposes to use CH75 and CH76.  These channels are guard band channels of CH16 and are only maritime dedicated channels except CH16 and CH70.

- In the future GMDSS system, the AIS technologies may be the basis.  The distress alerting, urgency and safety communications should be by both terrestrial and satellite communications; therefore dedicated maritime frequencies are needed. The satellite detection of AIS is one way system (from earth to satellite); however in the GMDSS, the acknowledgement (from satellite to earth) is essential as a two way system.  The CH75 and CH76 could be used for this purpose.

- AIS channels 5 and 6 for Data communication purposes

- In considering the future AIS system for terrestrial (non satellite) data communication purposes, additional two frequencies will be needed.  These frequencies would be used to support the transmission of area warnings and advice, meteorological and hydrographical data, traffic management information and general ship-shore data exchanges.

- Channel 70 for Channel Management purposes

- In order to fully use available VHF spectrum there must be a worldwide channel dedicated to manage and coordinate usage of the VHF Data Link.

- TDMA or other protocols may share Channel 70 with minor changes to Appendix 18, footnote j) according to ITU-R M.822-1.

#### Digital data communication

The use of VHF for digital data transfer has yet to be broadly implemented by the maritime community.

A shore based network of digital transceivers has been trialled in Norway. This system deploys a network of radio modems capable of switching between nine narrowband duplex VHF channels in the maritime mobile band or else these nine channels can be combined into one 200 kHz wideband channel. The wideband radio has proved to be not sufficiently stable and the throughput slowed down when the signal strength is low.

Narrowband radio 25 kHz duplex on the other hand has proved to be very robust and stable and can performa variety of service and gives a good throughput of data.

IALA is aware other investigations are on-going into other techniques to be applied more widely, from 25 kHz bandwidth systems providing 43 kbps data rates through to 100 kHz systems offering 307 kbps. Further consideration of how best to achieve spectral efficiency in this band is worthwhile and needs to address coverage range, robustness, channel spacing, modulation schemes.

The data speed/coverage trade off in the coastal range must be considered to achieve a cost effective service.

#### Digital voice communication

Digital voice communication may eventually replace the present analogue VHF voice communication service, i.e. ship-ship/ship-shore/shore-ship.

### Ultra High Frequency Band / Super High Frequency Band (UHF/SHF)

#### Satellite communication using the UHF/SHF band

In the future, navigational satellites’ payloads may include transponders connected with GMDSS and may function as additional SAR resources.

#### Public mobile wireless communications

Public mobile wireless communications such as GPRS, CDMA and 3G are already being used by mariners in coastal waters and could be further developed to support evolving maritime communication needs.

## Overview of the current and future voice and data communication technologies

As explained above, radio communication technologies are characterised by performance parameters. These include range, bandwidth, latency, and the need or not for shore facilities.

The tables below list, in the column headings, the communications techniques which are presently available for maritime voice and data communications. It also includes some techniques which may become available in the short to medium term. Separate tables are provided for ship-shore and for ship-ship communications.

Six geographical regions are listed at the left column, defined by the ranges of a selection from the available communications technologies. The GMDSS sea areas which approximately correspond are also noted in the second column.

The suggested most likely candidates for voice and data communications, for e-Navigation, in each of the six geographical areas are indicated by yellow colour, with “E” for existing technologies, and “F” for future technologies or technologies currently not widely used in the maritime domain.

- Geographical classification of e-Navigation data and voice communication techniques

## Vision: Automated selection process of available communication technologies

The tables above identify distinct communication technologies but don’t consider the optimisation that could be achieved through an automated selection process.

Some existing marine communication technologies, such as AIS, automatically carry out data communications without any need for the ship’s crew to intervene in the communications process. (In the specific case of AIS, the crew must set certain voyage parameters at the start of each voyage, but the communications process is automatic.) Other existing marine communications technologies, such as VHF voice communications, require the ship’s crew to adjust the communications apparatus, in the case of VHF voice this involves deciding on and selecting the appropriate communications channel.

An ideal e-Navigation communications system would operate automatically, selecting the best communications technology, channel, and characteristics in accordance with the ship’s location, and the type of data to be exchanged.  This automatic process would be managed in accordance with rules and the needs of the mariner which might include the following:

- Need to avoid latency – such as when exchanging safety and navigation data with other vessels or receiving it from electronic systems ashore or on the water surface;

- Ability to delay the sending or receipt of data – such as the sending of non-urgent administrative data, or the receipt of chart corrections for a subsequent voyage or for a port to be visited later in the present voyage;

- Priority message handling – such as distress, urgency and safety related messages sending from own ships and  receiving from another ships or shore in GMDSS;

- Cost of communications technology;

- Whether the communications cost is recovered;

- Importance of the data for commercial services on board;

- Rate of data transfer (capacity of transmission);

Some of the rules managing this seamless data communications would be programmed into the system, for example those concerned with safety of navigation.  Other rules might be set by the ship’s operator in accordance with house rules and the operator’s commercial model.  Others might be adjusted from time to time by the ship’s crew. Such an automatic seamless management of e-Navigation communications appears to be essential in the future.

The ship’s crew would have programmed its requirements at the start of the voyage.

# Securing the spectrum

## Situation with respect to existing spectrum usage

Section 3.3 described the existing maritime technologies which use the radio spectrum and indicated the bands in which they operate.  Whilst there is some ongoing consideration of variations to the technologies which may be used, all such variations take, as their base assumption, the ongoing use of existing spectrum allocations.  These variations may give rise to a need to change the channelisation within certain bands (this is discussed in more detail in section 4.2 as it is one of the agenda items for the forthcoming world radio conference); however there are no indications, at present, that any additional or any different spectrum is required.

Please refer to the table in Annex 1 for system details, Annex 2 for maritime spectrum allocations and Annex 4 for specific ITU technical characteristics associated with the systems described in section 3.3 and 3.4.

## WRC-12 agenda items

A number of the agenda items for the 2012 World Radio Conference (WRC-12) have the direct or indirect potential to affect maritime use of the spectrum, namely:

### Agenda Item 1.9

Agenda Item 1.9:  “to revise frequencies and channelling arrangements of Appendix 17 to the Radio Regulations, in accordance with Resolution 351 (Rev.WRC 07), in order to implement new digital technologies for the maritime mobile service”.

Description:  Appendix 17 of the ITU Radio Regulations provides a comprehensive channelling arrangement for maritime HF communications, based on the use of either single side-band (SSB), Morse telegraphy or NBDP transmissions (with channel spacing of 3 kHz, 0.5 kHz and 0.5 kHz respectively).  In recognition of the development of digital HF communication technologies and the reduction in use of frequencies for Morse telegraphy, Resolution 351 seeks to review these channelling arrangements in order to enable the use of digital technologies whilst protecting GMDSS.

As Morse telegraphy and NBDP use declines, it would therefore appear to make sense to re-assign some of these frequencies for additional 3 kHz channels, and also modify the Appendix 17 rules to permit the combining of (adjacent) channels to permit wider bandwidth transmissions.

IALA Position:  IALA supports the frequency allocation for new digital communication in the Morse and NDBP bands within Appendix 17. New allocation shall not cause harmful interference to the GMDSS function.

### Agenda Item 1.10

Agenda item 1.10:  to examine the frequency allocation requirements with regard to operation of safety systems for ships and ports and the related regulatory provisions, in accordance with Resolution 357 (WRC-07);

Description:  Resolution 357 (WRC-07) resolves:

1.  that WRC-11 consider amendments to provisions of the Radio Regulations necessary to provide for the operation of ship and port security and maritime safety systems;

2.  that WRC-11 consider additional allocations to the maritime mobile service below 1 GHz to support the requirements identified in resolves 1;

3.  that WRC-11 consider additional allocations to the maritime mobile-satellite service in frequency bands allocated to the maritime mobile service between 156 and 162.025 MHz to support the requirements identified in resolves 1,

The following issues need to be considered under this agenda item:

- matters related to Automatic Identification System (AIS);

- matters related to security communications and the broadcasts of security information to and from ships;

- spectrum requirements for identification and security of cargo containers entering and leaving international ports and ships; and

- modernization of shipboard and port safety and security communication systems including e-navigation.

1)	Matters related to AIS

International Maritime Organization (IMO) Resolution MSC 74(69) required that AIS “improve the safety of navigation by assisting in the efficient navigation of ships, protection of the environment, and operation of Vessel Traffic Services (VTS), by satisfying the following functional requirements:

- in a ship-to-ship mode for collision avoidance;

- as a means for littoral States to obtain information about a ship and its cargo; and

- as a VTS tool, i.e. ship-to-shore (traffic management).”

Although these IMO functional requirements clearly specify safety and surveillance functions, the ITU-R Radio Regulations only recognize the AIS-SART operation as having a safety function on the two AIS frequencies.

IALA Position:

  - Designate additional worldwide channels for satellite detection of AIS allocated exclusively to the maritime mobile satellite service.

  - Designate additional AIS channels as exclusive for “Safety of Communications”.

  - Protect existing AIS channels (AIS 1 and AIS 2) as exclusive for “Safety of Communications”

2)	Matters related to security communications and the broadcast of security information to and from ships

Article 33 of the Radio Regulations describes the operational procedures for maritime urgency and safety communications, including the transmission of maritime safety information.Security communications and transmissions of maritime security information are not explicitly included in these operational procedures, but neither are they explicitly prohibited.  Since it can be argued that security relates to the movements and needs of ships and since certain security information may be classified as urgent, most transmissions may already meet the requirements of Article 33 and may be made over GMDSS systems.

IALA Position:

  - Allocate the 495 - 505 kHz band for the maritime mobile service to support NAVTEX, Data, and Voice communications.  Oppose the amateur allocation in the 415 - 526.5 kHz band under Agenda Item 1.23.

3)	Spectrum requirements for identification and security of cargo containers entering and leaving international ports and ships

The large and increasing numbers of cargo containers entering and leaving international ports and the growing need to identify and maintain the security of these containers from port of origination to port of destination requires technology using radio spectrum consistently available and protected in any port accepting cargo containers in any country.  ISO TC104 is developing standards for RFID devices used on cargo containers for this purpose.

IALA Position:

  - Follow future developments and studies in ITU to determine IALA position.

4)	Modernization of shipboard and port safety and security communication systems including e-navigation

The IMO COMSAR and NAV Subcommittees are reviewing technology which may require amendments to the Radio Regulations and possibly new allocations of spectrum.  For example COMSAR 12 was in favour of leaving options for the development of VHF radios systems and technology open for further discussions, including but not limited to retention of FM voice communications, possibly using 12.5 kHz channel spacing; narrow band digital voice and data communication using 6.25 kHz channel spacing; and broadband data communications using two or more 25 kHz adjacent channels, using digital data technologies in infrastructure mode or in direct mode.  The 100 and 1 200 baud data limitations of 518 kHz NAVTEX and Inmarsat C SafetyNET respectively have long been a complaint by certain users and providers of maritime safety information, and is likely inadequate for the evolving need for graphical navigation as well as increasing needs for meteorological, search and rescue and security information, particularly information intended for presentation on integrated shipboard navigational display systems.  Provision of such information on demand by users, provided only when the ship operator desires it should be available without the need to amend the Radio Regulations, but provision of information to mariners by responsible authorities in an urgent situation, such as a storm warning or a closed port, would likely require a Radio Regulation amendment if more data than can be provided by NAVTEX, SafetyNET or a voice announcement is necessary.

General data communications over VHF requires spectrum and studies are being undertaken although it is not clear at present what the specific requirements are. Consideration of bandwidth, coverage and data rate is needed to determine the most efficient use of spectrum.

IALA Position:

  - Designate Appendix 18 VHF Public Correspondence frequencies for e-Navigation service requirements.

### Agenda Item 1.14

Agenda Item 1.14: “to consider requirements for new applications in the radiolocation service and review allocations or regulatory provisions for implementation of the radiolocation service in the range 30 - 300 MHz, in accordance with Resolution 611 (WRC 07)”

Description: There is a recognised need to provide spectrum for radiolocation services used, in particular, to track the launch and manoeuvring of spacecraft, detecting space debris and for new aeronautical surveillance services.  Various proposals were put to WRC-07 for 2 MHz of spectrum in the frequency range 30 – 300 MHz, and in particular in the range 138 – 173 MHz (with a particular focus on 154 - 156 MHz) for such services, leading to this Agenda item for the 2012 conference.

Resolution 611 which invites the ITU to investigate this issue, specifically addresses the need to avoid the emergency, AIS and some other VHF maritime mobile channels.  Resolve 2 of the Resolution states:

“that the introduction of new systems in the radiolocation service shall be avoided in the frequency bands 156.4875-156.8375 MHz and 161.9625-162.0375 MHz”

It should be noted that wider maritime VHF usage is not being specifically identified for protection.  The resolution may not afford sufficient protection for the introduction of additional AIS channels.

There is potential for the proposed new services to cause interference to maritime VHF usage over a wide area, even out to sea away from land and it would therefore be prudent to take a watching interest in the work of the group on this matter in order to ensure protection of maritime services.  Any technical studies conducted in order to analyse compatibility with existing services should include suitable compatibility criteria for maritime services. The frequency range 142 to 144 MHz is the most suitable candidate band, which would have minimal impact on maritime mobile use.

IALA Position: New allocations to the radiolocation services shall not be allowed in the VHF Maritime Mobile Band and shall not cause harmful interference to the VHF Maritime Mobile Band even in situations of abnormal propagation conditions.

### Agenda Item 1.15

Agenda Item 1.15: “to consider possible allocations in the range 3 - 50 MHz to the radiolocation service for oceanographic radar applications, taking into account the results of ITU R studies, in accordance with Resolution 612 (WRC 07)”

Description:  Oceanographic radar using HF frequencies are used as a means of determining sea currents and wave size.  These devices typically use multiple frequencies across the HF spectrum in order to elicit responses from different wave heights.  They typically occupy over 100 kHz of radio spectrum and use transmitter powers of around 50 Watts.

At present the use of such radar is authorised locally by the spectrum administration concerned on frequencies ranging from 3 to 30 MHz on a non-interference basis (i.e. they must not cause interference to any other user nor can they claim protection from interference from other spectrum users).  Due, however, to the nature of radio propagation in the HF frequency range, the emissions from such devices have the potential to cause harmful interference to stations thousands of kilometres away and indeed such interference is widely reported.

In order to minimise interference to other users, oceanographic radar users are seeking to identify an exclusive (primary) allocation for their equipment.  Three specific frequency allocations have been requested, around 4.5, 13 and 25 MHz with each band not exceeding 600 kHz.

Resolution 612 invites the ITU to

“complete the necessary studies, as a matter of urgency, taking into account the present use of the allocated band, with a view to presenting, at the appropriate time, the technical information likely to be required as a basis for the work of WRC-11”

The amount of spectrum and the frequency ranges concerned have the potential to overlap with maritime mobile use, in particular in the bands 4063 to 4438 kHz, 8100 to 8815 kHz, 12230 to 13200 kHz and 25070 to 25210 kHz.

It is unlikely that any final allocation which directly interferes with maritime operation will be selected for these devices given the nature of the conflicts between the service, even adjacent frequency use has the potential to cause problems.  However, other services within the HF spectrum are equally fervent in support of protection for their allocations and counterbalancing any maritime argument is the fact that the service delivers most benefit to the maritime community, either directly or through oceanographic research.

IALA Position:  New allocations to the radiolocation services shall not cause harmful interference to the HF Maritime Mobile Bands.

Track the results of ITU WP5B work and studies to ensure that any resulting recommendations accurately reflect maritime needs and protect maritime HF communications.

### Agenda Item 1.23

Agenda Item 1.23:  “to consider an allocation of about 15 kHz in parts of the band 415-526.5 kHz to the amateur service on a secondary basis, taking into account the need to protect existing services”

Description:  Reduction in the use of spectrum around 500 kHz has led to radio amateurs approaching various regulators in order to gain access to a portion of the band.  In fact, some countries have already allowed amateurs (limited) access to frequencies around 500 kHz.

With the exception of some continued use in the Pacific and use for NBDP and NAVTEX transmissions on 490 and 518 kHz respectively, the use of other frequencies and of satellite technology are reducing reliance on long-established LF and MF services for maritime communications. However, France and the United States are developing digital data systems to support e-Navigation around 500 kHz.

IALA Position: DGNSS technology and e-Navigation digital communications technologies are likely candidates to make use of this band in the near future. Allowing secondary use by amateur services is not in the interest of the maritime service.

ITU WP 5A is responsible for this agenda item. ITU WP 5B should liaise with ITU WP 5A to inform of the maritime use for e-Navigation around 500 kHz.

## Other WRC 2012 issues that may impact maritime communication

Other than the concerns identified in section 4.2, the maritime mobile bands are generally well protected with no specific indication that further effort is needed to secure their continued use. The situation with respect to maritime mobile-satellite use is complex, however use for GMDSS is protected via footnote 5.535A in ITU-R Radio Regulations Article 5 and thus there appears no particular need to take action other than to safeguard the protection offered by this footnote.

There are three key activities which need to take place in reference to protecting existing spectrum allocations:

- General continued protection of the status of the maritime mobile service across the RF spectrum.

- Keeping a watching brief on the (ITU) development of frequency plans for the 450 to 470 MHz band so as to protect on-board use;

- Engaging in discussions concerning the use of PLT/BPL at a national level as and when the possibility or necessity arises;

- Should these issues develop further IALA needs to establish a position that protects maritime spectrum.

# Summary

The IALA Maritime Radio Communication Plan identifies current and future technologies used for maritime communication and addresses the IALA position regarding maritime related agenda items for WRC 2012.

e-Navigation is the future, digital concept for the maritime sector that responds to structural changes and will have a profound and long-term impact on the way the maritime sector operates. It is foreseen to be supported by many current and future technologies identified in this plan. The communications infrastructure should be designed to enable authorised seamless information transfer onboard ship, between ships, between ship and shore and between shore authorities and other parties. This infrastructure will have to be capable of not only supporting future e-Navigation applications, but will also have to support legacy applications – at least during the transition to e-Navigation, if not beyond. It will therefore be bandwidth intensive and possibly rely upon a range of technologies.

Securing spectrum, or changing the use within existing allocations is important for the e-Navigation concept due to the planning timescales required to protect or extend the use of the radio spectrum through the ITU WRC.

In general it is concluded that there are few concerns relating to e-Navigation from a spectrum management perspective. The general continued protection of the maritime mobile service allocations need to take place.  Particular emphasis should be given to following items:

- keeping a watching brief on the (ITU) development of frequency plans for the 450 to 470 MHz band so as to protect on-board use; and

- engaging in discussions concerning the use of PLT/BPL at a national level as and when the possibility or necessity arises;

- Protect the spectrum around 500 kHz for digital communications for e-Navigation;

- Protect the existing AIS 1 and 2 channels by designating the channels for the exclusive use for safety of communication.

In relation to the forthcoming WRC meeting, spectrum requirements to support e-Navigation should be addressed. In particular:

- Support for modifications to Appendix 17 HF maritime channelisation to permit new digital systems;

- Support for maritime mobile-satellite allocations within the VHF maritime mobile band to enable satellite based AIS communication;

- Support for additional AIS channel allocations within the VHF maritime mobile band for digital communications;

- Allocate the 495 - 505 kHz band for the maritime mobile service to support NAVTEX, Data, and Voice communications. Oppose the amateur allocation in the 415 - 526.5 kHz band under Agenda Item 1.23;

- Monitor developments with respect to VHF radiolocation allocations so as to protect maritime VHF use;

- Track the results of ITU WP5B work and studies to ensure that any resulting recommendations accurately reflect maritime needs and protect maritime HF communications.

Finally, the plan suggests e-Navigation should develop an automated selection process of available communication technologies that would operate automatically, selecting the best communications technology, channel, and characteristics in accordance with the ship’s location, and the type of data to be exchanged.

Without a complete understanding of the e-Navigation concept and future user requirements, it is difficult to speculate what specific systems and spectrum will be required to achieve e-Navigation. As e-Navigation develops the IALA Maritime Radio Communication Plan needs to be reviewed and updated.

- System Description

- System Description Table

| System | Band | Frequency Channel | Bandwidth Data rate | Status | Life Span | Ownership | Mode | Service | Purposes | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Distress and safety communication within GMDSS |  |  |  |  |  |  |  |  |  |  |
| MF/HF Voice | MF/HF | 2182 kHz 4125 kHz 6215 kHz 8291 kHz 12290 kHz 16420 kHz | 3 kHz | Current | Long | Regional and International | Analogue voice. | Mobile to mobile. Fixed to mobile. Mobile to fixed. | Distress communication | Long distance, > 250 nm |
| MF/HF DSC | MF/HF . | 2187.5 kHz 4207.5 kHz 6312.0 kHz 8414.5 kHz 12577 kHz 16804.5 kHz | 0.5 kHz 100 bps | Current | Long | International | Data (Digital) | Mobile to mobile. Fixed to mobile. Mobile to fixed. | Distress alerting | Long distance > 250 nm |
| 121.5 DF | VHF. | 121.5 MHz |  | Current | Short | International | Carrier (Analogue) | Mobile to mobile. | Location. | Line of sight |
| VHF DSC | VHF. | 156.525 MHz (Ch 70) | 25 kHz 1200 bps | Current | Long | International | Data (Digital) | Mobile to mobile. Fixed to mobile. Mobile to fixed. | Distress alerting | Line of sight |
| VHF voice | VHF . | 156.300 MHz (Ch 06) 156.650 MHz (Ch 13) 156.800 MHz (Ch 16) | 25 kHz | Current | Long | International | Voice | Mobile to mobile. Fixed to mobile. Mobile to fixed. | Distress communication | Line of sight |
| Two way VHF | VHF . | 156.025 to 161.950 MHz. | 25 kHz | Current | Long | International | Voice | Mobile to mobile. | On scene communication | Line of sight |
| AIS SART | VHF | 161.975 MHz, 162.025 MHz | 25 kHz 9600 bps/ TDMA | Near future | Long | International | digital | Mobile to mobile | Homing | Line of sight |
| EPIRB; | UHF | 406 MHz |  | Current | Long | International | Digital | Mobile to satellite | Distress alerting | COSPAS-SARSAT Satellite; Global converge |
| Satellite INMARSAT C, B, F | UHF | Tx 1626.5 to 1646.5 MHz. Rx 1525.0 to 1545.0 MHz. |  | Current | Long | 3rd party | Digital. Voice and data. | Satellite to earth. Earth to satellite. | Distress alerting, distress communication. | Global converge |
| RADAR SART, x-Band | SHF | 9.2 – 9.5 GHz |  | Current | medium | International | analogue | Mobile to mobile | Homing | Line of sight |
| Maritime safety information promulgation |  |  |  |  |  |  |  |  |  |  |
| NAVTEX | MF/HF | 518 kHz 490 kHz and 4209.5 kHz (for local language) | 0.5 kHz | Current | Medium | International | Text | Fixed to mobile. | Reception of maritime safety information. | Long distance, > 250 nm Broadcast to mobile only. |
| Narrow band direct print. (NBDP) | HF. | . 4210 kHz 6314 kHz 8416.5 kHz 12509 kHz 16806.5 kHz 19680.5 kHz 22376.0 kHz 26100.5 kHz | 0.5 kHz | Current | Medium | International. | Text | Mobile to mobile. Fixed to mobile. Mobile to fixed. | Reception of maritime safety information | Long distance, > 250 nm |
| EGC | UHF | Tx 1626.5 to 1646.5 MHz. Rx 1525.0 to 1545.0 MHz. |  | Current | Long | international | Digital data. | Satellite to mobile | Reception of maritime safety information (Safety Net) | Global coverage |
| Safety of navigation |  |  |  |  |  |  |  |  |  |  |
| Loran and e-Loran | LF | 100 kHz |  | Current | Very Long | Regional | Carrier (Analogue) | Fixed to mobile. | Positioning | Long distance |
| DGNSS | MF | 285 to 325 kHz | 50/100 bps | Current | Medium. | Regional. | Digital | Fixed to mobile. | Augmentation of positioning | Medium distance |
| AIS | VHF | 161.975MHz (AIS 1) 162.025MHz (AIS 2) | 25 kHz 9600 bps/ TDMA | Current | Long | International | Data (Digital) | Mobile to mobile. Fixed to mobile. Mobile to fixed. | Collision avoidance, Environmental protection, Security, VTS, Augmentation of positioning, communication for navigational and safety related purposes. | Line of sight |
| AIS ATON | VHF | 161.975MHz (AIS 1) 162.025MHz (AIS 2) | 25 kHz 9600 bps/ TDMA | Current | Long | International | Data (Digital) | ATON (fixed/floating) to mobile/shore | Navigational aid | Line of sight |
| VHF voice | VHF Mobile. | 156.025 to 161.950 MHz. | 25 kHz | Current | Long | International | Voice | Mobile to mobile. Fixed to mobile. Mobile to fixed. | communication for navigational and safety related purposes | Line of sight |
| GNSS | UHF | GPS: 1227.6 MHz 1575.42 MHz GLONASS: Compass Galileo |  | Current Current Future Future | Long Long Long Long | International International Regional International | Digital | Satellite to earth | Positioning | Global coverage |
| LRIT | Satellite | Tx 1626.5 to 1646.5 MHz. Rx 1525.0 to 1545.0 Mhz. |  | Future | Short | 3rd party | Data (Digital) | Inmarsat | Security. Vessel tracking. Environmental protection. | Global coverage |
| Radar S Band. |  | 2.9 to 3.1 GHz |  | Current | Long | International |  | Mobile and fixed | Collision avoidance, Navigational aid. | Line of sight |
| Radar X Band |  | 9.2 to 9.5 GHz |  | Current | Long | International |  | Mobile and fixed. | Collision avoidance,. Navigational aid | Line of sight |
| RACON |  | 2.9 to 3.1 GHz and 9.2 to 9.5 GHz |  | Current | Long | International |  | Fixed. | Navigational aid | Line of sight |
| General communication |  |  |  |  |  |  |  |  |  |  |
| HF Voice | MF/HF | 1.6 to 26.5 MHz | 3 kHz | Current | Long | Regional and International | Analogue voice. (ALE) | Mobile to mobile. Fixed to mobile. Mobile to fixed. | General voice communication. | Long distance, > 250 nm |
| TOR/SITOR | MF & HF. | 1.6 to 26.5 MHz. | 0.5 kHz | Near Dead | Short. | Regional. | Text | Mobile to mobile. Fixed to mobile. Mobile to fixed. | General text communication. | Long distance, > 250 nm |
| Narrow band direct print. (NBDP) | MF & HF. | 1.6 to 26.5 MHz. | 0.5 kHz | Current | Long for GMDSS. Short for general communication. | International. | Text | Mobile to mobile. Fixed to mobile. Mobile to fixed. | General text communication. . | Long distance, > 250 nm |
| HF Voice | MF/HF | 1.6 to 26.5 MHz | 3 kHz | Current | Long | Regional and International | Analogue voice. (ALE) | Mobile to mobile. Fixed to mobile. Mobile to fixed. | General voice communication. | Long distance, > 250 nm |
| HF Data | MF/HF | 1.6 to 26.5 MHz | 0.5 – 3 kHz | Current | Long | Regional and International | Digital data. | Fixed to mobile. Mobile to fixed. | General data communication. | Long distance, > 250 nm |
| VHF voice | VHF MMBe | 156.025 to 161.950 MHz. | 25 kHz | Current | Long | international | Voice | Mobile to mobile. Fixed to mobile. Mobile to fixed. | General voice communication | Line of sight |
| VHF data | VHF MMBe | 156.025 to 161.950 MHz. | 25 kHz | Current | Long | regional | data | Mobile to mobile. Fixed to mobile. Mobile to fixed. | General data communication. | Line of sight |
| Satellite | UHF | Tx 1626.5 to 1646.5 MHz. Rx 1525.0 to 1545.0 MHz |  | Current | Long | global 3rd party | Digital. Voice and data. | Satellite to earth. Earth to satellite. | General voice and data communication. | Global coverage |
| GSM | UHF |  |  | Current | Long | regional 3rd party | Digital. Voice and data. | Cellular. Mobile-fixed-mobile. | General voice and data communication | Medium range in locality |
| GPRS | UHF |  |  | Current | Long | regional 3rd party | Digital. Voice and data. | Cellular. Mobile-fixed-mobile. | General voice and data communication | Short range in locality |
| UMTS/3G | UHF |  |  | Current | Long | regional 3rd party | Digital. Voice and data. | Cellular. Mobile-fixed-mobile. | General voice and data communication | Short range in locality |
| IEEE 802.11 (WiFi) | UHF/ SHF | 2.4 – 2.5 GHz 5.17 – 5.33 GHz 4.9 GHz | 11 Mbps 54 - 300 Mbps 54 Mbps | Current | Long | local 3rd party | Digital data | LAN Peer-to peer | Internet access | Sort in locality < 100m <10 km |
| IEEE 802.16 (WiMax) | UHF/ SHF | 2.5 GHz 3.3 GHz | 75 Mbps | Current | Long | local 3rd party | Digital data | LAN Peer-to peer | Internet access | Short range in locality < 50 km |
| IEEE 802.15.4 (ZigBee) | UHF | 868-870 MHz 902 – 928 MHz 2.4 GHz | 20 kbps 40 kbps 250 kbps | Current | Long | local 3rd party | Digital data |  |  | Short range in locality < 30 km |
| Future development |  |  |  |  |  |  |  |  |  |  |
| HF Digital data. | HF | 4 to 26 MHz. | 10 to 20 kHz. | Near Future | Long term | International | Data (Digital) | Mobile to mobile. Fixed to mobile. Mobile to fixed. | Binary data communication for navigational and safety related purposes. | KENTA (France) Has range advantage over VHF 40 to 250 NM (>250 sky wave). |
| AIS 3rd & 4th Channels | VHF | Within RR App 18 | 25kHz Min 9600bps TDMA | Near Future | Long term | International | Data (Digital) | Mobile to satellite. | Security. Environmental protection. Vessel tracking. GMDSS (AIS EPIRB) | Closely related to current AIS. Same transponder on ship |
| AIS 5th & 6th Channels | VHF | Within RR App 18 | 25kHz Min 9600bps TDMA | Near Future | Long term | International | Data (Digital) | Mobile to mobile. Fixed to mobile. Mobile to fixed. | Binary data communication for navigational and safety related purposes. Security and tracking. | Closely related to current AIS. Same transponder on ship |
| AIS MMB channel management | VHF | Channel 70 (AIS 1 and AIS 2) | 25kHz 9600bps TDMA | Near Future | Long term | International | Data (Digital) | Mobile to mobile. Fixed to mobile. Mobile to fixed. | To manage data transfer over channels in the MMB. | Eg. “go to channel XX to get chart update for Tokyo Bay. |
| VHF Digital data. - ECDIS chart data - Digital voice - Virtual channels - UMDM support - Internet access | VHF Mobile. | 156.025 to 161.950 MHz. | 25 to 100 kHz. See note 3. | Near Future | Long term | International | Data (Digital) | Mobile to mobile. Fixed to mobile. Mobile to fixed. | Binary data communication for navigational and safety related purposes. | ITU 1842 SC 123 Managed or autonomous systems. |

- Maritime Frequency Allocations

The table below sets out the various current maritime frequency allocations as allocated by ITU Radio Regulation.  Due to the size, footnotes to the allocation in the ITU Radio Regulation are not included.  Please refer to Article 5 in the ITU Radio Regulation, Edition of 2008.

- Maritime Frequency Allocations

| Frequency | Region 1 Allocation | Region 2 Allocation | Region 3 Allocation | Status of Maritime Use |
| --- | --- | --- | --- | --- |
| 90 – 110 kHz | RADIONAVIGATION 5.62 Fixed 5.64 |  |  | No constraints |
| 285 – 315 kHz | AERONAUTICAL RADIONAVIGATION MARITIME RADIONAVIGATION (radiobeacons) 5.73 5.72 5.74 | AERONAUTICAL RADIONAVIGATION MARITIME RADIONAVIGATION (radiobeacons) 5.73 |  | No constraints |
| 315 – 325 kHz | AERONAUTICAL RADIONAVIGATION Maritime radionavigation (radiobeacons) 5.73 5.72 5.75 | MARITIME RADIONAVIGATION (radiobeacons) 5.73 Aeronautical radionavigation | AERONAUTICAL RADIONAVIGATION MARITIME RADIONAVIGATION (radiobeacons) 5.73 | Secondary in Region 1 |
| 415 – 435 kHz | MARITIME MOBILE 5.79 AERONAUTICAL RADIONAVIGATION 5.72 | MARITIME MOBILE 5.79 5.79A Aeronautical radionavigation 5.80 5.77 5.78 5.82 |  | No constraints |
| 435 – 495 kHz | MARITIME MOBILE 5.79 5.79A Aeronautical radionavigation 5.72 5.82 | MARITIME MOBILE 5.79 5.79A Aeronautical radionavigation 5.80 5.77 5.78 5.82 |  | No constraints |
| 495 – 505 kHz | MOBILE 5.82A 5.82B |  |  | No constraints |
| 505 – 510 kHz | MARITIME MOBILE 5.79 5.79A 5.84 AERONAUTICAL RADIONAVIGATION 5.72 | MARITIME MOBILE 5.79 | MARITIME MOBILE 5.79 5.79A 5.84 AERONAUTICAL RADIONAVIGATION Aeronautical mobile Land mobile | No constraints |
| 510 – 525 kHz | MARITIME MOBILE 5.79 5.79A 5.84 AERONAUTICAL RADIONAVIGATION 5.72 | MOBILE 5.79A 5.84 AERONAUTICAL RADIONAVIGATION | MARITIME MOBILE 5.79 5.79A 5.84 AERONAUTICAL RADIONAVIGATION Aeronautical mobile Land mobile | No constraints |
| 1606.5 – 1625 kHz | FIXED MARITIME MOBILE 5.90 LAND MOBILE 5.92 | BROADCASTING 5.89 5.90 | FIXED MOBILE RADIOLOCATION RADIONAVIGATION 5.91 | Region 1 only and subject to restrictions on permitted coverage area |
| 1635 – 1705 kHz | FIXED MARITIME MOBILE 5.90 LAND MOBILE 5.92 5.96 | FIXED MOBILE BROADCASTING 5.89 Radiolocation 5.90 | FIXED MOBILE RADIOLOCATION RADIONAVIGATION 5.91 | Region 1 only and subject to restrictions on permitted coverage area |
| 1705 – 1800 kHz | FIXED MARITIME MOBILE 5.90 LAND MOBILE 5.92 5.96 | FIXED MOBILE RADIOLOCATION AERONAUTICAL RADIONAVIGATION | FIXED MOBILE RADIOLOCATION RADIONAVIGATION 5.91 | Region 1 only and subject to restrictions on permitted coverage area |
| 2045 – 2065 kHz | FIXED MARITIME MOBILE LAND MOBILE 5.92 | FIXED MOBILE |  | Region 1 only |
| 2065 – 2107 kHz | FIXED MARITIME MOBILE LAND MOBILE 5.92 | MARITIME MOBILE 5.105 5.106 |  | No constraints |
| 2107 – 2160 kHz | FIXED MARITIME MOBILE LAND MOBILE 5.92 | FIXED MOBILE |  | Region 1 only |
| 2170 – 2173.5 kHz | MARITIME MOBILE |  |  | No constraints |
| 2190.5 – 2194 kHz | MARITIME MOBILE |  |  | No constraints |
| 2625 – 2650 kHz | MARITIME MOBILE MARITIME RADIONAVIGATION 5.92 | FIXED MOBILE |  | Region 1 only |
| 4000 – 4063 kHz | FIXED MARITIME MOBILE 5.127 5.126 |  |  | No constraints |
| 4063 – 4438 kHz | MARITIME MOBILE 5.79A 5.109 5.110 5.130 5.131 5.132 5.128 |  |  | No constraints |
| 6200 – 6525 kHz | MARITIME MOBILE 5.109 5.110 5.130 5.132 5.137 |  |  | No constraints |
| 8100 – 8195 kHz | FIXED MARITIME MOBILE |  |  | No constraints |
| 8195 – 8815 kHz | MARITIME MOBILE 5.109 5.110 5.132 5.145 5.111 |  |  | No constraints |
| 12230 – 13200 kHz | MARITIME MOBILE 5.109 5.110 5.132 5.145 |  |  | No constraints |
| 16360 – 17410 kHz | MARITIME MOBILE 5.109 5.110 5.132 5.145 |  |  | No constraints |
| 18780 – 18900 kHz | MARITIME MOBILE |  |  | No constraints |
| 19680 – 19800 kHz | MARITIME MOBILE 5.132 |  |  | No constraints |
| 22000 – 22855 kHz | MARITIME MOBILE 5.132 5.156 |  |  | No constraints |
| 25070 – 25210 kHz | MARITIME MOBILE |  |  | No constraints |
| 26100 – 26175 kHz | MARITIME MOBILE 5.132 |  |  | No constraints |
| 156 – 156.4875 MHz | FIXED MOBILE except aeronautical mobile (R) 5.226 | FIXED MOBILE 5.225 5.226 |  | Operation protected by footnote 5.226 |
| 156.4875 MHz – 156.5625 MHz | MARITIME MOBILE (distress and calling via DSC) 5.111 5.226 5.227 |  |  | No constraints |
| 156.5625 – 156.7625 MHz | FIXED MOBILE except aeronautical mobile (R) 5.226 | FIXED MOBILE 5.225 5.226 |  | Operation protected by footnote 5.226 |
| 156.7625 MHz – 156.8375 MHz | MARITIME MOBILE (distress and calling) 5.111 5.226 |  |  | No constraints |
| 156.8375 - 162.050 MHz | FIXED MOBILE except aeronautical mobile (R) 5.226 5.227A 5.229 | FIXED MOBILE 5.226 5.227A 5.230 5.231 5.232 |  | Operation protected by footnote 5.226 |
| 406 – 406.1 MHz | MOBILE-SATELLITE (Earth-to-space) 5.266 5.267 |  |  | No constraints |
| 456 – 459 MHz | FIXED MOBILE 5.286AA 5.271 5.287 5.288 |  |  | Footnote 5.287 specifies maritime mobile use |
| 460 – 470 MHz | FIXED MOBILE 5.286AA Meteorological-satellite (space-to-Earth) 5.287 5.288 5.289 5.290 |  |  | Footnote 5.287 specifies maritime mobile use |
| 1525 – 1530 MHz | SPACE OPERATION (space-to-Earth) FIXED MOBILE-SATELLITE (space-to-Earth) 5.208B 5.351A Earth exploration-satellite Mobile except aeronautical mobile 5.349 5.341 5.342 5.350 5.351 5.352A 5.354 | SPACE OPERATION (space-to-Earth) MOBILE-SATELLITE (space-to-Earth) 5.208B 5.351A Earth exploration-satellite Fixed Mobile 5.343 5.341 5.351 5.354 | SPACE OPERATION (space-to-Earth) FIXED MOBILE-SATELLITE (space-to-Earth) 5.208B 5.351A Earth exploration-satellite Mobile 5.349 5.341 5.351 5.352A 5.354 | Complex |
| 1530 – 1535 MHz | SPACE OPERATION (space-to-Earth) MOBILE-SATELLITE (space-to-Earth) 5.208B 5.351A 5.353A Earth exploration-satellite Fixed Mobile except aeronautical mobile 5.341 5.342 5.351 5.354 | SPACE OPERATION (space-to-Earth) MOBILE-SATELLITE (space-to-Earth) 5.208B 5.351A 5.353A Earth exploration-satellite Fixed Mobile 5.343 5.341 5.351 5.354 |  | Complex |
| 1535 – 1559 MHz | MOBILE-SATELLITE (space-to-Earth) 5.208B 5.351A 5.341 5.351 5.353A 5.354 5.355 5.356 5.357 5.357A 5.359 5.362A |  |  | Complex |
| 1616 – 1626.5 MHz | MOBILE-SATELLITE (Earth-to-space) 5.351A AERONAUTICAL RADIONAVIGATION Mobile-satellite (space-to-Earth) 5.208B 5.341 5.355 5.359 5.364 5.365 5.366 5.367 5.368 5.369 5.371 5.372 | MOBILE-SATELLITE (Earth-to-space) 5.351A AERONAUTICAL RADIONAVIGATION RADIODETERMINATION- SATELLITE (Earth-to-space) Mobile-satellite (space-to-Earth) 5.208B 5.341 5.364 5.365 5.366 5.367 5.368 5.370 5.372 | MOBILE-SATELLITE (Earth-to-space) 5.351A AERONAUTICAL RADIONAVIGATION Mobile-satellite (space-to-Earth) 5.208B Radiodetermination-satellite (Earth-to-space) 5.341 5.355 5.359 5.364 5.365 5.366 5.367 5.368 5.369 5.372 | Complex |
| 1626.5 – 1646 MHz | MOBILE-SATELLITE (Earth-to-space) 5.351A 5.341 5.351 5.353A 5.354 5.355 5.357A 5.359 5.362A 5.374 5.375 5.376 |  |  | Complex |
| 2900 – 3100 MHz | RADIOLOCATION 5.424A RADIONAVIGATION 5.426 5.425 5.427 |  |  | No constraints |
| 9200 – 9300 MHz | RADIOLOCATION MARITIME RADIONAVIGATION 5.472 5.473 5.474 |  |  | No constraints |
| 9300 – 9500 MHz | RADIONAVIGATION EARTH EXPLORATION-SATELLITE (active) SPACE RESEARCH (active) RADIOLOCATION 5.427 5.474 5.475 5.475A 5.475B 5.476A |  |  | No constraints |

- Current and evolving wireless telecommunications technologies in the Maritime environment

There are an ever increasing number of wireless technologies which aim to deliver voice and data connectivity to mobile users and which may be of use in the maritime environment.  Aside from their technical capabilities, there may be regulatory issues and associated technical restrictions which prevent them being fully exploited.  Such issues may include:

- the ITU radio regulations do not permit maritime mobile operation in the frequency bands concerned;

- frequencies are licensed on a national basis, such that there is no continuity of operation from country to country (this may even require equipment to be switched off when entering certain jurisdictions);

- the system parameters may have the potential to cause interference to ship-borne equipment;

- planning parameters may make use at sea (or even in ports) complex or difficult.

The following terrestrial technologies are currently in the process of either being rolled-out or standardised on an international basis and thus may be candidates for use in the maritime environment, particularly for commercial port services:

- Digital PMR (to replace analogue PMR for on-board comms): TETRA, TETRAPOL, P25, dPMR, DMR, TDMA;

- 3G Mobile: UMTS (TDD & FDD), cdma2000, TD-SCDMA, IEEE802.20 (including iBurst), HSDPA;

- ‘3.5G’ Mobile: WiMAX;

- 4G Mobile: LTE, UMB.

In addition to these commercial service technologies, there are various technologies which may offer potential solutions under the banner of ‘licence exempt’ or ‘low-power’ technologies such as WiFi, Bluetooth, ZigBee and UWB.  The range of such services, however, is exceptionally limited (normally to 100 metres or much less) and the frequencies employed are globally allocated for the purpose (e.g. the 2.4 GHz ISM band) such that there is no need for a specific consideration of the impact on spectrum of their usage in a maritime environment.

Table 4 below details, for each of the commercial technologies, the frequency band(s) in which they ‘prefer’ to operate, i.e. those for which there are known services, as well as the general range of frequencies over which they are specified to operate.

- Summary of spectrum requirements of new commercial data links

| Technology | Frequency Range | Notes |
| --- | --- | --- |
| TETRA | 380 – 400 MHz 410 – 430 MHz 450 – 470 MHz 806 – 821 // 851 – 866 MHz | EN 300 392 |
| TETRA 2 (TEDS) |  | EN 302 561 |
| TETRAPOL | 70 – 933 MHz |  |
| P25 | 136 – 870 MHz |  |
| DMR | 30 – 900 MHz | TS 102 361 |
| dPMR | 30 – 900 MHz | EN 301 166 |
| GSM (incl EDGE) | 380 – 400 MHz 410 – 430 MHz 450 – 470 MHz 478 – 496 MHz 698 – 746 MHz 747 – 792 MHz 806 – 866 MHz 824 – 894 MHz 870 – 921 MHz 876 – 925 MHz 880 – 960 MHz 1710 – 1880 MHz 1850 – 1990 MHz |  |
| cdma2000 | 450 – 470 MHz 790 – 862 MHz 824 – 894 MHz 880 – 960 MHz 1710 – 1880 MHz 1850 – 1990 MHz 1820 – 2170 MHz 2300 – 2400 MHz 2500 – 2690 MHz |  |
| W-CDMA (UMTS) |  |  |
| TD-CDMA | 1900 – 1920 MHz 2010 – 2025 MHz (2570 – 2620 MHz) | ‘TDtv’ applications have been trialled in some EU countries |
| TD-SCDMA |  |  |
| WiMAX | 2 – 11 GHz 10 – 66 GHz | 3400 – 3600 MHz is being seen as a potential target following WRC-07 and EU WAPECS moves |
| 802.20 (MBWA) | Below 3.5 GHz |  |
| WiBro | 2.3 – 2.4 GHz | Currently Korea only |
| iBurst (HC-SDMA) | Below 3.5 GHz | Often uses 1785 – 1805 MHz or 1900 – 1920 MHz |

LTE and UMB are 4G extensions to UMTS and cdma2000 respectively. At present, it is envisaged that existing spectrum assignments will be re-farmed such that no new spectrum will be necessary.

From the above, the bands of particular interest, and where there are most likely to be mobile communication systems providing value in maritime applications are:

- 380 to 400 MHz

- 410 to 430 MHz

- 450 to 470 MHz

- 790 to 862 MHz (the ‘Digital Dividend’)

- 824 to 849 // 869 to 894 MHz (US ‘800 MHz’ cellular band)

- 880 to 915 // 925 to 960 MHz (EU ‘900 MHz’ cellular band)

- 1710 to 1785 // 1815 to 1880 MHz (EU ‘1800 MHz’ cellular band)

- 1850 to 1910 // 1930 to 1990 MHz (US ‘1900 MHz’ cellular band)

- 1880 to 1920 MHz (EU ‘3G TDD’ band)

- 1920 to 1980 // 2110 to 2170 MHz (EU ‘3G’ band)

- 2010 to 2015 MHz (EU ‘3G TDD’ band)

- 2400 to 2483.5 MHz (‘2.4 GHz’ licence exempt, low-power band)

- 2500 to 2690 MHz (‘3G expansion’ band)

- 3400 to 3600 MHz

Table 5 below shows, based on the latest version of the ITU Radio Regulations, the allocations in those bands (including any footnotes), highlighting the position of maritime services in those bands.

- Summary of maritime radio spectrum

| Frequency (MHz) | Region 1 Allocation | Region 2 Allocation | Region 3 Allocation | Status of Maritime Use |
| --- | --- | --- | --- | --- |
| 380 – 387 | FIXED MOBILE 5.254 |  |  | No constraints |
| 387 – 390 | FIXED MOBILE Mobile-satellite (space-to-Earth) 5.208A 5.208B 5.254 5.255 |  |  | No constraints |
| 390 – 399.9 | FIXED MOBILE 5.254 |  |  | No constraints |
| 410 – 420 | FIXED MOBILE except aeronautical mobile SPACE RESEARCH (space-to-space) 5.268 |  |  | No constraints |
| 420 – 430 | FIXED MOBILE except aeronautical mobile Radiolocation 5.269 5.270 5.271 |  |  | No constraints |
| 450 – 455 | FIXED MOBILE 5.286 AA 5.209 5.271 5.286 5.286A 5.286B 5.286C 5.286D 5.286 E |  |  | No constraints |

| Frequency (MHz) | Region 1 Allocation | Region 2 Allocation | Region 3 Allocation | Status of Maritime Use |
| --- | --- | --- | --- | --- |
| 455 – 456 | FIXED MOBILE 5.286AA 5.209 5.271 5.286A 5.286B 5.286C 5.286 E | FIXED MOBILE 5.286AA MOBILE-SATELLITE (Earth-to-space) 5.286A 5.286B 5.286C 5.209 | FIXED MOBILE 5.286AA 5.209 5.271 5.286A 5.286B 5.286C 5.286 E | No constraints |
| 456 – 459 | FIXED MOBILE 5.286AA 5.271 5.287 5.288 |  |  | No constraints |
| 459 – 460 | FIXED MOBILE 5.286AA 5.209 5.271 5.286A 5.286B 5.286C 5.286 E | FIXED MOBILE 5.286AA MOBILE-SATELLITE (Earth-to-space) 5.286A 5.286B 5.286C 5.209 | FIXED MOBILE 5.286AA 5.209 5.271 5.286A 5.286B 5.286C 5.286 E | No constraints |
| 460 – 470 | FIXED MOBILE 5.286AA Meteorological-satellite (space-to-Earth) 5.287 5.288 5.289 5.290 |  |  | No constraints |
| 790 – 806 | FIXED BROADCASTING MOBILE except aeronautical mobile 5.316B 5.317A 5.312 5.314 5.315 5.316 5.316A 5.319 | BROADCASTING Fixed MOBILE 5.313B 5.317A 5.293 5.309 5.311A | FIXED MOBILE 5.313A 5.317A BROADCASTING 5.149 5.305 5.306 5.307 5.311A 5.320 | No constraints |
| 806 – 862 | FIXED BROADCASTING MOBILE except aeronautical mobile 5.316B 5.317A 5.312 5.314 5.315 5.316 5.316A 5.319 | FIXED MOBILE 5.317A BROADCASTING 5.317 5.318 | FIXED MOBILE 5.313A 5.317A BROADCASTING 5.149 5.305 5.306 5.307 5.311A 5.320 | No constraints |
| 862 – 890 | FIXED MOBILE except aeronautical mobile 5.317A BROADCASTING 5.322 5.319 5.323 | FIXED MOBILE 5.317A BROADCASTING 5.317 5.318 | FIXED MOBILE 5.313A 5.317A BROADCASTING 5.149 5.305 5.306 5.307 5.311A 5.320 | No constraints |
| 890 – 902 | FIXED MOBILE except aeronautical mobile 5.317A BROADCASTING 5.322 Radiolocation 5.323 | FIXED MOBILE except aeronautical mobile 5.317A Radiolocation 5.318 5.325 | FIXED MOBILE 5.317A BROADCASTING Radiolocation 5.327 | No constraints |
| 902 – 928 | FIXED MOBILE except aeronautical mobile 5.317A BROADCASTING 5.322 Radiolocation 5.323 | FIXED Amateur Mobile except aeronautical mobile 5.325A Radiolocation 5.150 5.325 5.326 | FIXED MOBILE 5.317A BROADCASTING Radiolocation 5.327 | No constraints |
| 928 – 942 | FIXED MOBILE except aeronautical mobile 5.317A BROADCASTING 5.322 Radiolocation 5.323 | FIXED MOBILE except aeronautical mobile 5.317A Radiolocation 5.325 | FIXED MOBILE 5.317A BROADCASTING Radiolocation 5.327 | No constraints |
| 942 – 960 | FIXED MOBILE except aeronautical mobile 5.317A BROADCASTING 5.322 5.323 | FIXED MOBILE 5.317A | FIXED MOBILE 5.317A BROADCASTING 5.320 | No constraints |
| 1710 – 1930 | FIXED MOBILE 5.384A 5.388A 5.388B 5.149 5.341 5.385 5.386 5.387 5.388 |  |  | No constraints |
| 1930 – 1970 | FIXED MOBILE 5.388A 5.388B 5.388 | FIXED MOBILE 5.388A 5.388B Mobile-satellite (Earth-to-space) 5.388 | FIXED MOBILE 5.388A 5.388B 5.388 | No constraints |
| 1970 – 1980 | FIXED MOBILE 5.388A 5.388B 5.388 |  |  | No constraints |
| 1980 – 2010 | FIXED MOBILE MOBILE-SATELLITE (Earth-to-space) 5.351A 5.388 5.389A 5.389B 5.389F |  |  | No constraints |
| 2010 – 2015 | FIXED MOBILE 5.388A 5.388B 5.388 | FIXED MOBILE MOBILE-SATELLITE (Earth-to-space) 5.388 5.389C 5.389E | FIXED MOBILE 5.388A 5.388B 5.388 | No constraints |
| 2110 – 2120 | FIXED MOBILE 5.388A 5.388B SPACE RESEARCH (deep space) (Earth-to-space) 5.388 |  |  | No constraints |
| 2120 – 2160 | FIXED MOBILE 5.388A 5.388B 5.388 | FIXED MOBILE 5.388A 5.388B Mobile-satellite (space-to-Earth) 5.388 | FIXED MOBILE 5.388A 5.388B 5.388 | No constraints |
| 2160 – 2170 | FIXED MOBILE 5.388A 5.388B 5.388 | FIXED MOBILE MOBILE-SATELLITE (space-to-Earth) 5.388 5.389C 5.389E | FIXED MOBILE 5.388A 5.388B 5.388 | No constraints |
| 2400 – 2450 | FIXED MOBILE 5.384A Amateur Radiolocation 5.150 5.282 5.395 | FIXED MOBILE 5.384A RADIOLOCATION Amateur 5.150 5.282 5.393 5.394 5.396 |  | No constraints |
| 2450 – 2483.5 | FIXED MOBILE Radiolocation 5.150 5.397 | FIXED MOBILE RADIOLOCATION 5.150 |  | No constraints |
| 2500 – 2520 | FIXED 5.410 MOBILE except aeronautical mobile 5.384A 5.405 5.412 | FIXED 5.410 FIXED-SATELLITE (space-to-Earth) 5.415 MOBILE except aeronautical mobile 5.384A 5.404 | FIXED 5.410 FIXED-SATELLITE (space-to-Earth) 5.415 MOBILE except aeronautical mobile 5.384A MOBILE-SATELLITE (space-to-Earth) 5.351A 5.407 5.414 5.414A 5.404 5.415A | No constraints |
| 2520 – 2535 | FIXED 5.410 MOBILE except aeronautical mobile 5.384A BROADCASTING-SATELLITE 5.413 5.416 5.339 5.405 5.412 5.417C 5.417D 5.418B 5.418C | FIXED 5.410 FIXED-SATELLITE (space-to-Earth) 5.415 MOBILE except aeronautical mobile 5.384A BROADCASTING-SATELLITE 5.413 5.416 5.339 5.417C 5.417D 5.418B 5.418C | FIXED 5.410 FIXED-SATELLITE (space-to-Earth) 5.415 MOBILE except aeronautical mobile 5.384A BROADCASTING-SATELLITE 5.413 5.416 5.403 5.414A 5.415A | No constraints |
| 2535 – 2655 | FIXED 5.410 MOBILE except aeronautical mobile 5.384A BROADCASTING-SATELLITE 5.413 5.416 5.339 5.405 5.412 5.417C 5.417D 5.418B 5.418C | FIXED 5.410 FIXED-SATELLITE (space-to-Earth) 5.415 MOBILE except aeronautical mobile 5.384A BROADCASTING-SATELLITE 5.413 5.416 5.339 5.417C 5.417D 5.418B 5.418C | FIXED 5.410 MOBILE except aeronautical mobile 5.384A BROADCASTING-SATELLITE 5.413 5.416 5.339 5.417A 5.417B 5.417C 5.417D 5.418 5.418A 5.418B 5.418C | No constraints |
| 2655 – 2670 | FIXED 5.410 MOBILE except aeronautical mobile 5.384A BROADCASTING-SATELLITE 5.208B 5.413 5.416 Earth exploration-satellite (passive) Radio astronomy Space research (passive) 5.149 5.412 | FIXED 5.410 FIXED-SATELLITE (Earth-to-space) (space-to-Earth) 5.415 MOBILE except aeronautical mobile 5.384A BROADCASTING-SATELLITE 5.413 5.416 Earth exploration-satellite (passive) Radio astronomy Space research (passive) 5.149 5.208B | FIXED 5.410 FIXED-SATELLITE (Earth-to-space) 5.415 MOBILE except aeronautical mobile 5.384A BROADCASTING-SATELLITE 5.413 5.416 Earth exploration-satellite (passive) Radio astronomy Space research (passive) 5.149 5.208B 5.420 | No constraints |
| 2670 – 2690 | FIXED 5.410 MOBILE except aeronautical mobile 5.384A Earth exploration-satellite (passive) Radio astronomy Space research (passive) 5.149 5.412 | FIXED 5.410 FIXED-SATELLITE (Earth-to-space) (space-to-Earth) 5.208B 5.415 MOBILE except aeronautical mobile 5.384A Earth exploration-satellite (passive) Radio astronomy Space research (passive) 5.149 | FIXED 5.410 FIXED-SATELLITE (Earth-to-space) 5.415 MOBILE except aeronautical mobile 5.384A MOBILE-SATELLITE (Earth-to-space) 5.351A 5.419 Earth exploration-satellite (passive) Radio astronomy Space research (passive) 5.149 | No constraints |
| 3400 – 3500 | FIXED FIXED-SATELLITE (space-to-Earth) Mobile 5.430A Radiolocation 5.431 | FIXED FIXED-SATELLITE (space-to-Earth) Amateur Mobile 5.431A Radiolocation 5.433 5.282 | FIXED FIXED-SATELLITE (space-to-Earth) Amateur Mobile 5.432B Radiolocation 5.433 5.282 5.432 5.432A | Secondary |
| 3500 – 3600 | FIXED FIXED-SATELLITE (space-to-Earth) Mobile 5.430A Radiolocation 5.431 | FIXED FIXED-SATELLITE (space-to-Earth) MOBILE except aeronautical mobile Radiolocation 5.433 | FIXED FIXED-SATELLITE (space-to-Earth) MOBILE except aeronautical mobile 5.433A Radiolocation 5.433 | Secondary in Region 1 |

None of the footnotes relating to the use of these bands specifically preclude any use for maritime services.

With the exception of the band 3400 – 3600 MHz, the mobile service (which includes, unless explicitly stated, the maritime mobile service) has a primary allocation meaning that the use of these bands for maritime applications will pose no regulatory issues at the ITU.  In the band 3400 – 3600 MHz, many of the footnotes relate to the upgrading of the mobile service from secondary to primary status, however it is known that in some parts of the world (especially in high rainfall areas such as around the equator) this band is still heavily used for satellite downlinks and as such its development for mobile services is likely to remain constrained.

Conclusions

There does not appear to be any necessity to expend significant effort on the standardisation or regulation of these mobile communication technologies to make them suitable for the maritime community.

- Development of new characteristics

The following tables summarise the status of the various ITU technical characteristics relating to maritime communication applications both for those already existing as well as for those foreseen in e-Navigation. They outline where existing characteristics are likely to need updating or else where new characteristics need to be developed. If changes are required they will take place through the appropriate ITU Study Groups.

ITU Study Groups (SG) are responsible for developing draft recommendations to be provided to the periodic World Radio Conferences (WRC). Study groups are established and assigned study questions by a Radiocommunication Assembly to prepare draft Recommendations for approval by ITU Member States.  In the case of e-Navigation two SG’s are likely to be involved in the development of material to be agreed at WRC, namely SG4 and SG5.

SG4 relates to satellite services, covering systems and networks for the following service:

- Fixed-satellite service;

- Mobile-satellite service;

- Broadcasting-satellite service; and

- Radiodetermination-satellite service.

SG5 relates to terrestrial services, covering systems and networks for the following services:

- Fixed;

- Mobile;

- Radiodetermination;

- Amateur; and

- Amateur-satellite.

Below the SG’s are subgroups, such as Working Parties (WP) and Task Groups (TG) that are established to study the questions assigned to the different Study Groups. For example, SG4 has the following WP’s:

- Working Party 4A - Efficient orbit/spectrum utilization for FSS and BSS;

- Working Party 4B - Systems, air interfaces, performance and availability objectives for FSS, BSS and MSS, including IP-based applications and satellite news gathering;

- Working Party 4C - Efficient orbit/spectrum utilization for MSS and RDSS.

SG5 has the following WP’s:

- Working Party 5A - Land mobile service excluding IMT; amateur and amateur-satellite service.;

- Working Party 5B - Maritime mobile service including Global Maritime Distress and Safety System (GMDSS); aeronautical mobile service and radiodetermination service;

- Working Party 5C - Fixed wireless systems; HF systems in the Fixed and Land Mobile Services;

- Working Party 5D - IMT Systems;

- Joint Task Group 5-6 - Studies on the use of the band 790-862 MHz by mobile applications and by other services.

For maritime e-Navigation applications any necessary revision to, or creation of, technical characteristics will take place in one of two WP’s – SG4 WP4C and SG5 WP5B.

- ITU recommendations for Maritime Radio Applications

| Application |  | Current characteristics |  | Amendments | Mechanism |
| --- | --- | --- | --- | --- | --- |
| DSC | VHF | ITU-R M.493-12 | Digital selective-calling system for use in the maritime mobile service. | No amendments are foreseen for the use of VHF DSC within the e-Navigation concept as currently envisaged. However, if the VHF voice channels are changed to make them more spectrally efficient (e.g. see VHF voice below) there may be a need to revise the VHF DSC characteristics. | N/A |
|  |  | ITU-R M.541-9 | Operational Procedures for the Use of Digital Selective-Calling Equipment in the Maritime Mobile Service |  |  |
|  |  | ITU-R M.693 | Technical characteristics of VHF emergency position-indicating radio beacons using digital selective calling (DSC VHF EPIRB) |  |  |
|  |  | ITU-R M.689-2 | International Maritime VHF Radiotelephone Systems with Automatic Facilities Based on DSC Signalling Format |  |  |
|  |  | ITU-R M.821-1 | Optional Expansion of the Digital Selective-Calling System for Use in the Maritime Mobile Service |  |  |
|  | MF/HF | ITU-R M.493-12 | Digital selective-calling system for use in the maritime mobile service. | No amendments are foreseen for the use of HF/MF DSC within the e-Navigation concept as currently envisaged. N/A | N/A |
|  |  | ITU-R M.541-9 | Operational Procedures for the Use of Digital Selective-Calling Equipment in the Maritime Mobile Service |  |  |

| Application |  | Current characteristics |  | Amendments | Mechanism |
| --- | --- | --- | --- | --- | --- |
|  |  | ITU-R M.1173 | Technical characteristics of single-sideband transmitters used in the maritime mobile service for radiotelephony in the bands between 1 606.5 kHz (1 605 kHz Region 2) and 4 000 kHz and between 4 000 kHz and 27 500 kHz |  |  |
|  |  | ITU-R M.1082-1 | International maritime MF/HF radiotelephone system with automatic facilities based on DSC signalling format |  |  |
|  |  | ITU-R M.476-5 | Direct-Printing Telegraph Equipment in the Maritime Mobile Service |  |  |
|  | Satellite | ITU-R M.541-9 | Operational Procedures for the Use of Digital Selective-Calling Equipment in the Maritime Mobile Service | DSC over satellite is not currently defined explicitly through a technical characteristic. However, it could be appropriate to develop a complementary characteristic to those available for VHF and HF. | This task should be coordinated through WP4C/SG4 under existing question 227/4. |
| LRIT | HF |  |  | The use of HF and Satellite data links to support the LRIT application should not need a new technical characteristic, as they are simply applications supported by existing data links. However, there may be merit in amending the existing recommendations in relation to HF and satellite communications to explicitly recognise the use of LRIT. | The subject is on the WRC-11 agenda under Item 1.10. However, there is unlikely to be any directly relevant outcomes. The work would therefore have to be addressed through WP4C/SG4 Questions 84-4/4 (Iridium), 87-4/4 (satellite) coordinating with WP5B/SG5 for HF. |
|  | Satellite |  |  |  |  |
| AIS |  | ITU-R M.1371-3 | Technical characteristics for an automatic identification system using time division multiple access in the VHF maritime mobile band | There are a number of potential changes to AIS under e-Navigation that would require revision to the existing technical characteristics. Firstly, the possible need for a dedicated channel to enable the detection of AIS transmissions from space. Secondly the allocation of additional channels to provide extra capacity for e.g. environmental or security related messaging. Thirdly, the possible need to define new AIS messages such as those relating to channel management. Fourthly, the possible use of the AIS data link as a candidate technology for future maritime VHF data communications could also be a possibility. | WRC-11 agenda item 1.10 is considering the expanded use of VHF for port safety and security. Under this item, the allocation of a 3rd AIS channel for detection from space is being considered. As a result of this item, there could be a need to revise technical characteristics. This task would likely fall to WP5B/SG5 under existing questions 232/5 and 96-2/5. |
| VHF Comms | Voice | ITU-R M.1084-4 | Interim solutions for improved efficiency in the use of the band 156-174 MHz by stations in the maritime mobile service | Under e-Navigation, a general push to make more efficient use of the spectrum available for maritime voice communications may take place as existing channels are increasingly allocated to digital services. The existing characteristics (e.g. M.1084) propose possible interim solutions such as the use of 12.5kHz channel spacing. It may be desirable to develop a new technical characteristic to describe a permanent solutions for e.g. 12.5 kHz or 8.33 kHz voice channels in the future communication environment. | This task should be coordinated through WP5B/SG5 possibly under existing question 96-2/5. |
|  |  | ITU-R M.489-2 | Technical characteristics of VHF radiotelephone equipment operating in the maritime mobile service in channels spaced by 25 kHz |  |  |
|  | Data | ITU-R M.1842 | Characteristics of VHF radio system and equipment for the exchange of data and electronic mail in the maritime mobile service RR Appendix 18 channels | A new technical characteristic will be required to support the use of VHF channels for digital data transmission – assuming that AIS is not selected as the maritime domain datalink of choice. The current characteristic for VHF data exchange refers to the specific solution implemented by Telenor in Norway. If this solution is accepted by the maritime community, this characteristic could be amended. However, in the more likely scenario of a new datalink technology being proposed a new characteristic will be required. | This task should be coordinated through WP5B/SG5 possibly under existing question 96-2/5. |
|  | SAR | ITU-R M.489-2 | Technical characteristics of VHF radiotelephone equipment operating in the maritime mobile service in channels spaced by 25 kHz | No e-Nav related comms change is anticipated for the use of 121.5 MHz for short range SAR purposes. However, were the VHF channels to be revised the SAR associated characteristic (M.489) would in any event be updated. | N/A |
| HF Comms | Voice | ITU-R M.1173 | Technical characteristics of single-sideband transmitters used in the maritime mobile service for radiotelephony in the bands between 1 606.5 kHz (1 605 kHz Region 2) and 4 000 kHz and between 4 000 kHz and 27 500 kHz | No e-Nav related comms change is envisaged for HF voice services. However, if there is a broad encroachment of digital techniques into the band there may be a need to revise existing voice characteristics to accommodate data. | N/A |
|  |  | ITU-R M.1082-1 | International maritime MF/HF radiotelephone system with automatic facilities based on DSC signalling format |  |  |
|  | Data | ITU-R M.1081 | Automatic HF facsimile and data system for maritime mobile users | There are a number of existing technical characteristics relating to data exchange in the HF band that include both outmoded legacy technologies as well as candidate future technologies. There needs to be either an update to M.1798, or else a new characteristic to describe the selected HF digital data scheme to be used under e-Navigation. Currently M.1798 and F.1821 describes multi-channel HF data links such as the system currently deployed by Globe Wireless for commercial services (email, etc.). | The HF channelling requirements are being addressed within WRC-11 agenda item 1.9. The outcome could lead to a need to develop new characteristics in relation to the e-Navigation preferred HF datalink. See WP5B/SG5 and questions 145-2/5 and 158-1/5. |
|  |  | ITU-R M.1798 | Characteristics of HF radio equipment for the exchange of digital data and electronic mail in the maritime mobile service |  |  |
|  |  | ITU-R M.627-1 | Technical characteristics for HF maritime radio equipment using narrow-band phase-shift keying (NBPSK) telegraphy |  |  |
|  |  | ITU-R F.1821 | Characteristics of advanced digital high frequency (HF) radiocommunication systems |  |  |
| NAVTEX / NBDP | MIO | ITU-R M.625-3 | Direct-Printing Telegraph Equipment Employing Automatic Identification in the Maritime Mobile Service | If under e-Navigation the NAVTEX data is carried across different communication links there may be a need to provide a new characteristic to accommodate it. However, if it is intended to include NAVTEX data along with other safety related information e.g. Maritime Information Objects, etc. then it may be more appropriate for a specific characteristic to encompass all maritime safety information. | This task should be coordinated through WP5B/SG5 possibly under existing question 98-5. |
|  |  | ITU-R M.540-2 | Operational and Technical Characteristics for an Automated Direct- Printing Telegraph System for Promulgation of Navigational and Meteorological Warnings and Urgent Information to Ships - Section 8B - Maritime Mobile Service; Telegraphy and Related Subjects. |  |  |
|  |  | ITU-R M.688 | Technical characteristics for a high frequency direct-printing telegraph system for promulgation of high seas and NAVTEX-type maritime safety information |  |  |
|  |  | ITU-R M.492-6 | Operational procedures for the use of direct-printing telegraph equipment in the maritime mobile service |  |  |
|  |  | ITU-R M.476-5 | Direct-Printing Telegraph Equipment in the Maritime Mobile Service |  |  |
| On board Comms | UHF | ITU-R M.1174-2 | Technical characteristics of equipment used for on-board vessel communications in the bands between 450 and 470 MHz | There is a need to update the existing characteristics to support the implementation of digital PMR. | This task should be coordinated through WP5B/SG5 most likely under a new question. |
|  | WiFi | ITU-R F.2086 | Technical and operational characteristics and applications of broadband wireless access in the fixed service | If WiFi stations are to be adopted for on-board data communications there may be a need for a new characteristic associated with is use in the maritime mobile domain. | This task should be coordinated through WP5B/SG5 most likely under a new question. |
| Satellite Comms | Broadband | ITU-R S.1709 | Technical characteristics of air interfaces for global broadband satellite systems | Broadband over satellite is an existing application of current satellite systems and does not warrant a specific characteristic for the maritime domain | N/A |
|  | MSI | ITU-R S.1709 | Technical characteristics of air interfaces for global broadband satellite systems | The transmission of MSI over broadband may warrant a characteristic to define the data layers. | This task should be coordinated through WP4C/SG4 under existing question 87-4/4. |
| DGNSS |  | ITU-R M.823-3 | Technical characteristics of differential transmissions for global navigation satellite systems from maritime radio beacons in the frequency band 283.5-315 kHz in Region 1 and 285-325 kHz in Regions 2 and 3 | The current characteristic M.823-3 will ultimately need to be updated to accommodate new developments in GNSS such as Galileo and COMPASS. | This task should be coordinated through WP5B/SG5. |
| Port Services | WiMax / 2G / 3G / 4G |  |  | The maritime community will make use of technologies developed and standardised for other applications in port. No maritime specific characteristics are required. | N/A |
| Inter-ship Broadband | WiMax | ITU-R M.1801 | Radio interface standards for broadband wireless access systems, including mobile and nomadic applications, in the mobile service operating below 6 GHz | Under e-Navigation, ad-hoc mesh WiMax networks could be deployed to provide broadband coastal networks. This would require a specific form of implementation of WiMax that would necessitate a specific technical characteristic. | This task should be coordinated through WP5B/SG5 possibly under existing question 212-3/5. |
| Loran | eLoran data channel | ITU-R M.589-3 | Technical characteristics of methods of data transmission and interference protection for radionavigation services in the frequency bands between 70 and 130 kHz | The existing technical characteristic will need to be revised to accommodate the definitive 9th pulse, Eurofix and/or new modulation schemes selected for the eLoran data channel. | This task should be coordinated through WP5B/SG5 under a new question. |
| Radar | SART | ITU-R M.628-4 | Technical Characteristics for Search and Rescue Radar Transponders | Relates to 9 GHz SART, so no change required. | N/A |
|  | Racon | ITU-R M.824-3 | Technical parameters of radar beacons (Racons) | No e-Nav related comms change. | N/A |
|  | Radio determination | ITU-R M.1460 | Technical and operational characteristics and protection criteria of radiodetermination radars in the 2 900-3 100 MHz band | No e-Nav related comms change. | N/A |
| EPIRB | VHF | ITU-R M.690-1 | Technical characteristics of emergency position-indicating radio beacons (EPIRBs) operating on the carrier frequencies of 121.5 MHz and 243 MHz | No e-Nav related comms change. | N/A |
|  | Satellite (COSPAS / SARSAT) | ITU-R M.633-3 | Transmission characteristics of a satellite emergency position-indicating radio beacon (satellite EPIRB) system operating through a satellite system in the 406 MHz band | No e-Nav related comms change. | N/A |
|  | Satellite | ITU-R M.632-3 | Transmission Characteristics of a Satellite Emergency Position Indicating Radio Beacon (Satellite EPIRB) System Operating Through Geostationary Satellites in the 1.6 GHz Band | No e-Nav related comms change. | N/A |

http://www.globewireless.com/network_digitalhf.php

The self contained printer requirement was removed from IMO performance standards (MSC.148(77)) subject to provide a printer port for the installations on or after 1 July 2005.

Inmarsat B, C, F77 are elements of GMDSS

See ITU-R Radio Regulations Appendix 15 (Rev.WRC-07).

COMSAR 12/15 §4.26.

“User pull”, such as accessed through a web site.

“Provider push”, such as a NAVTEX or SafetyNET broadcast, or an announcement on VHF channel 16.

Industrial, Scientific and Medical.  These bands are used by devices such as industrial heaters and driers and domestic equipments such as microwave ovens.

According to 3GPP TS 45.005