IALA TECHNICAL CLARIFICATIONS

ON

RECOMMENDATION ITU-R M.1371-3

Edition 2.3

Table of Contents

1	Introduction	5

2	Technical Clarifications of ITU-R M.1371-3	7

2.1	A1; §2.1, AIS VHF data link (VDL) non-controlling stations	8

2.2	A2; § 2.2 Transceiver characteristics	8

2.3	A 2; § 2.11 Transmitter transient response	9

2.4	A 2; § 3.1.6 Slot State	10

2.5	A 2; § 3.2.2.9 Summary of the default transmission packet	11

2.6	A 2; § 3.3.1.2 Candidate slots	12

2.7	A 2; § 3.3.4.3.1 FATDMA algorithm	13

2.8	A4;	14

2.9	A5: § 1.2  Definition of application identifiers	16

2.10	A5: § 3.2  Regional functional messages	17

2.11	A5: § 4  Guidelines for drafting functional messages (FM)	17

2.12	A7: § 4.3.3.6  VDL message use	18

2.13	A8: § 2  Message Summary	19

2.14	A8; § 3.1  Messages 1, 2, 3: Position reports	19

2.15	A8; § 3.2 Message 4: Base station report /  Message 11: UTC and date response	21

2.16	A8; §3.3. Message 5: Ship static and voyage related data	23

2.17	A8; § 3.7  Message 9: Standard SAR aircraft position report	26

2.18	A8; § 3.12 Message 14: Safety Related broadcast message	28

2.19	A8; § 3.16 Message 18: Standard Class B equipment position report	29

2.20	A8; §3.17 Message 19: Extended Class B equipment position report	31

2.21	A 8; § 3.18 Message 20: Data link management message	32

2.22	A8; § 3.19 Message 21: Aids-to-navigation report (AtoN)	33

2.23	A8; §3.21 Message 23: Group Assignment Command	36

2.24	A8; § 3.22 Message 24: Static data report	38

2.25	A8; § 3.25 Message 27: Long Range AIS Broadcast Message	40

2.26	A 9; Requirements for stations using burst transmissions	42

2.27	A 10 (previous A 9); Abbreviations in Recommendation ITU-R M. 1371-3	45

3	Appendix A ; AIS message for static information from SAR aircrafts	46

4	Editorial corrections	49

Technical Clarifications of

Recommendation ITU-R M.1371-3

Edition 2.3

# Introduction

  - The purpose of AIS is to improve the maritime safety and efficiency of navigation, safety of life at sea and the protection of the marine environment.

  - It has long been realized that an automatic reporting device fitted to a ship would have the potential to increase significantly the safety of navigation and would allow the improved control and monitoring of the maritime environment.  Taking up this challenge, the International Maritime Organization (IMO) together with the International Telecommunications Union (ITU) and the International Electrotechnical Commission (IEC) developed a new navigation system called the Automatic Identification System (AIS).

  - Recommendation ITU-R M.1371-3 is the document that describes the technical characteristics for an Automatic Identification System using Time Division Multiple Access in the VHF Maritime Mobile Band. As with any technical document some concepts are subject to interpretation. This document presents a series of clarifications for some of these concepts as agreed by the IALA e-NAV Committee.

  - Figure 1 (attached) illustrates the inter-relationship of this document to IMO SOLAS Chapter V; the IMO recommendation on Performance Standards for a Universal Shipborne Automatic Identification System (AIS), (MSC.74(69)); the IMO Guidelines for the Onboard Operational Use of the Shipborne AIS (A.917(22)); the ITU Recommendation on the Technical Characteristics for an Automatic Identification System (AIS) using Time Division Multiple Access in the Maritime Mobile Band (ITU-R M.1371-3); and the IEC Standard 61993-Part 2: Universal Shipborne Automatic Identification System (AIS) Operational Performance Requirements, Methods of testing and required test results; the IALA Guidelines on Automatic Identification Systems (AIS).

  - As shown in Figure 1, each of the referenced documents in paragraph 1.4 including this document has a specific focus in relation to an overall AIS as follows.

This document, "Recommendation on Technical Clarification of ITU-1371-3", deals specifically with the VHF Data Link (VDL) of an AIS. It acknowledges the fact that some of the specific technical paragraphs in ITU-R M.1371-3 might require interpretations as to their precise, intended meaning.  Consequently, this document presents a set of unifying recommendations that will, hopefully, result in a standardized application of these paragraphs by potential manufacturers, system integrators as well as users of the systems.  Adherence to these recommendations will ensure the viability of the VDL and the interoperability among AIS units obtained from different sources.

  - However, this document also covers some other specific, technical details of various possible AIS station or device configurations that are only mentioned in general terms in ITU-R M.1371.

  - All of the identified documents have a specific focus and address specific user needs. Nevertheless, IALA also recognized that there is a need for a high level document that consolidates all of technical and operational aspects of the sub-systems described in Figure 1.  Consequently, IALA is at this time also drafting a general IALA Guideline on AIS  which will incorporate all of the elements in Figure 1 in a descriptive manner.

| IALA Documents |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NAVGUIDE [AIS Manual] |  |  |  |  |  |  |  |  |  |  |  |
| Guidelines 1028-1029 AIS as an e-Nav service |  | Guideline 1059 Comparison of AIS stations |  |  |  |  |  | Guideline 1062 Establishment AIS as AtoN |  | Guideline xxx Future guidelines |  |
| A-123 Provision of shore based AIS | A-124 AIS Service |  |  |  |  | A-126 AIS in marine aids to navigation |  |  |  | A-yyy Future recommendations |  |
|  | Annex FATDMA planning |  | Annex …. |  | Annex …. |  |  |  |  |  |  |
| Non IALA Documents |  |  |  |  |  |  |  |  |  |  |  |
| IEC 61993-2 Class A | IEC 62287-1/-2 Class B “CS”/”SO” |  |  | IEC 62320-1 Base station |  |  | IEC 62320-2 AtoN station |  | IEC 61097-14 AIS-SART |  | IEC zzz Future standards |
| ITU-R documentation on AIS, e.g. ITU-R M.1371-3 |  |  |  |  |  |  |  |  |  |  | IALA Technical Clarifications on 1371-3 |
| IMO documentation on AIS, e.g. AIS Performance standards |  |  |  |  |  |  |  |  |  |  |  |

Figure 1  Documents Matrix

# Technical Clarifications of ITU-R M.1371-3

Important General Remark

Every clarification of this Chapter is presented in a uniform way:

- Every clarification is introduced in the appropriate context of Recommendation ITU-R M.1371-3, which is always quoted as a whole section. This is done to ensure, that all clarifications are to be understood in their appropriate context.

- The verbatim text of Recommendation ITU-R M.1371-3 is given in italics; additions by the clarification are given upright and underlined as of previous clarification editions and upright and double underlined in the present edition. Deleted text portions are identified with strike-out as of previous clarification editions and double strike out in the present edition.

- For every clarification a rationale is given to allow complete understanding as to why a clarification was needed also. Eventually, the clarifications will be incorporated into a future revision of Recommendation ITU-R M.1371-3).

- A date of when the clarification was drafted is given.

- A comment contains additional information on the clarification itself. For instance, some clarifications have already been incorporated in the first edition of the IEC standard for Class A shipborne mobile AIS stations (IEC 61993-2 latest edition). A future substantial change of the clarification result in the creation of a legacy issue. Any future change for those clarifications should therefore be done with the utmost caution.

## A1; §2.1, AIS VHF data link (VDL) non-controlling stations

### Proposed Clarifying Text:

2.1.1	AIS shipborne station

2.1.1.1	Class A shipborne mobile equipment using SOTDMA technology as described in Annex 2 will comply with relevant IMO AIS carriage requirement:

2.1.1.2	Class B shipborne mobile equipment will provide facilities not necessarily in full accordance with IMO AIS carriage requirement.

–	Class B “SO” using SOTDMA technology as described in Annex 2;

–	Class B “CS” using CSTDMA as described in Annex 7.

2.1.2	Aids to navigation-AIS station

2.1.3	Limited base station (no VDL control functionality)

2.1.4	Search and rescue mobile aircraft equipment

The AIS SAR aircraft station shall transmit position report message 9, and static data using message 5 and and message 24A and 24B as described in Annex 8

2.1.5	Repeater station

2.1.6	AIS_Search and Rescue Transmitter (AIS-SART station)
The AIS SART station shall transmit message 1 and message 14 using the burst transmissions as described in Annex 9.

The messages 1and 14  shall use a the user ID 970xxyyyy (where xx = manufacturer ID 01 to 99; yyyy = the sequence number 0000 to 9999) and Navigational Status 14.

The message 14 shall have the following content:

When active:	SART ACTIVE

Under test: 	SART TEST

### Rationale for Clarification:

Introduce AIS SAR aircraft station

Define AIS SART station

### Date of amendment: 16 January 2009

### Comment

## A2; § 2.2 Transceiver characteristics

### Proposed Clarifying Text:

TABLE 6

Definitions of timing for Fig. 2

| Reference |  | Bits | Time (ms) | Definition |
| --- | --- | --- | --- | --- |
| T0 |  | 0 | 0 | Start of transmission slot. Power shall NOT exceed –50 dB of Pss before T0 |
| TA |  | 0-6 | 0-0,624 0-0,625 | Power exceeds –50 dB of Pss |
| TB | TB1 | 6 | 0,624 0,625 | Power shall be within +1.5 or –3 dB of Pss |
|  | TB2 | 8 | 0,8324 0,833 | Power shall be within +1.5 or –1 dB of Pss (start of Training sequence) |
| TE (includes 1 stuffing bit) |  | 233 | 24,024 24,271 | Power shall remain within +1.5 or –1 dB of Pss during the period TB2 to TE |
| TF (includes 1 stuffing bit) |  | 241 | 26,146 25,104 | Power shall be –50 dB of Pss and stay below this |
| TG |  | 256 | 26,624 26,667 | Start of next transmission time period |

### Rationale for Clarification:

Correct definition of  slot timing. The interpretation of the specification used bit 6 instead of bit 8, for the training sequence start, and consequently TE and TF have been incremented by 2 and the slot times adjusted to suit.

### Date of amendment: 22 September 2009

### Comment

## A 2; § 2.11 Transmitter transient response

### Proposed Clarifying Text:

Figure 2

Transmitter output envelope versus time

### Rationale for Clarification:

Training sequence start incorrect

### Date of amendment: 22 September 2009

### Comment

## A 2; § 3.1.6 Slot State

### Proposed Clarifying Text:

Each slot can be in one of the following states:

Free: meaning that the energy level should be <= 10 dB above the background noise level. The energy level is  determined by examining the first two milliseconds of the slot. The background noise is measured over a rolling 60 s interval on each Rx channel separately. The minimum noise level should be –117 dBm and should be tracked for a range of at least 50 dB (which results in a maximum noise level of –67 dBm) 1

- slot is unused within the receiving range of the own station. Externally allocated slots that have not been used during the preceding three frames are also Free slots. This slot may be considered as a candidate slot for use by own station (see § 3.3.1.2).

–	Internal allocation: meaning that the slot is allocated by own station and can be used for transmission.

–	External allocation: meaning that the slot is allocated for transmission by another station.

–	Available: meaning that the slot is externally allocated by a station or was determined not to be a free slot due to the energy level in the slot and is a possible candidate for slot reuse (see § 4.4.1).

–	Unavailable: meaning that the slot is externally allocated by a station and cannot be a candidate for slot reuse (see § 4.4.1).

1The following example is compliant with the requirement:

Sample the RF signal strength at a rate >1 kHz, average the samples over a sliding 20 ms period and over a 4 s interval determine the minimum period value. Maintain a history of 15 such intervals. The minimum of all 15 intervals is the background level. Add a fixed 10 dB offset to give the free slot detection threshold.

### Rationale for Clarification:

To clarify the definition of a free slot under garbled conditions

### Date of amendment: 24 September 2009

### Comment

## A 2; § 3.2.2.9 Summary of the default transmission packet

### Proposed Clarifying Text:

TABLE  12

| Ramp up | 8 bits | T0 to T1TS in Fig. 8 |
| --- | --- | --- |
| Training sequence | 24 bits | Necessary for synchronization |
| Start flag | 8 bits | In accordance with HDLC (7Eh) |
| Data | 168 bits | Default |
| CRC | 16 bits | In accordance with HDLC |
| End flag | 8 bits | In accordance with HDLC (7Eh) |
| Buffer | 24 bits | Bit stuffing distance delays, repeater delay and jitter |
| Total | 256 bits |  |

### Rationale for Clarification:

Termination of ramp up incorrectly defined as T1 – corrected to TTS

### Date of amendment: 22 September 2009

### Comment

## A 2; § 3.3.1.2	Candidate slots

### Proposed Clarifying Text:

### Rationale for Clarification:

Slot timing calculated figures in table were incorrect (inserted new Figure)

### Date of amendment: 22 September 2009

### Comment

## A 2; § 3.3.4.3.1 FATDMA algorithm

### Proposed Clarifying Text:

Access to the data link should be achieved with reference to frame start. Each allocation should be pre-configured by the competent authority and not changed for the duration of the operation of the station, or until re-configured. Except where the time-out value is otherwise determined, receivers of Data Link Management Message (message 20) FATDMA messages should set a slot time-out value in order to determine when the FATDMA slot will become free. The slot time-out should be reset with each reception of the message.

FATDMA reservations should consist of a Base Station Report (message 4) in conjunction with a Data Link Management Message (message 20) with the same base station ID (MMSI).  FATDMA reservations apply within a range of 120 nautical miles from the reserving base station. AIS stations (except when using FATDMA) should not use FATDMA reserved slots within this range.

A Data Link Management Message (message 20) without  a Base Station Report (message 4) should be ignored.

Base stations may reuse FATDMA reserved slots within this range for their own FATDMA transmissions but may not reuse FATDMA reserved slots for RATDMA transmissions.

FATDMA reservations do not apply beyond 120 nautical miles from the reserving base station. All stations may consider these slots as available.

### Rationale for Clarification:

Provide clarification for AIS station behaviour regarding the 120 nm rule

### Date of amendment: 16 January 2009

### Comment

## A4;

### Proposed Clarifying Text:

Annex 4

Long-range applications

1	General

Long range applications should be by interface to other equipment and by broadcast.

2	Long range applications by interface to other equipment

Class A shipborne mobile equipment should provide a two-way interface for equipment which provides for long-range communications. This interface should comply with IEC 61162 series.

Applications for long-range communications should consider that:

–	The long-range application of AIS must operate in parallel with the VDL. Long-range operation will not be continuous. The system will not be designed for constructing and maintaining real-time traffic images for a large area. Position updates will be in the order of 2-4 times per hour (maximum). Some applications require an update of just twice a day. It can be stated that long-range application forms hardly any workload to the communication system or the transponder and will not interfere with the normal VDL operation.

–	The long-range operational mode will be on interrogation basis only for geographical areas. Base stations shall interrogate AIS systems, initially by geographical area, followed by addressed interrogation. Only AIS information will be included in the reply; e.g. position and static and voyage-related data.

–	The communication system for long-range AIS is not defined in this Recommendation. Inmarsat-C, as part of global maritime distress and safety system (GMDSS) on many vessels, can be a candidate to facilitate the long-range application, but this will not be mandatory. Most of the current Inmarsat-C, but also all other long-range communication systems, do not support the IEC 61162-2 interface. Because the IEC 61162 series will be standard on all future maritime on-board systems, the AIS will be supported by this interface only. This requires for long-range application an active interface box to translate the long-range AIS 61162-2 messages to the required messages suitable for the chosen communication system and vice versa. This active interface can also gather the information which is not available as standard in the AIS. This can be another information system aboard (if installed).

Example configuration:

Operation with Inmarsat-C.

The general set-up of the long-range configuration is in Fig. 25.

Because of the lack of IEC 61162-2 interfaces on long-range communication systems, the configuration shown in Fig. 26 can be used as an interim solution.

3	Long Range Applications by Broadcast

Long range AIS receiving systems may receive long range AIS broadcast messages, provided these messages are appropriately structured and transmitted to suit the receiving systems.

3.1	Packet bit structure for the AIS long range broadcast message

Long range AIS receiving systems require suitable buffering in order to preserve the integrity of the AIS message in the AIS slot boundaries. Table 1 shows a modified packet bit structure that is designed to support reception of AIS messages by satellites with orbital altitudes up to 1000 km.

Annex 4 TABLE 1

Modified packet bit structure for long range AIS message reception

| Slot composition | Bits | Notes |
| --- | --- | --- |
| Ramp up | 8 | Standard |
| Training sequence | 24 | Standard |
| Start flag | 8 | Standard |
| Data field | 96 | Data field is 168 bits for other single-slot AIS messages. This field is shortened by 72 bits to support the long range receiving system buffer |
| CRC | 16 | Standard |
| End flag | 8 | Standard |
| Long range AIS receiving system buffer | 96 | Bit stuffing = 4 bits Synch jitter (mobile station) = 3 bits Synch jitter (mobile/satellite) = 1 bit Propagation time delay difference = 87 bits Spare = 1 bit |
| Total | 256 | Standard (NOTE – Only 160 bits are used in the 17 millisecond transmission) |

3.2	Long range AIS broadcast message

The long range AIS broadcast message – Message 27 - data field is shown in Annex 8 Table 81

3.3	Transmission method for the long range AIS broadcast message

The long range AIS broadcast message should be transmitted by the AIS Class A stations at the current power setting using the transmission interval, access scheme, AIS shore station qualifier and designated frequencies specified below. The AIS Class A should perform the long range AIS broadcast as a transmit-only function.

3.3.1	Transmission interval

The nominal transmission interval for the long range AIS broadcast message should be 3 minutes.

3.3.2	Access scheme

The access scheme for transmitting the long range AIS broadcast message should be RATDMA. The AIS should consider only the two frequencies in the AIS VDL for qualifying the candidate slots for the transmission of the long range AIS message. The AIS VDL is comprised only of channel A (by default AIS 1) and channel B (by default AIS 2). The long range AIS broadcast on the long range frequencies should not be considered part of the AIS VDL for the purpose of scheduling the RATDMA transmissions of the long range AIS broadcast messages.

3.3.3	AIS shore station qualifier

Transmission of the long range AIS broadcast message should be suppressed when the AIS Class A station is in range of an AIS base station. This should be implemented by reset of the 3-minute timer for the nominal transmission interval upon receipt of a Message 4 (the AIS base station message).

3.3.4	Transmitting the long range broadcast message

The long range AIS broadcast message should be transmitted on two channels. The transmissions should alternate between these two channels such that each channel is used once every 6 minutes.

### Rationale for Clarification:

Additional material added from  ITU-R07-WP5B-C-0296!N01!MSW-E

### Date of amendment: 22 September 2009

### Comment

## A5: § 1.2	 Definition of application identifiers

### Proposed Clarifying Text:

The DAC is a 10-bit number. DAC assignments are:

–	international (DAC = 1-9), maintained by international agreement for global use;

–	regional (DAC > 10), maintained by the regional authorities affected; and

–	test (DAC = 0), used for test purposes.

It is recommended that DAC 2-9 be used to identify subsequent versions of international specific messages and that the administrator of application specific messages base the DAC selection on the maritime identification digit (MID) of the administrator’s country or region. It is the intention that any application specific message can be utilized worldwide. The choice of the DAC does not limit the area where the message can be used.

### Rationale for Clarification:

Requirement to include a method of version identification in application messages

### Date of amendment: 24 September 2009

### Comment

## A5: § 3.2	 Regional functional messages

### Proposed Clarifying Text:

The following should be considered when creating regional functional messages:

–	published regional and international functional messages;

–	legacy and compatibility issues with current, superseded or obsolete message structures (e.g. 3-bit FI version indicator);

–	period of time needed and cost to formally introduce a new functionality;

–	each functional message should have a unique identifier (AI);

–	limited number of functional identifiers allocated for local, regional, national, or multinational use;

–	requirements for encrypted messages.

### Rationale for Clarification:

Requirement to include a method of version identification in application messages

### Date of amendment: 24 September 2009

### Comment

## A5: § 4	 Guidelines for drafting functional messages (FM)

### Proposed Clarifying Text:

When developing functional messages, the following should be considered:

–	a message for test and evaluation purposes to ensure integrity in an operational system;

–	rules given in § 3.3.7, Annex 2 (Message structure), and § 3, Annex 8 (Message descriptions);

–	values for not available, normal, or malfunctioning should be defined for every data field, as appropriate;

–	default values should be defined for each data field.

When position information is included, in addition to latitude and longitude, and if applicable, should comprise data fields in the following order (see AIS Messages 1 and 5):

–	position accuracy;

–	longitude;

- latitude;

- precision

–	type of electronic position fixing device;

- time stamp.

### Rationale for Clarification:

Requirement to specify the precision (number of decimal places) of the latitude and longitude that should be applied by the recipient of the message when considering the position information

### Date of amendment: 24 September 2009

### Comment

## A7: § 4.3.3.6	 VDL message use

### Proposed Clarifying Text:

(Message 27 added to end of Table 39 as shown below)

TABLE 39

Use of VDL messages by a Class B “CS” AIS

| Message No. | Name of message | Annex 8 reference | Receive and process (1) | Transmit by own station | Remark |
| --- | --- | --- | --- | --- | --- |
| 25 | Single slot binary message | § 3.23 | Optional | No |  |
| 26 | Mult. slot binary message with Communications State | § 3.24 | No | No |  |
| 27 | Position report for long range applications | § 3.25 | No | No |  |
| 278-63 | Undefined | None | No | No | Reserved for future use |
| (1) “Receive and process” in this table means functionality visible for the user, e.g. output to an interface or display. For synchronization it is necessary to receive and internally process messages according to § 4.3.1.1; this applies to Messages 1, 2, 3, 4, 18, 19. |  |  |  |  |  |

### Rationale for Clarification:

Message 27 missing from Table

### Date of amendment: 22 September 2009

### Comment

## A8: § 2  Message Summary

### Proposed Clarifying Text:

(Message 27 added to end of Table 43 as shown below)

TABLE 43

| Message ID | Name | Description | Priority | Access scheme | Communi-cation state | M/B |
| --- | --- | --- | --- | --- | --- | --- |
| 26 | Multiple slot binary message with Communications State | Scheduled binary data transmission (Broadcast or addressed) | 4 | SOTDMA, RATDMA, ITDMA | SOTDMA, ITDMA | M/B |
| 27 | Position report for long range applications | Scheduled position report; (Class A shipborne mobile equipment outside base station coverage) | 1 | RATDMA, | N/A | M |

### Rationale for Clarification:

Message 27 missing from Table

### Date of amendment: 22 September 2009

### Comment

## A8; § 3.1  Messages 1, 2, 3: Position reports

### Proposed Clarifying Text:

TABLE 45

| Parameter |  | Number of bits |  | Description |
| --- | --- | --- | --- | --- |
| Message ID |  | 6 |  | Identifier for this message 1, 2 or 3 |
| Repeat indicator |  | 2 |  | Used by the repeater to indicate how many times a message has been repeated. See § 4.6.1, Annex 2; 0-3; 0 = default; 3 = do not repeat any more |
| User ID |  | 30 |  | Unique identifier such as MMSI number |
| Navigational status |  | 4 |  | 0 = under way using engine, 1 = at anchor, 2 = not under command, 3 = restricted manoeuvrability, 4 = constrained by her draught, 5 = moored, 6 = aground, 7 = engaged in fishing, 8 = under way sailing, 9 = reserved for future amendment of navigational status for ships carrying DG, HS, or MP, or IMO hazard or pollutant category C, high speed craft (HSC), 10 = reserved for future amendment of navigational status for ships carrying dangerous goods (DG), harmful substances (HS) or marine pollutants (MP), or IMO hazard or pollutant category A, wing in grand (WIG); 11-14 = reserved for future use, 15 = not defined = default 0 = under way using engine (Rule 23(a)) or Rule 25(e)), 1 = at anchor (Rule 30(a-c)), 2 = not under command (Rule 27(a)), 3 = restricted manoeuvrability (Rule 27(b-h)), 4 = constrained by her draught (Rule 28), 5 = moored, 6 = aground (Rule 30(d)), 7 = engaged in fishing-trawling (Rule 26(b)), 8 = under way sailing (Rule 25), 9 = engaged in fishing other than trawling (Rule 26(c)), 10 = air-cushion vessel in non-displacement mode or WIG craft taking off, landing or in flight (Rule23(b-c)), 11 = power-driven vessel towing astern (Rule 24(a)), 12 = power-driven vessel pushing ahead or towing alongside (Rule 24(b)), 13= in distress or requiring assistance (Rule 37), 14 = AIS-SART (active), seeking to attract attention (Rule 36), 15 = not defined = default (also used by AIS-SART under test) |
| Rate of turn ROTAIS | 8 |  | 0 to +126 = turning right at up to 708° per min or higher 0 to –126 = turning left at up to 708° per min or higher Values between 0 and 708° per min coded by ROTAIS = 4.733 SQRT(ROTsensor) degrees per min where ROTsensor is the Rate of Turn as input by an external Rate of Turn Indicator (TI). ROTAIS is rounded to the nearest integer value. +127 = turning right at more than 5º per 30 s (No TI available) –127 = turning left at more than 5º per 30 s (No TI available) –128 (80 hex) indicates no turn information available (default). ROT data should not be derived from COG information. |  |
| SOG | 10 |  | Speed over ground in 1/10 knot steps (0-102.2 knots) 1 023 = not available, 1 022 = 102.2 knots or higher |  |
| Position accuracy | 1 |  | The position accuracy (PA) flag should be determined in accordance with Table 1 = high (< > 10 m) 0 = low (> < 10 m) 0 = default |  |
| Longitude | 28 |  | Longitude in 1/10 000 min (180º, East = positive (as per 2’s complement), West = negative (as per 2’s complement). 181= (6791AC0h) = not available = default) |  |
| Latitude | 27 |  | Latitude in 1/10 000 min (±90°, North = positive (as per 2’s complement), South = negative (as per 2’s complement). 91° (3412140h) = not available = default) |  |
| COG | 12 |  | Course over ground in 1/10 = (0-3599). 3600 (E10h) = not available = default. 3 601-4 095 should not be used |  |
| True heading | 9 |  | Degrees (0-359) (511 indicates not available = default) |  |

| Time stamp | 6 | UTC second when the report was generated by the electronic position system (EPFS) (0-59, or 60 if time stamp is not available, which should also be the default value, or 61 if positioning system is in manual input mode, or 62 if electronic position fixing system operates in estimated (dead reckoning) mode, or 63 if the positioning system is inoperative) |
| --- | --- | --- |
| special manoeuvre indicator | 2 | 0 = not available = default 1 = not engaged in special manoeuvre 2 = engaged in special manoeuvre (i.e.: regional passing arrangement on Inland Waterway) |
| Spare | 3 | Not used. Should be set to zero. Reserved for future use. |
| RAIM-flag | 1 | Receiver autonomous integrity monitoring (RAIM) flag of electronic position fixing device; 0 = RAIM not in use = default; 1 = RAIM in use. See Table |
| Communication state | 19 | See Table 46 |
| Number of bits | 168 |  |

### Rationale for Clarification:

- Describe user ID as a unique identifier

- Define Navigational Status for AIS-SART station

- Correct position accuracy flag definition

### Date of amendment: 16 January 2009

### Comment

## A8; § 3.2	Message 4: Base station report /  Message 11: UTC and date response

### Proposed Clarifying Text:

Should be used for reporting UTC time and date and, at the same time, position. A base station should use Message 4 in its periodical transmissions. A mobile station should output Message 11 only in response to interrogation by Message 10.

Message 11 is only transmitted as a result of a UTC request message (Message 10). The UTC and date response should be transmitted on the channel, where the UTC request message was received.

TABLE 48

| Parameter | Number of bits | Description |
| --- | --- | --- |
| Message ID | 6 | Identifier for this message 4 or 11 4 = UTC and position report from base station 11 = UTC and position response from mobile station |
| Repeat indicator | 2 | Used by the repeater to indicate how many times a message has been repeated. Refer to § 4.6.1, Annex 2; 0-3; 0 = default; 3 = do not repeat any more |
| User ID | 30 | MMSI number |
| UTC year | 14 | 1-9999; 0 = UTC year not available = default |
| UTC month | 4 | 1-12; 0 = UTC month not available = default; 13-15 not used |
| UTC day | 5 | 1-31; 0 = UTC day not available = default |

TABLE 48 (end)

| Parameter | Number of bits | Description |
| --- | --- | --- |
| UTC hour | 5 | 0-23; 24 = UTC hour not available = default; 25-31 not used |
| UTC minute | 6 | 0-59; 60 = UTC minute not available = default; 61-63 not used |
| UTC second | 6 | 0-59; 60 = UTC second not available = default; 61-63 not used |
| Position accuracy | 1 | 1 = high (< > 10 m) 0 = low (> < 10 m) 0 = default The PA flag should be determined in accordance with Table 47 |
| Longitude | 28 | Longitude in 1/10 000 min (180º, East = positive (as per 2’s complement), West = negative (as per 2’s complement); 181= (6791AC0h) = not available = default) |
| Latitude | 27 | Latitude in 1/10 000 min (90º, North = positive (as per 2’s complement), South = negative (as per 2’s complement); 91 = (3412140h) = not available = default) |
| Type of electronic position fixing device | 4 | Use of differential corrections is defined by field position accuracy above: 0 = undefined (default) 1 = global positioning system (GPS) 2 = GNSS (GLONASS) 3 = combined GPS/GLONASS 4 = Loran-C 5 = Chayka 6 = integrated navigation system 7 = surveyed 8 = Galileo 9-1415 = not used 15 = internal GNSS |
| Spare | 10 | Not used. Should be set to zero. Reserved for future use |
| RAIM-flag | 1 | RAIM (Receiver autonomous integrity monitoring) flag of electronic position fixing device; 0 = RAIM not in use = default; 1 = RAIM in use see Table 47 |
| Communication state | 19 | SOTDMA communication state as described in § 3.3.7.2.1, Annex 2 |
| Number of bits | 168 |  |

### Rationale for Clarification:

a) Correct position accuracy flag definition

b) Provide information about AIS mobile station internal GNSS receiver is in use for position reporting

### Date of amendment: 13March 2008

### Comment

.No comment.

## A8; §3.3. Message 5: Ship static and voyage related data

### Proposed Clarifying Text: Should only be used by Class A shipborne and SAR aircraft AIS stations when reporting static or voyage related data.

TABLE 49

| Parameter | Number of bits | Description |
| --- | --- | --- |
| Message ID | 6 | Identifier for this Message 5 |
| Repeat indicator | 2 | Used by the repeater to indicate how many times a message has been repeated. Refer to § 4.6.1, Annex 2; 0-3; 0 = default; 3 = do not repeat any more |
| User ID | 30 | MMSI number |
| AIS version indicator | 2 | 0 = station compliant with Recommendation ITU-R M.1371-1 1 = station compliant with Recommendation ITU-R M.1371-3 2-3 = station compliant with future editions |
| IMO number | 30 | 1-999999999; 0 = not available = default – Not applicable to SAR aircraft |
| Call sign | 42 | 7 = 6 bit ASCII characters, @@@@@@@ = not available = default |
| Name | 120 | Maximum 20 characters 6 bit ASCII, as defined in Table 44 “@@@@@@@@@@@@@@@@@@@@” = not available = default For SAR aircraft, it should be set to “SAR AIRCRAFT NNNNNNN” where NNNNNNN equals the aircraft registration number. |
| Type of ship and cargo type | 8 | 0 = not available or no ship = default 1-99 = as defined in § 3.3.2 100-199 = reserved, for regional use 200-255 = reserved, for future use Not applicable to SAR aircraft |
| Overall dimension/ reference for position | 30 | Reference point for reported position. Also indicates the dimension of ship (m) (see Fig. 42 and § 3.3.3) For SAR Aircraft, the use of this field may be decided by the responsible administration. If used it should indicate the maximum dimensions of the craft. As de-fault should A=B=C=D be set to “0” |
| Type of electronic position fixing device | 4 | 0 = undefined (default) 1 = GPS 2 = GLONASS 3 = combined GPS/GLONASS 4 = Loran-C 5 = Chayka 6 = integrated navigation system 7 = surveyed 8 = Galileo, 9-1415 = not used 15 = internal GNSS |
| ETA | 20 | Estimated time of arrival; MMDDHHMM UTC Bits 19-16: month; 1-12; 0 = not available = default Bits 15-11: day; 1-31; 0 = not available = default Bits 10-6: hour; 0-23; 24 = not available = default Bits 5-0: minute; 0-59; 60 = not available = default For SAR aircraft, the use of this field may be decided by the responsible administration |
| Maximum present static draught | 8 | In 1/10 m, 255 = draught 25.5 m or greater, 0 = not available = default; in accordance with IMO Resolution A.851 Not applicable to SAR aircraft, should be set to 0. |
| Destination | 120 | Maximum 20 characters using 6-bit ASCII; @@@@@@@@@@@@@@@@@@@@ = not available For SAR aircraft, the use of this field may be decided by the responsible administration |
| DTE | 1 | Data terminal equipment (DTE) ready (0 = available, 1 = not available = default) (see § 3.3.1) |
| Spare | 1 | Spare. Not used. Should be set to zero. Reserved for future use. |
| Number of bits | 424 | Occupies 2 slots |

This message should be transmitted immediately after any parameter value has been changed.

3.3.1	The data terminal equipment (DTE) indicator

The purpose of the DTE indicator is to indicate to an application on the receiving side that, if set to available, the transmitting station conforms at least to the minimum keyboard and display requirements. On the transmitting side, the DTE indicator may also be set by an external application via the Presentation Interface. On the receiving side, the DTE indicator is only used as information provided to the application layer, that the transmitting station is available for communications.

### 3.3.2	Type of ship

TABLE 50

| Identifiers to be used by ships to report their type |  |
| --- | --- |
| Identifier No. | Special craft |
| 50 | Pilot vessel |
| 51 | Search and rescue vessels |
| 52 | Tugs |
| 53 | Port tenders |
| 54 | Vessels with anti-pollution facilities or equipment |
| 55 | Law enforcement vessels |
| 56 | Spare – for assignments to local vessels |
| 57 | Spare – for assignments to local vessels |
| 58 | Medical transports (as defined in the 1949 Geneva Conventions and Additional Protocols) |
| 59 | ships and aircraft of States not parties to an armed conflict Ships according to RR Resolution No. 18 (Mob-83) |

| Identifiers to be used by ships to report their type |  |  |  |
| --- | --- | --- | --- |
| Other ships |  |  |  |
| First digit(1) | Second digit(1) | First digit(1) | Second digit(1) |
| 1 – Reserved for future use | 0 – All ships of this type | – | 0 – Fishing |
| 2 – WIG | 1 – Carrying DG, HS, or MP, IMO hazard or pollutant category AX(2) | – | 1 – Towing |
| 3 – See right column | 2 – Carrying DG, HS, or MP, IMO hazard or pollutant category BY(2) | 3 – Vessel | 2 – Towing and length of the tow exceeds 200 m or breadth exceeds 25 m |
| 4 – HSC | 3 – Carrying DG, HS, or MP, IMO hazard or pollutant category CZ(2) | – | 3 – Engaged in dredging or underwater operations |
| 5 – See above | 4 – Carrying DG, HS, or MP, IMO hazard or pollutant category DOS(2) | – | 4 – Engaged in diving operations |
|  | 5 – Reserved for future use | – | 5 – Engaged in military operations |
| 6 – Passenger ships | 6 – Reserved for future use | – | 6 – Sailing |
| 7 – Cargo ships | 7 – Reserved for future use | – | 7 – Pleasure craft |
| 8 – Tanker(s) | 8 – Reserved for future use | – | 8 – Reserved for future use |
| 9 – Other types of ship | 9 – No additional information | – | 9 – Reserved for future use |

DG: Dangerous Goods.

HS: Harmful Substances.

MP: Marine Pollutants.

(1)  NOTE   –  The identifier should be constructed by selecting the appropriate first and second digits.

(2) NOTE  – The digits 1, 2, 3 and 4 reflecting  categories X, Y, Z and OS formerly were categories A, B, C and D.

### Rationale for Clarification:

This proposed change will: (1)  allow the use of this Message by SAR Aircraft, and, (2)  harmonise hazard or pollutant categories (i.e. A, B, C and D) used in the 2nd digit of Identifiers to be used by ships to report their type with new categories (i.e. X. Y. Z and OS) implemented under the 2004 amendments to MARPOL Annex II on 1 January 2007. 
(3) To maintain the context  “ships and aircraft of States not parties to an armed conflict” when Resolution 18 (Rev.WRC-07) is deleted

(4) Provide information about AIS mobile station internal GNSS receiver is in use for position reporting

(5) Provide specific information on the use of Message 5 by SAR aircraft

### Date of amendment: 17-09-2008, 13-03-2009 and 22-09-2009

### Comment

Most existing Class A stations do not process Message 24 (output on the PI and display on the MKD) and do not provide information about internal GNSS in use.  Message 24 can be transmitted by other AIS stations, i.e. Base Stations, SAR Aircraft, to provide additional static data.  Message 5 is processed by AIS stations, permitting its use by SAR Aircraft ensures that this important information is received by all AIS stations, legacy or new.

Future Class A stations, based on ITU=R M.1371-3, should be able to process message 24.

## A8; § 3.7  Message 9: Standard SAR aircraft position report

### Proposed Clarifying Text:

This message should be used as a standard position report for aircraft involved in SAR operations. Stations other than aircraft involved in SAR operations should not transmit this message. The default reporting interval for this message should be 10 s.

TABLE 56

| Parameter | Number of bits | Description |
| --- | --- | --- |
| Message ID | 6 | Identifier for Message 9; always 9 |
| Repeat indicator | 2 | Used by the repeater to indicate how many times a message has been repeated. See § 4.6.1, Annex 2; 0-3; 0 = default; 3 = do not repeat any more |
| User ID | 30 | MMSI number |
| Altitude (GNSS) | 12 | Altitude (derived from GNSS or barometric (see altitude sensor parameter below)) (m) (0-4 094 m) 4 095 = not available, 4 094 = 4 094 m or higher |
| SOG | 10 | Speed over ground in knot steps (0-1 022 knots) 1 023 = not available, 1 022 = 1 022 knots or higher |
| Position accuracy | 1 | 1 = high (< > 10 m) 0 = low (> < 10 m) 0 = default The PA flag should be determined in accordance with table. |
| Longitude | 28 | Longitude in 1/10 000 min (180º, East = positive (as per 2’s complement), West = negative (as per 2’s complement); 181= (6791AC0h) = not available = default) |
| Latitude | 27 | Latitude in 1/10 000 min (90º, North = positive (as per 2’s complement), South = negative (as per 2’s complement); 91= (3412140h) = not available = default) |
| COG | 12 | Course over ground in 1/10 = (0-3 599). 3 600 (E10h) = not available = default; 3 601-4 095 should not be used |
| Time stamp | 6 | UTC second when the report was generated by the EPFS (0-59 or 60 if time stamp is not available, which should also be the default value or 61 if positioning system is in manual input mode or 62 if electronic position fixing system operates in estimated (dead reckoning) mode or 63 if the positioning system is inoperative) |
| Altitude sensor | 1 | 0 = GNSS 1 = barometric source |
| Spare | 7 | Not used. Should be set to zero. Reserved for future use |
| DTE | 1 | Data terminal ready (0 = available 1 = not available = default) (see § 3.3.1) |
| Spare | 3 | Not used. Should be set to zero. Reserved for future use |
| Assigned mode flag | 1 | 0 = Station operating in autonomous and continuous mode = default 1 = Station operating in assigned mode |
| RAIM-flag | 1 | RAIM flag of electronic position fixing device; 0 = RAIM not in use = default; 1 = RAIM in use see Table 47 |
| Communication state selector flag | 1 | 0 = SOTDMA communication state follows 1 = ITDMA communication state follows |
| Communication state | 19 | SOTDMA communication state (see § 3.3.7.2.1, Annex 2), if communication state selector flag is set to 0, or ITDMA communication state (see § 3.3.7.3.2, Annex 2), if communication state selector flag is set to 1 |
| Number of bits | 168 |  |

### Rationale for Clarification:

Correct position accuracy flag definition

### Date of amendment: 13March 2008

### Comment

.No comment.

## A8; § 3.12 Message 14: Safety Related broadcast message

### Proposed Clarifying Text:

TABLE 60

| Parameter | Number of bits | Description |
| --- | --- | --- |
| Message ID | 6 | Identifier for Message 14; always 14. |
| Repeat indicator | 2 | Used by the repeater to indicate how many times a message has been repeated. See § 4.6.1, Annex 2; 0-3; 0 = default; 3 = do not repeat any more |
| Source ID | 30 | Unique identifier such as MMSI number of source station of message |
| Spare | 2 | Not used. Should be set to zero. Reserved for future use |
| Safety related text | Maximum 968 | 6-bit ASCII as defined in Table 44 |
| Maximum number of bits | Maximum 1 008 | Occupies 1 to 5 slots subject to the length of text. For Class B mobile AIS stations the length of the message should not exceed 2 slots |

Additional bit stuffing will be required for this message type. For details refer to transport layer, § 5.2.1, Annex 2.

Table 61 gives the number of 6-bit ASCII characters, so that the whole message fits into a given number of slots. It is recommended that any application minimizes the use of slots by limiting the number of characters to the numbers given, if possible:

TABLE 61

| Number of slots | Maximum 6-bit ASCII characters |
| --- | --- |
| 1 | 16 |
| 2 | 53 |
| 3 | 90 |
| 4 | 128 |
| 5 | 161 |

These numbers also take bit stuffing into account.

The AIS-SART should use Message 14, and the safety related text should be:

1.	For the active SART, the text should be “SART ACTIVE”.

2.	For the test mode, the text should be “SART TEST”.

### Rationale for Clarification:

AIS-SART does not use an MMSI, and so the description in the Source ID field needs to be changed to reflect this fact.

No definition of Safety Related Text included for AIS-SART

### Date of amendment: 22 September 2009

### Comment

.No comment.

## A8; § 3.16	Message 18: Standard Class B equipment position report

### Proposed Clarifying Text:

The Standard Class B equipment position report should be output periodically and autonomously instead of Messages 1, 2, or 3 by Class B shipborne mobile equipment, only. The reporting interval should default to the values given in table , Annex 1, unless otherwise specified by reception of a Message 16 or 23; and depending on the current SOG and navigational status flag setting.

TABLE 67

| Parameter | Number of bits | Description |
| --- | --- | --- |
| Message ID | 6 | Identifier for Message 18; always 18 |
| Repeat indicator | 2 | Used by the repeater to indicate how many times a message has been repeated. See § 4.6.1, Annex 2; 0-3; 0 = default; 3 = do not repeat any more; shall be 0 for “CS” transmissions |
| User ID | 30 | MMSI number |
| Spare | 8 | Not used. Should be set to zero. Reserved for future use |
| SOG | 10 | Speed over ground in 1/10 knot steps (0-102.2 knots) 1 023 = not available, 1 022 = 102.2 knots or higher |
| Position accuracy | 1 | 1 = high (< > 10 m) 0 = low (> < 10 m) 0 = default The PA flag should be determined in accordance with Table 47 |
| Longitude | 28 | Longitude in 1/10 000 min (180º, East = positive (as per 2’s complement), West = negative (as per 2’s complement); 181º (6791AC0h) = not available = default) |
| Latitude | 27 | Latitude in 1/10 000 min (90º, North = positive (as per 2’s complement), South = negative (as per 2’s complement); 91= (3412140h) = not available = default) |
| COG | 12 | Course over ground in 1/10= (0-3 599). 3 600 (E10h) = not available = default; 3 601-4 095 should not be used |
| True heading | 9 | Degrees (0-359) (511 indicates not available = default) |
| Time stamp | 6 | UTC second when the report was generated by the EPFS (0-59 or 60 if time stamp is not available, which should also be the default value or 61 if positioning system is in manual input mode or 62 if electronic position fixing system operates in estimated (dead reckoning) mode or 63 if the positioning system is inoperative) 61, 62, 63 are not used by “CS” AIS |

TABLE 67 (end)

| Parameter | Number of bits | Description |
| --- | --- | --- |
| Spare | 2 | Not used. Should be set to zero. Reserved for future use |
| Class B unit flag | 1 | 0 = Class B SOTDMA unit 1 = Class B “CS” unit |
| Class B display flag | 1 | 0 = No display available; not capable of displaying Message 12 and 14 1 = Equipped with integrated display displaying Message 12 and 14 |
| Class B DSC flag | 1 | 0 = Not equipped with DSC function 1 = Equipped with DSC function (dedicated or time-shared) |
| Class B band flag | 1 | 0 = Capable of operating over the upper 525 kHz band of the marine band 1 = Capable of operating over the whole marine band (irrelevant if “Class B Message 22 flag” is 0) |
| Class B Message 22 flag | 1 | 0 = No frequency management via Message 22 , operating on AIS1, AIS2 only 1 = Frequency management via Message 22 |
| Mode flag | 1 | 0 = Station operating in autonomous and continuous mode = default 1 = Station operating in assigned mode |
| RAIM-flag | 1 | RAIM (Receiver autonomous integrity monitoring) flag of electronic position fixing device; 0 = RAIM not in use = default; 1 = RAIM in use see Table 47 |
| Communication state selector flag | 1 | 0 = SOTDMA communication state follows 1 = ITDMA communication state follows (always “1” for Class-B “CS”) |
| Communication state | 19 | SOTDMA communication state (see § 3.3.7.2.1, Annex 2), if communication state selector flag is set to 0, or ITDMA communication state (see § 3.3.7.3.2, Annex 2), if communication state selector flag is set to 1 Because Class B “CS” does not use any Communication State information, this field shall be filled with the following value: 1100000000000000110. |
| Number of bits | 168 | Occupies one slot |

### Rationale for Clarification:

Correct position accuracy flag definition

### Date of amendment: 13March 2008

### Comment

.No comment.

## A8; §3.17 Message 19: Extended Class B equipment position report

### Proposed Clarifying Text:

This message should be used by Class B shipborne mobile equipment. This message should be transmitted once every 6 min in two slots allocated by the use of Message 18 in the ITDMA communication state. This message should be transmitted immediately after the following parameter values change: dimension of ship/reference for position or type of electronic position fixing device.

TABLE 68

| Parameter | Number of bits | Description |
| --- | --- | --- |
| Message ID | 6 | Identifier for Message 19; always 19 |
| Repeat indicator | 2 | Used by the repeater to indicate how many times a message has been repeated. See § 4.6.1, Annex 2; 0-3; 0 = default; 3 = do not repeat any more |
| User ID | 30 | MMSI number |
| Spare | 8 | Not used. Should be set to zero. Reserved for future use |
| SOG | 10 | Speed over ground in 1/10 knot steps (0-102.2 knots) 1 023 = not available, 1 022 = 102.2 knots or higher |
| Position accuracy | 1 | 1 = high (< > 10 m) 0 = low (> < 10 m) 0 = default The PA flag should be determined in accordance with table |
| Longitude | 28 | Longitude in 1/10 000 min (180º, East = positive (as per 2’s complement), West = negative (as per 2’s complement); 181° (6791AC0h) = not available = default) |
| Latitude | 27 | Latitude in 1/10 000 min (90°, North = positive (as per 2’s complement), South = negative (as per 2’s complement); 91= (3412140h) = not available = default) |
| COG | 12 | Course over ground in 1/10= (0-3 599). 3 600 (E10h) = not available = default; 3 601-4 095 should not be used |
| True heading | 9 | Degrees (0-359) (511 indicates not available = default) |
| Time stamp | 6 | UTC second when the report was generated by the EPFS (0-59 or 60) if time stamp is not available, which should also be the default value or 61 if positioning system is in manual input mode or 62 if electronic position fixing system operates in estimated (dead reckoning) mode, or 63 if the positioning system is inoperative) |
| Spare | 4 | Not used. Should be set to zero. Reserved for future use |
| Name | 120 | Maximum 20 characters 6-bit ASCII, as defined in Table 44. @@@@@@@@@@@@@@@@@@@@ = not available = default |
| Type of ship and cargo type | 8 | 0 = not available or no ship = default 1-99 = as defined in § 3.3.2 100-199 = reserved, for regional use 200-255 = reserved, for future use |
| Dimension of ship/reference for position | 30 | Dimensions of ship in metres and reference point for reported position (see Fig. 41 and § 3.3.3) |
| Type of electronic position fixing device | 4 | 0 =Undefined (default); 1 = GPS, 2 = GLONASS, 3 = combined GPS/GLONASS, 4 = Loran-C, 5 = Chayka, 6 = integrated navigation system, 7 = surveyed; 8 = Galileo, 9-1415 = not used; 15 = internal GNSS |
| RAIM-flag | 1 | RAIM (Receiver autonomous integrity monitoring) flag of electronic position fixing device; 0 = RAIM not in use = default; 1 = RAIM in use see Table 47 |
| DTE | 1 | Data terminal ready (0 = available 1 = not available; = default) (see § 3.3.1) |
| Assigned mode flag | 1 | 0 = Station operating in autonomous and continuous mode = default 1 = Station operating in assigned mode |
| Spare | 4 | Not used. Should be set to zero. Reserved for future use |
| Number of bits | 312 | Occupies two slots. |

### Rationale for Clarification:

a) Correct position accuracy flag definition

b) Provide information about AIS mobile station internal GNSS receiver is in use for position reporting

### Date of amendment: 13March 2008

### Comment

No comment.

## A 8; § 3.18	Message 20: Data link management message

### Proposed Clarifying Text:

This message should be used by base station(s) to pre-announce the fixed allocation schedule (FATDMA) for one or more base station(s) and it should be repeated as often as required. This way the system can provide a high level of integrity for base station(s). This is especially important in regions where several base stations are located adjacent to each other and mobile station(s) move between these different regions. These reserved slots cannot be autonomously allocated by mobile stations.

The mobile station, within 120 nautical miles *, should then reserve the slots for transmission by the base station(s) until time-out occurs. The base station should refresh the time-out value with each transmission of Message 20 in order to allow mobile stations to terminate their reservation for the use of the slots by the base stations (refer to § 3.3.1.2, Annex 2).

The parameters: offset number, number of slots, time-out, and increment should be treated as a unit, meaning that if one parameter is defined all other parameters should be defined within that unit. The parameter offset number should denote the offset from the slot in which Message 20 was received to the first slot to be reserved. The parameter number of slots should denote the number of consecutive slots to be reserved starting with the first reserved slot. This defines a reservation block. This reservation block should not exceed 5 slots. The parameter increment should denote the number of slots between the starting slot of each reservation block. An increment of zero indicates one reservation block per frame. The values recommended for increment are as follows: 2, 3, 5, 6, 9, 10, 15, 18, 25, 30, 45, 50, 75, 90, 125, 150, 225, 250, 375, 450, 750, or 1125. Use of one of these values guarantees symmetric slot reservations throughout each frame. This message applies only to the frequency channel in which it is transmitted.

If interrogated and no data link management information available, only offset number 1, number of slots 1, time-out 1, and increment 1 should be sent. These fields should all be set to zero.

*note: A Base Station Report (message 4) in conjunction with a Data Link Management Message (message 20) with the same base station ID (MMSI) must be received by the mobile station so that it can determine its distance from the transmitting base station.

### Rationale for Clarification:

Provide clarification for AIS station behaviour regarding the 120 nm rule

### Date of amendment: 16 January 2009

### Comment

## A8; § 3.19	Message 21: Aids-to-navigation report (AtoN)

### Proposed Clarifying Text:

This message should be used by an AtoN AIS station. This station may be mounted on an aid-to-navigation or this message may be transmitted by a fixed station when the functionality of an AtoN station is integrated into the fixed station. This message should be transmitted autonomously at a Rr of once every three (3) min or it may be assigned by an assigned mode command (Message 16) via the VHF data link, or by an external command. This message should not occupy more than two slots.

TABLE 70

| Parameter | Number of bits | Description |
| --- | --- | --- |
| Message ID | 6 | Identifier for Message 21 |
| Repeat indicator | 2 | Used by the repeater to indicate how many times a message has been repeated. See § 4.6.1, Annex 2; 0-3; 0 = default; 3 = do not repeat any more |
| ID | 30 | MMSI number, (see Article 19 of the RR and Recommendation ITU-R M.585) |
| Type of aids-to-navigation | 5 | 0 = not available = default; refer to appropriate definition set up by IALA; see Table 71 |
| Name of Aids-to-Navigation | 120 | Maximum 20 characters 6-bit ASCII, as defined in Table 44 “@@@@@@@@@@@@@@@@@@@@” = not available = default. The name of the AtoN may be extended by the parameter “Name of Aid-to-Navigation Extension” below |
| Position accuracy | 1 | 1 = high (< > 10 m) 0 = low (> < 10 m) 0 = default. The PA flag should be determined in accordance with Table 47 |
| Longitude | 28 | Longitude in 1/10 000 min of position of an AtoN (180º, East = positive, West = negative 181 = (6791AC0h) = not available = default) |
| Latitude | 27 | Latitude in 1/10 000 min of an AtoN (90º, North = positive, South = negative 91 = (3412140h) = not available = default) |
| Dimension/ reference for position | 30 | Reference point for reported position; also indicates the dimension of an AtoN (m) (see Fig. 42 and § 4.1), if relevant(1) |
| Type of electronic position fixing device | 4 | 0 = Undefined (default) 1 = GPS 2 = GLONASS 3 = Combined GPS/GLONASS 4 = Loran-C 5 = Chayka 6 = Integrated Navigation System 7 = surveyed. For fixed AtoN and virtual AtoN, the charted position should be used. The accurate position enhances its function as a radar reference target 8 = Galileo 9-1415 = not used 15 = internal GNSS |

TABLE 70 (end)

| Parameter | Number of bits | Description |
| --- | --- | --- |
| Time stamp | 6 | UTC second when the report was generated by the EPFS (0-59 or 60) if time stamp is not available, which should also be the default value or 61 if positioning system is in manual input mode or 62 if electronic position fixing system operates in estimated (dead reckoning) mode or 63 if the positioning system is inoperative) |
| Off-position indicator | 1 | For floating AtoN, only: 0 = on position; 1 = off position. NOTE 1 – This flag should only be considered valid by receiving station, if the AtoN is a floating aid, and if time stamp is equal to or below 59. For floating AtoN the guard zone parameters should be set on installation |
| AtoN status | 8 | Reserved for the indication of the AtoN status 00000000 = default |
| RAIM-flag | 1 | RAIM (Receiver autonomous integrity monitoring) flag of electronic position fixing device; 0 = RAIM not in use = default; 1 = RAIM in use see Table 47 |
| Virtual AtoN flag | 1 | 0 = default = real AtoN at indicated position; 1 = virtual AtoN, does not physically exist(2). |
| Assigned mode flag | 1 | 0 = Station operating in autonomous and continuous mode = default 1 = Station operating in assigned mode |
| Spare | 1 | Spare. Not used. Should be set to zero. Reserved for future use |
| Name of Aid-to-Navigation Extension | 0, 6, 12, 18, 24, 30, 36, ... 84 | This parameter of up to 14 additional 6-bit-ASCII characters for a 2-slot message may be combined with the parameter “Name of Aid-to-Navigation” at the end of that parameter, when more than 20 characters are needed for the name of the AtoN. This parameter should be omitted when no more than 20 characters for the name of the A-to-N are needed in total. Only the required number of characters should be transmitted, i.e. no @-character should be used |
| Spare | 0, 2, 4, or 6 | Spare. Used only when parameter “Name of Aid-to-Navigation Extension” is used. Should be set to zero. The number of spare bits should be adjusted in order to observe byte boundaries |
| Number of bits | 272-360 | Occupies two slots |
| Notes to Table 70: (1) When using Fig. 41 for AtoN the following should be observed: – For fixed Aids-to-Navigation, virtual AtoN, and for off-shore structures, the orientation established by the dimension A should point to true north. – For floating aids larger than 2 m * 2 m the dimensions of the AtoN should always be given approximated to a circle, i.e. the dimensions should always be as follows A=B=C=D≠0. (This is due to the fact that the orientation of the floating Aid to Navigation is not transmitted. The reference point for reported position is in the centre of the circle.) – A=B=C=D=1 should indicate objects (fixed or floating) smaller than or equal to 2 m * 2 m. (The reference point for reported position is in the centre of the circle.) |  |  |
| Notes to Table 70 (cont.): – Floating off shore structures that are not fixed, such as rigs, should be considered as Code 31 type from Table 71 AtoN. These structures shall have their “Dimension/reference for position” parameter as determined above in Note (1). For fixed off shore structures, Code 3 type from Table 71, shall have their “Dimension/reference for position” parameter as determined above in Note (1). Hence, all off shore AtoN and structures have the dimension determined in the same manner and the actual dimensions are contained in Message 21. (2) When transmitting virtual AtoN information, i.e. the virtual/pseudo AtoN Target Flag is set to one (1), the dimensions should be set to A=B=C=D=0 (default). This should also be the case, when transmitting “reference point” information (see Table 70). |  |  |

This message should be transmitted immediately after any parameter value was changed.

### Rationale for Clarification:

a) Correct position accuracy flag definition

b) Provide information about AIS mobile station internal GNSS receiver is in use for position reporting

### Date of amendment: 13March 2008

### Comment

.No comment.

## A8; §3.21 Message 23: Group Assignment Command

### Proposed Clarifying Text:

The Group Assignment Command is transmitted by a Base station when operating as a controlling entity (see § 4.3.3.3.2 Annex 7 and §3.20). This message should be applied to a mobile station within the defined region and as selected either by its position, by its “Ship and Cargo Type” or by its “Station Type”. The receiving station should consider all selector fields concurrently.  It controls the following operating parameters of a mobile station;

- transmit/ receive mode;

- reporting interval; and

- the duration of a quiet time.

Table 73

| Parameter | Number of bits | Description |
| --- | --- | --- |
| Message ID | 6 | Identifier for Message 23; always 23 |
| Repeat indicator | 2 | Used by the repeater to indicate how many times a message has been repeated. 0-3; default = 0; 3 = do not repeat any more |
| Source ID | 30 | MMSI of assigning station |
| Spare | 2 | Spare. Shall be set to zero |
| Longitude 1 | 18 | Longitude of area to which the group assignment applies; upper right corner (north-east); in 1/10 min (180, East = positive, West = negative) |
| Latitude 1 | 17 | Latitude of area to which the group assignment applies; upper right corner (north-east); in 1/10 min (90, North = positive, South = negative) |
| Longitude 2 | 18 | Longitude of area to which the group assignment applies; lower left corner (south-west); in 1/10 min (180, East = positive, West = negative) |
| Latitude 2 | 17 | Latitude of area to which the group assignment applies; lower left corner (south-west); in 1/10 min (90, North = positive, South = negative) |
| Station type | 4 | 0 = all types of mobiles (default); 1 = Class A mobile station only reserved for future use; 2 = all types of Class B mobile stations; 3 = SAR airborne mobile station; 4 = Class B “SO” mobile stations only AtoN station; 5 = Class B “CS” shipborne mobile station only; 6 = inland waterways; 7 to 9 = regional use and; 10 to 15 = for future use |
| Type of ship and cargo type | 8 | 0 = all types (default) 1...99 see Table 50 100...199 reserved for regional use 200...255 reserved for future use |
| Spare | 22 | Not used. Should be set to zero. Reserved for future use. |
| Tx/Rx mode | 2 | This parameter commands the respective stations to one of the following modes: 0 = TxA/TxB, RxA/RxB (default); 1 = TxA, RxA/RxB , 2 = TxB, RxA/RxB, 3 = reserved for future use |
| Reporting interval | 4 | This parameter commands the respective stations to the reporting interval given in Table 74 |
| Quiet time | 4 | 0 = default = no quiet time commanded; 1-15 = quiet time of 1 to 15 min. |
| Spare | 6 | Not used. Should be set to zero. Reserved for future use. |
| Number of bits | 160 | Occupies one-time period |

Table 74

Reporting interval settings for use with Message 23

| Reporting interval field setting | Reporting interval for Message 23 |
| --- | --- |
| 0 | As given by the autonomous mode |
| 1 | 10 minutes |
| 2 | 6 minutes |
| 3 | 3 minutes |
| 4 | 1 minute |
| 5 | 30 seconds |
| 6 | 15 seconds |
| 7 | 10 seconds |
| 8 | 5 seconds |
| 9 | 2 seconds (not applicable to the Class B “CS”) |
| 9 | Next shorter reporting interval |
| 11 10 | Next longer reporting interval |
| 11 | 2 seconds (not applicable to the Class B “CS”) |
| 12-15 | Reserved for future use |

NOTE – When the dual channel transmission is suspended by Tx/Rx mode command 1 or 2, the required reporting interval should be maintained using the remaining transmission channel.

### Rationale for Clarification:

Clarify the concurrent evaluation of parameters position, ship and cargo type and station type:

This proposed change harmonises the Message 23 reporting interval settings with the Class-B “CS” testing standard (IEC 62287-1 Ed.1).

### Date of amendment: 21 September 2007

### Comment

.No comment.

## A8; § 3.22 Message 24: Static data report

### Proposed Clarifying Text:

Message 24 Part A and Part B may be used by any AIS station to associate a MMSI with the static information.

Message 24 Part A and Part B shall be used by Class B “CS” shipborne mobile equipment. The message consists of two parts. Message 24B shall be transmitted within 1 min following Message 24A.

In case of an interrogation for a Class B “CS” on a Message 24, the response shall include Part A and Part B.

Table 75

Message 24 Part A

| Parameter | Number of bits | Description |
| --- | --- | --- |
| Message ID | 6 | Identifier for Message 24; always 24 |
| Repeat indicator | 2 | Used by the repeater to indicate how many times a message has been repeated. 0 = default; 3 = do not repeat any more |
| User ID | 30 | MMSI number |
| Part number | 2 | Identifier for the message part number; always 0 for Part A |
| Name | 120 | Name of the MMSI-registered vessel. Maximum 20 characters 6-bit ASCII, @@@@@@@@@@@@@@@@@@@@ = not available = default. For SAR aircraft, it should be set to “SAR AIRCRAFT NNNNNNN” where NNNNNNN equals the aircraft registration number. |
| Number of bits | 160 | Occupies one-time period |

Table 76

Message 24 Part B

| Parameter | Number of bits | Description |
| --- | --- | --- |
| Message ID | 6 | Identifier for Message 24; always 24 |
| Repeat indicator | 2 | Used by the repeater to indicate how many times a message has been repeated. 0 = default; 3 = do not repeat any more |
| User ID | 30 | MMSI number |
| Part number | 2 | Identifier for the message part number; always 1 for Part B |
| Type of ship and cargo type | 8 | 0 = not available or no ship = default 1-99 = as defined in § 3.3.2 100-199 = reserved, for regional use 200-255 = reserved, for future use Not applicable to SAR aircraft |
| Vendor ID | 42 | Unique identification of the Unit by a number as defined by the manufacturer (option; “@@@@@@@“ = not available = default). See Table 76A below |
| Call sign | 42 | Call sign of the MMSI-registered vessel. 7 X 6 bit ASCII characters, “@@@@@@@“ = not available = default |
| Dimension of ship/reference for position. Or, for unregistered daughter vessels, use the MMSI of the mother ship. | 30 | Dimensions of ship in meters and reference point for reported position (see Figure 41 and §3.3.3). Or, for an unregistered daughter vessel, use the MMSI of the associated mother ship in this data field For SAR Aircraft, the use of this field may be decided by the responsible administration. If used it should indicate the maximum dimensions of the craft. As de-fault should A=B=C=D be set to “0” |
| Spare | 6 |  |
| Number of bits | 168 | Occupies one-time period |

Table 76A – Vendor ID field

| Bit | Information | Description |
| --- | --- | --- |
| (MSB) 41 …...... 24 (18bits) | Manufacturer’s ID | The Manufacturer's ID bits indicate the manufacture’s mnemonic code consisting of three 6 bit ASCII characters. 1) |
| 23 …...... 20 (4bits) | Unit Model Code | The Unit Model Code bits indicate the binary coded series number of the model. The first model of the manufacture uses “1” and the number is incremented at the release of a new model. The code reverts to “1” after reaching to “15”. The “0” is not used. |
| 19 …...... 0 (LSB) (20bits) | Unit Serial Number | The Unit Serial Number bits indicate the manufacture traceable serial number. When the serial number is composed of numeric only, the binary coding should be used. If it includes figure(s), the manufacture can define the coding method. The coding method shall be mentioned in the manual. |

1) IALA is presently considering to create an open international register of manufacturers ID’s

### Rationale for Clarification:

In the ITU-R M.1371-3, the vendor ID field (42bits) is prepared in the Message 24B for the Class-B AIS station, however the detail is not defined.

The Table 76A proposes the clarifications needed for the additional details.

Part B is needed to allow AIS stations to transmit static data.

Provide specific information on the use of Message 24A/B by SAR aircraft

### Date of amendment: 17 September 2008

### Comment

No comment.

## A8; § 3.25 Message 27: Long Range AIS Broadcast Message

### Proposed Clarifying Text:

This message is primarily intended for long range detection of AIS Class A equipped vessels (typically by satellite). This message has a similar content to Messages 1,2 and 3, but the total number of bits has been compressed to allow for increased propagation delays associated with long range detection. Refer to Annex 4 for details on Long Range applications

TABLE 81

| Parameter | Number of bits | Description |
| --- | --- | --- |
| Message ID | 6 | Identifier for this message ; always 27 |
| Repeat indicator | 2 | Always 3 |
| User ID | 30 | MMSI number |
| Position accuracy | 1 | As defined for Message 1 |
| RAIM flag | 1 | As defined for Message 1 |
| Navigational status | 4 | As defined for Message 1 |
| Longitude | 18 | Longitude in 1/10 min (180º, East = positive, West = negative |
| Latitude | 17 | Latitude in 1/10 min (90º, North = positive, South = negative |
| SOG | 6 | Knots (0-62); 63 = not available = default |
| COG | 9 | Degrees (0-359); 511 = not available = default |
| Status of current GNSS position | 1 | 0 = Position is the current GNSS position; 1 = Reported position is not the current GNSS position = default |
| Spare | 1 | Set to zero, to preserve byte boundaries |
| Total number of bits | 96 |  |

NOTE - There is no time stamp in this message. The receiving system is expected to provide the time stamp when this message is received.

### Rationale for Clarification:

Definition of Message 27.

### Date of amendment: 22 September 2009

### Comment

No comment.

## A 9; Requirements for stations using burst transmissions

### Proposed Clarifying Text:

A9; §1 Requirements for stations using burst transmissions

This annex specifies how data shall be formatted and transmitted for units that have limited range and operate in a low volume VDL.  Burst transmission behaviour will increase the probability of reception and is required for units such as an AIS SART.

Burst behaviour conforms with Annex 2 in ITU-R M.1371 with minor modifications in the following sections:

- Transceiver characteristics

- Transmitter transient response

- Synchronisation accuracy

- Channel access scheme

- User ID (Unique identifier)

A9; §2 Transceiver characteristics

Annex 9 Table 1 - Required parameter settings

| Symbol | Parameter name | Setting |
| --- | --- | --- |
| PH.AIS1 | Channel 1 (default channel 1) | 161.975 MHz |
| PH.AIS2 | Channel 2 (default channel 2) | 162.025 MHz |
| PH.BR | Bit rate | 9 600 bps |
| PH.TS | Training sequence | 24 bits |
| PH.TST | Transmitter settling time (Transmit power within 20% of final value. Frequency stable to within  1.0 kHz of final value). Tested at manufacturers declared transmit power. |  1.0 ms |
|  | Ramp down time |  832 µs |
|  | Transmission duration | ≤ 26,6 ms |
|  | Transmitter output power | Nominal 1W EIRP |

In addition, the constants of the physical layer AIS station shall comply with the values given in Annex 9 Annex 9 Table 2 and Annex 9 Annex 9 Table 3.

Annex 9 Table 2 - Required settings of physical layer constants

| Symbol | Parameter name | Value |
| --- | --- | --- |
| PH.DE | Data encoding | NRZI |
| PH.FEC | Forward error correction | Not used |
| PH.IL | Interleaving | Not used |
| PH.BS | Bit scrambling | Not used |
| PH.MOD | Modulation | Bandwidth adapted GMSK |

Annex 9 Table 3 - Modulation parameters of the physical layer

| Symbol | Name | Value |
| --- | --- | --- |
| PH.TXBT | Transmit BT-product | 0.4 |
| PH.MI | Modulation index | 0.5 |

#### A9; §3 Transmitter requirements

The technical characteristics as specified in Annex 9 Annex 9 Table 4 should apply to the transmitter.

Annex 9 Table 4 - Minimum required transmitter characteristics

| Transmitter parameters | Requirements |
| --- | --- |
| Carrier power | nominal radiated power 1W |
| Carrier frequency error |  500 Hz (normal). + 1000 Hz (extreme) |
| Slotted modulation mask | –20 dBc Δfc> ±10 kHz –40 dBc ±25 kHz<Δfc< ±62.5 kHz see Annex 9 Annex 9 Figure |
| Transmitter test sequence and modulation accuracy | <3400 Hz for Bit 0, 1 (normal and extreme) 2400 Hz ± 480 Hz for Bit 2, 3 (normal and extreme) 2400 Hz ± 240 Hz for Bit 4 ... 31 (normal, 2400 + 480 Hz extreme) For Bits Bit 32 …199 1740 ± 175 Hz (normal, 1740 + 350 Hz extreme) for a bit pattern of 0101 2400 Hz ± 240 Hz (normal, 2400 + 350 Hz extreme) for a bit pattern of 00001111 |
| Transmitter output power versus time | Power within mask shown in Annex 2 Figure 2 and timings given in Annex 2 Table 6 |
| Spurious emissions | maximum 25 μW 108 MHz to 137 MHz, 156 MHz to 161.5 MHz, and 1 525 MHz to 1 610 MHz |

For information the emission mask specified above is shown in Annex 9 Figure 1.

Annex 9 Figure 1 – Emission mask

#### A9; §4 Synchronisation accuracy

During UTC direct synchronisation, the transmission timing error, including jitter, of the AIS station shall be ± 3bits (± 312 s).

A9; §5 Channel access scheme

The AIS station shall operate autonomously and determine its own schedule for transmission of its messages based on random selection of the first slot of the first burst. The other 7 slots within the first burst shall be fixed referenced to the first slot of the burst. The increment between transmissions slots within a burst shall be 75 slots and the transmissions shall alternate between AIS1 and AIS2. The AIS station  transmits messages in a burst of 8 messages no more than once per minute.

In active mode the AIS station shall use messages with a communication state in the first burst. The communication state shall set a slot-time-out=7 in the first burst, thereafter the slot-time-out shall be decreased according to the rules of SOTDMA. All slots shall be regarded as candidates in the selection process. When time out occurs, the offset to the next set of 8 bursts is randomly selected between 1 minute ± 6 seconds.

After the first burst any messages can be used in subsequent transmissions but should be on the slots reserved by the first burst.

In test mode, messages with a communication state shall set slot-time-out=0 and sub-message=0 in the first and only burst.

The slot-time-out values of all messages´ communication state within every burst shall be the same.

Messages shall be transmitted alternately on AIS 1 and AIS 2.

Ch 1

Ch 2

Time

To

To + 1min

To + 7min

To + (8min +/- 6s)

8 Frames

Figure 1 – Burst transmissions in active mode

A9; § 6 User ID (Unique identifier)

The user ID shall have a unique pattern such as the  AIS-SART where the user ID is 970xxyyyy (where xx = manufacturer ID 01 to 99; xx = 00 is reserved for test purposes; yyyy = the sequence number 0000 to 9999).

### Rationale for Clarification:

Define Requirements for stations using burst transmissions such as AIS-SART

### Date of amendment: 13 March 2009

### Comment

## A 10 (previous A 9); Abbreviations in Recommendation ITU-R M. 1371-3

### Proposed Clarifying Text:

AIS-SART	AIS Search and Rescue Transmitter

### Rationale for Clarification:

Define AIS SART station

### Date of amendment: 13 March 2009

### Comment

# Appendix A ; AIS message for static information from SAR aircrafts [This text left in document for completeness – relevant changes from 1371-3 already flagged above]

Background.

The e-Nav Committee is requested to give guidance on the transmission of static information for SAR aircrafts. Presently ITU-R M. 1371-3 don’t specify any message for transmission of static information for SAR aircrafts. There are however two possible messages for this purpose:

- Msg 5  designed for use by Class A shipborne AIS

- Msg 24A and 24B,  designed as a general message for static information for any AIS station

Discussion

The two messages have very similar information content, not tailor made for SAR aircrafts. but they can include relevant information.  They also include data fields which not are applicable for SAR aircrafts.

Msg 5 is forwarded to the presentation interface on all existing shipborne AIS stations. It is not clear if Msg 5 for a SAR aircraft is associated with Msg 9 (SAR aircraft position report) in a MKD. It is presented correctly at least by some presentation systems, while others may need a SW upgrade.

The transmission of Msg 5 from SAR aircrafts requires an amendment of ITU-R M.1371-3 to allow the use for SAR Aircraft.

Msg 24 is “unknown” to most existing Class A shipborne AIS stations and will not be forwarded to the presentation interface. A SW upgrade (if possible) of these AIS units is required together with a SW upgrade of their display systems. This may strongly reduce the value of the transmissions since there is no requirement for SW upgrades of AIS units.

Msg 24 may be transmitted by SAR aircrafts without any changes of existing standards

Proposal

It is proposed that both messages are allowed for use by SAR aircrafts and they may be used in parallel to achieve the highest probability for a correct presentation, now and in the future. The IALA Clarifications of ITU-R M.1371 should include an amendment of the text for Msg 5 to allow the use by SAR aircrafts.

It is not proposed to alter the interrogation response from Msg 15 for SAR aircrafts due to its limited use.

The reporting rate may be decided by the responsible administration.

The data fields in Msg 5 is proposed to be used as follows:

| Parameter | Number of bits | Description |
| --- | --- | --- |
| Message ID | 6 | Identifier for this message 5 |
| Repeat indicator | 2 | Used by the repeater to indicate how many times a message has been repeated. |
| User ID | 30 | MMSI number |
| AIS version indicator | 2 | 0  station compliant with Recommendation ITU-R M.1371-1 1 = station compliant with Recommendation ITU-R M.1371-3 2-3  station compliant with future editions |
| IMO number | 30 | Not applicable for SAR aircraft. Set to 0 or possible new code |
| Call sign | 42 | 7  6 bit ASCII characters, @@@@@@@  not available  default |
| Name | 120 | Maximum 20 characters 6 bit ASCII Should be set to “SAR AIRCRAFT NNNNNNN” where NNNNNNN equals the aircraft registration number. “@@@@@@@@@@@@@@@@@@@@”  not available  default |
| Type of ship and cargo type | 8 | Should be set to “0 “ not available or no ship  default |
| Overall Dimension/ reference for position | 30 | The use of this field may be decided by the responsible administration. If used it should indicate the maximum dimensions of the craft. As default should A=B=C=D be set to “0” Reference point for reported position. Also indicates the dimension of ship (m) |
| Type of electronic position fixing device | 4 | 0  undefined (default) 1  GPS 2  GLONASS 3  combined GPS/GLONASS 4  Loran-C 5  Chayka 6  integrated navigation system 7  surveyed 8 = Galileo, 9-14 15  not used 15 = internal GNSS |
| ETA | 20 | The use of this field may be decided by the responsible administration Estimated time of arrival; MMDDHHMM UTC Bits 19-16: month; 1-12; 0  not available  default Bits 15-11: day; 1-31; 0  not available  default Bits 10-6: hour; 0-23; 24  not available  default Bits 5-0: minute; 0-59; 60  not available  default |
| Maximum present static draught | 8 | Not applicable for SAR aircraft. Should be set to 0  not available  default; |
| Destination | 120 | The use of this field may be decided by the responsible administration Maximum 20 characters using 6-bit ASCII; @@@@@@@@@@@@@@@@@@@@  not available |
| DTE | 1 | Data terminal ready (0  available, 1  not available  default) |
| Spare | 1 | Spare. Not used. Should be set to zero. Reserved for future use. |
| Number of bits | 424 | Occupies 2 slots |

The data fields of Msg 24A and 24B should be used in a corresponding way.

Highlighted text has been used to indicate how the messages may be used for SAR

aircraft.

# Editorial corrections

The editorially corrected version of M.1371-3 was published in

March 2008, however the following editorial corrections should be incorporated into the next publication of ITU-R M. 1371:

(1) Page 100

In the position accuracy of Table 45, referring table number is

missing.  It should be corrected as "Table 47".

(2) Page 101

In the RAIM-flag of Table 45, referring table number is

missing.  It should be corrected as "Table 47".

(3) Page 110

In the position accuracy of Table 56, referring table number is

missing.  It should be corrected as "Table 47".

(4) Page 114

In the 5th paragraph of 3.14, referring table number is

missing.  It should be corrected as "Table 16, Annex 2".

(5) Page 117

In the 3rd line of 3.16, referring table number is

missing.  It should be corrected as "Table 2, Annex 1".

(6) Page 119

In the position accuracy of Table 68, referring table number is

missing.  It should be corrected as "Table 47".

(7) Page 128

In the type of ship and cargo type of Table 73, referring table

number is missing.  It should be corrected as "Table 50".