Liaison Note to
the IMO Correspondence Group on e-Navigation

Current IMO SOLAS navigation and communication
equipment list that may need to be modified for e-Navigation

# Introduction

The following tables at Annex shows the communications and navigation equipment currently required by SOLAS chapters III, IV and V.  These equipments are mandatory depending on the configuration of the ship (tonnage, etc.) and GMDSS Sea Area (A1, A2, A3 or A4). The INS has been added, although it is not currently a carriage requirement as it might be an essential element of e-navigation.

The tables have been prepared using the tables in the European Marine Equipment Directive, and may not be complete.

The 5th Column has been added to indicate whether IALA believes that the equipment can be used for e-Navigation.

E = can be used without modification to the existing standards

F = future upgrade needed for interfacing

M = standards need to be modified for e-NAV

X = not part of e-Navigation (exempt)

P = presentation rules may apply

(This work will be reviewed when further work on the configuration of shipborne equipment for e-Navigation is complete)

Notes:

- Some equipment standards are subject to clarification from circulars from MSC, NAV and COMSAR.

- Until the scoping exercise is complete at IMO, the communications devices in the table below are based on the current GMDSS.

- Where the equipment is radio-based, the appropriate ITU recommendations are cited.

# Action Requested

The IMO Correspondence Group on e-Navigation is requested to note this information when deciding on which current SOLAS equipment is suitable for e-Navigation.

Annex

Table 1 - Communications equipment from SOLAS CH. IV 
(including those required by SOLAS CH III lifesaving appliances)

E = can be used without modification to the existing standards

F = future upgrade needed for interfacing

M = standards need to be modified for e-NAV

X = not part of e-Navigation (exempt)

P = presentation rules may apply

| Item designation | SOLAS 74 where “type approval” is required | Regulations of SOLAS 74 and the relevant resolutions and circulars of the IMO, as applicable | Testing standards | e-NAV |
| --- | --- | --- | --- | --- |
| VHF radio capable of transmitting and receiving DSC and radiotelephony | Reg. IV/14, — Reg. X/3, — IMO Res. MSC.36(63)-(1994 HSC code) 14, — IMO Res. MSC.97(73)-(2000 HSC code) 14. | Reg. IV/7, — Reg. X/3, — IMO Res. A.385(X), — IMO Res. A.524(13), — IMO Res. A.694(17), — IMO Res. A.803(19), — IMO Res. MSC.36(63)-(1994 HSC code) 14, — IMO Res. MSC.97(73)-(2000 HSC code) 14, — IMO MSC/Circ.862, — IMO Comsar Circ.32, — ITU-R M.489-2 (10/95), — ITU-R M.493-12 (03/07), — ITU-R M.541-8 (10/97), — ITU-R M.689-2 (11/93). | IEC 61097-3 (1994), IEC 61097-7 (1996), IEC 61162 series, — IMO MSC/Circ.862. | E P |
| VHF DSC watchkeeping receiver | Reg. IV/14, — Reg. X/3, — IMO Res. MSC.36(63)-(1994 HSC code) 14, — IMO Res. MSC.97(73)-(2000 HSC code) 14. | Reg. IV/7, — Reg. X/3, — IMO Res. A.694(17), — IMO Res. A.803(19), — IMO Res. MSC.36(63)-(1994 HSC code) 14, — IMO Res. MSC.97(73)-(2000 HSC code) 14, — IMO Comsar Circ.32, — ITU-R M.489-2 (10/95), — ITU-R M.493-12 (03/07), — ITU-R M.541-8 (10/97). | IEC 60945 (2002), — IEC 61097-3 (1994), — IEC 61097-8 (1998). | E |
| Navtex receiver | Reg. IV/14, — Reg. X/3, — IMO Res. MSC.36(63)-(1994 HSC code) 14, — IMO Res. MSC.97(73)-(2000 HSC code) 14. | Reg. IV/7, — Reg. X/3, — IMO Res. A.694(17), — IMO Res. MSC.36(63)-(1994 HSC code) 14, — IMO Res. MSC.97(73)-(2000 HSC code) 14, — IMO Res. MSC.148(77), — IMO Comsar Circ.32, — ITU-R M.540-2 (06/90), — ITU-R M.625-3 (10/95). | IEC 60945 (2002), — IEC 61097-6 (2005-12). | M P |
| EGC receiver | Reg. IV/14, — Reg. X/3, — IMO Res. MSC.36(63)-(1994 HSC code) 14, — IMO Res. MSC.97(73)-(2000 HSC code) 14. | Reg. IV/7, — Reg. X/3, — IMO Res. A.570(14), — IMO Res. A.664(16), — IMO Res. A.694(17), — IMO Res. MSC.36(63)-(1994 HSC code) 14, — IMO Res. MSC.97(73)-(2000 HSC code) 14, — IMO Comsar Circ.32. | IEC 60945 (2002), — IEC 61097-4 (1994). | M P |
| HF marine safety information (MSI) equipment (HF NBDP receiver) | Reg. IV/14, — Reg. X/3, — IMO Res. MSC.36(63)-(1994 HSC code) 14, — IMO Res. MSC.97(73)-(2000 HSC code) 14 | Reg. IV/7, — Reg. X/3, — IMO Res. A.694(17), — IMO Res. A.699(17), — IMO Res. A.700(17), — IMO Res. A.806(19), — IMO Res. MSC.36(63)-(1994 HSC code) 14, — IMO Res. MSC.97(73)-(2000 HSC code) 14, — IMO Comsar Circ.32, — ITU-R M.491-1 (07/86), — ITU-R M.492-6 (10/95), — ITU-R M.540-2 (06/90), — ITU-R M.625-3 (10/95), — ITU-R M.688 (06/90). | ETSI ETS 300067 Ed.1 (1990-11), — ETSI ETS 300067/A1 Ed.1 (1993-10), — EN 60945 (2002), — EN 61162 Series. | M P |
| 406 MHz EPIRB (Cospas-Sarsat) | Reg. IV/14, — Reg. X/3, — IMO Res. MSC.36(63)-(1994 HSC code) 14, — IMO Res. MSC.97(73)-(2000 HSC code) 14. | Reg. IV/7, — Reg. X/3, — IMO Res. A.662(16), — IMO Res. A.694(17), — IMO Res. A.696(17), — IMO Res. A.810(19), — IMO Res. MSC.36(63)-(1994 HSC code) 14, — IMO Res. MSC.97(73)-(2000 HSC code) 14, — IMO MSC/Circ.862, — IMO Comsar Circ.32, — ITU-R M.633-2 (05/00), — ITU-R M.690-1 (10/95). | IEC 60945 (2002), — IEC 61097-2 (2002), — IMO MSC/Circ.862. Note: IMO MSC/Circ.862 is applicable only to the optional remote activation device, not to the EPIRB itself. | X |
| MF radio capable of transmitting and receiving DSC and radiotelephony | Reg. IV/14, — Reg. X/3, — IMO Res. MSC.36(63)-(1994 HSC code) 14, — IMO Res. MSC.97(73)-(2000 HSC code) 14. | Reg. IV/9, — Reg. IV/10, — Reg. X/3, — IMO Res. A.694(17), — IMO Res. A.804(19), — IMO Res. MSC.36(63)-(1994 HSC code) 14, — IMO Res. MSC.97(73)-(2000 HSC code) 14, — IMO Comsar Circ.32, — ITU-R M.493-12 (03/07), — ITU-R M.541-8 (10/97). | IEC 60945 (2002), — IEC 61097-3 (1994), — IEC 61097-9 (1997), — IEC 61162 series, — IMO MSC/Circ.862. Note: In line with IMO and ITU decisions, the requirements for two-tone alarm generator and transmission on H3E are no longer applicable in the testing standards | M P |
| MF DSC watchkeeping receiver | Reg. IV/14, — Reg. X/3, — IMO Res. MSC.36(63)-(1994 HSC code) 14, — IMO Res. MSC.97(73)-(2000 HSC code) 14. | Reg. IV/9, — Reg. IV/10, — Reg. X/3, — IMO Res. A.694(17), — IMO Res. A.804(19), — IMO Res. MSC.36(63)-(1994 HSC code) 14, — IMO Res. MSC.97(73)-(2000 HSC code) 14, — IMO Comsar Circ.32, — ITU-R M.493-12 (03/07), — ITU-R M.541-8 (10/97), — ITU-R M.1173 (10/95). | IEC 60945 (2002), — IEC 61097-3 (1994), — IEC 61097-8 (1998). | X |
| Inmarsat-B SES | Reg. IV/14, — Reg. X/3, — IMO Res. MSC.36(63)-(1994 HSC code) 14, — IMO Res. MSC.97(73)-(2000 HSC code) 14. | Reg. IV/10, — Reg. X/3, — IMO Res. A.570(14), — IMO Res. A.694(17), — IMO Res. A.808(19), — IMO Res. MSC.36(63)-(1994 HSC code) 14, — IMO Res. MSC.97(73)-(2000 HSC code) 14, — IMO MSC/Circ.862, — IMO Comsar Circ.32. | IEC 60945 (2002), — IEC 61097-10 (1999), — IMO MSC/Circ 862. | X obsolete |
| Inmarsat-C SES | Reg. IV/14, — Reg. X/3, — IMO Res. MSC.36(63)-(1994 HSC code) 14, — IMO Res. MSC.97(73)-(2000 HSC code) 14. | Reg. IV/10, — Reg. X/3, — IMO Res. A.570(14), — IMO Res. A.664 (16), (applicable only if the Inmarsat C SES comprises EGC functions), — IMO Res. A.694(17), — IMO Res. A.807(19), — IMO Res. MSC.36(63)-(1994 HSC code) 14, — IMO Res. MSC.97(73)-(2000 HSC code) 14, — IMO MSC/Circ.862, — IMO Comsar Circ.32. | ETSI ETS 300460 Ed.1 (1996-05), — ETSI ETS 300460/A1 (1997-11), — ETSI EN 300829 V1.1.1 (1998-03), — EN 60945 (2002), — IEC 61097-4 (2007), — EN 61162 series, — IMO MSC/Circ.862. | E P |
| MF/HF radio capable of transmitting and receiving DSC, NBDP and radiotelephony | Reg. IV/14, — Reg. X/3, — IMO Res. MSC.36(63)-(1994 HSC code) 14, — IMO Res. MSC.97(73)-(2000 HSC code) 14. | Reg. IV/10, — Reg. X/3, — IMO Res. A.694(17), — IMO Res. A.806(19), — IMO Res. MSC.36(63)-(1994 HSC code) 14, — IMO Res. MSC.97(73)-(2000 HSC code) 14, — IMO MSC/Circ.862, — IMO Comsar Circ.32, — ITU-R M.476-5 (10/95), — ITU-R M.491-1 (07/86), — ITU-R M.492-6 (10/95), — ITU-R M.493-12 (03/07), — ITU-R M.541-8 (10/97), — ITU-R M.625-3 (10/95), — ITU-R M.1173 (10/95). | IEC 60945 (2002), — IEC 61097-3 (1994), — IEC 61097-9 (1997), — EN 61162 series, — IMO MSC/Circ.862. Note: In line with IMO and ITU decisions, the requirements for two-tone alarm generator and transmission on A3H are no longer applicable in testing standards. | M P |
| MF/HF DSC watchkeeping receiver | Reg. IV/14, — Reg. X/3, — IMO Res. MSC.36(63)-(1994 HSC code) 14, — IMO Res. MSC.97(73)-(2000 HSC code) 14. | Reg. IV/10, — Reg. X/3, — IMO Res. A.694(17), — IMO Res. A.806(19), — IMO Res. MSC.36(63)-(1994 HSC code) 14, — IMO Res. MSC.97(73)-(2000 HSC code) 14, — IMO Comsar Circ.32, — ITU-R M.493-12 (03/07), — ITU-R M.541-8 (10/97). | IEC 60945 (2002), — IEC 61097-3 (1994), — IEC 61097-8 (1998). | X |
| Aeronautical two-way VHF radio telephone apparatus | Reg. IV/14, — Reg. X/3, — IMO Res. MSC.36(63)-(1994 HSC code) 14, — IMO Res. MSC.97(73)-(2000 HSC code) 14. | Reg. IV/7, — IMO Res. A.694(17), — IMO Res. MSC.36(63)-(1994 HSC code) 14, — IMO Res. MSC.97(73)-(2000 HSC code) 14, — IMO Res. MSC.80(70), — IMO Comsar Circ.32, — ICAO Convention, Annex 10, Radio Regulations. | ETSI EN 301688 V1.1.1 (2000-07), — IEC 60945 (2002). | X |
| Portable survival craft two-way VHF radiotelephone apparatus | Reg. IV/14, — Reg. X/3, — IMO Res. MSC.36(63)-(1994 HSC code) 14, — IMO Res. MSC.97(73)-(2000 HSC code) 14. | Reg. III/6, — IMO Res. A.694(17), — IMO Res. A.809(19), — IMO Res. MSC.36(63)-(1994 HSC code) 8, 14, — IMO Res. MSC.97(73)-(2000 HSC code) 8, 14, — IMO Res. MSC.149(77), — ITU-R M.489-2 (10/95), — ITU-R M.542.1 (07/82). | IEC 60945 (2002), — IEC 61097-12 (1996). | X |
| Fixed survival craft two-way VHF radiotelephone apparatus | Reg. IV/14, — Reg. X/3, — IMO Res. MSC.36(63)-(1994 HSC code) 14, — IMO Res. MSC.97(73)-(2000 HSC code) 14. | Reg. III/6, — IMO Res. A.694(17), — IMO Res. A.809(19), — IMO Res. MSC.36(63)-(1994 HSC code) 8, 14, — IMO Res. MSC.97(73)-(2000 HSC code) 8, 14, — ITU-R M.489-2 (10/95). | IEC 60945 (2002), — IEC 61097-12 (1996). | X |
| Inmarsat-F SES | Reg. IV/14, — Reg. X/3, — IMO Res. MSC.36(63)-(1994 HSC code) 14, — IMO Res. MSC.97(73)-(2000 HSC code) 14. | Reg. IV/10, — IMO Res. A.570 (14), — IMO Res. A.808 (19), — IMO Res. A.694 (17), — IMO Res. MSC.36(63)-(1994 HSC code) 14, — IMO Res. MSC.97(73)-(2000 HSC code) 14, — IMO MSC/Circ.862, — IMO Comsar Circ.32. | IEC 60945 (2002), — IEC 61097-13 (2003), — IMO MSC/Circ.862. | E |

Table 2 - Navigation equipment

(Including those required by SOLAS CH III)

E = can be used without modification to the existing standards

F = future upgrade needed for interfacing

M = standards need to be modified for e-NAV

X = not part of e-Navigation (exempt)

P = presentation rules may apply

| Item designation | SOLAS 74 where “type approval” is required | Regulations of SOLAS 74 and the relevant resolutions and circulars of the IMO, as applicable | Testing standards | e-Nav |
| --- | --- | --- | --- | --- |
| Integrated Navigation System (INS) | Reg. V/18 Not currently a carriage requirement | Reg. V19 IMO Res.A.694(17) IMO Res. MSC.252(83) | IEC 60945(2002) IEC 61924 ed 2(tba) | E P M |
| Magnetic compass | Reg. V/18. | Reg. V/19, — IMO Res. A.382(X), — IMO Res. A.694(17). | ISO 449 (1997), — ISO 694 (2000), — ISO 1069 (1973), — ISO 2269 (1992), — IEC 60945 (2002). | E |
| Transmitting heading device THD (magnetic method) | Reg. V/18, — Reg. V/19, — Reg. X/3, — IMO Res. MSC.36(63)-(1994 HSC code) 13, — IMO Res. MSC.97(73)-(2000 HSC code) 13. | Reg. V/19, — IMO Res. A.694(17), — IMO Res. MSC.36(63)-(1994 HSC code) 13, — IMO Res. MSC.97(73)-(2000 HSC code) 13, — IMO Res. MSC.116(73). | IEC 60945 (2002), — IEC 61162 series. — ISO 22090-2 (2004), including Corrigendum 2005. | E F |
| Gyro compass | Reg. V/18. |  | ISO 8728 (1997), — IEC 60945 (2002), — IEC 61162 series, — IEC 62288 Ed.1.0(2008). | E F |
| Echo Sounding Equipment | Reg. V/18, — Reg. X/3, — IMO Res. MSC.36(63)-(1994 HSC code) 13, — IMO Res. MSC.97(73)-(2000 HSC code) 13. | Reg. V/19, — IMO Res. A.424(XI), — IMO Res. A.694(17), Reg. V/19, — IMO Res. A.224(VII), — IMO Res. A.694(17), — IMO Res. MSC.36(63)-(1994 HSC code) 13, — IMO Res. MSC.74(69) Annex 4, — IMO Res. MSC.97(73)-(2000 HSC code) 13, — IMO Res. MSC.191(79).— IMO Res. MSC.191(79). | ISO 9875 (2000), — IEC 60945 (2002), — IEC 61162 series, — IEC 62288 Ed.1.0(2008). | E P F |
| Speed and Distance Measuring Equipment | Reg. V/18, — Reg. X/3, — IMO Res. MSC.36(63)-(1994 HSC code) 13, — IMO Res. MSC.97(73)-(2000 HSC code) 13. | Reg. V/19, — IMO Res. A.694(17), — IMO Res. A.824(19), — IMO Res. MSC.36(63)-(1994 HSC code) 13, — IMO Res. MSC.96(72), — IMO Res. MSC.97(73)-(2000 HSC code) 13, — IMO Res. MSC.191(79). | IEC 60945 (2002), — IEC 61023 (2007), — IEC 61162 series, — IEC 62288 Ed.1.0(2008) | E P F |
| Rate of Turn Indicator | Reg. V/18, — Reg. X/3, — IMO Res. MSC.36(63)-(1994 HSC code) 13, — IMO Res. MSC.97(73)-(2000 HSC code) 13. | Reg. V/19, — IMO Res. A.526(13), — IMO Res. A.694(17), — IMO Res. MSC.36(63)-(1994 HSC code) 13, — IMO Res. MSC.97(73)-(2000 HSC code) 13, — IMO Res. MSC.191(79). | IEC 60945 (2002), — IEC 61162 series, — ISO 20672 (2007), — IEC 62288 Ed.1.0(2008). | E P F |
| Loran-C equipment | Reg. V/18, — Reg. X/3, — IMO Res. MSC.36(63)-(1994 HSC code) 13, — IMO Res. MSC.97(73)-(2000 HSC code) 13. | Reg. V/19, — IMO Res. A.694(17), — IMO Res. A.818(19), — IMO Res. MSC.36(63)-(1994 HSC code) 13, — IMO Res. MSC.97(73)-(2000 HSC code) 13, — IMO Res. MSC.191(79). | IEC 60945 (2002), — IEC 61075 (1991), — IEC 61162 series, — IEC 62288 Ed.1.0(2008). | obsol |
| Chayka equipment | Reg. V/18, — Reg. X/3, — IMO Res. MSC.36(63)-(1994 HSC code) 13, — IMO Res. MSC.97(73)-(2000 HSC code) 13. | Reg. V/19, — IMO Res. A.694 (17), — IMO Res. A.818 (19), — IMO Res. MSC.36(63)-(1994 HSC code) 13, — IMO Res. MSC.97(73)-(2000 HSC code) 13, — IMO Res. MSC.191(79). | IEC 60945 (2002), — IEC 61075 (1991), — IEC 61162 series, — IEC 62288 Ed.1.0(2008). | X |
| GPS equipment | Reg. V/18, — Reg. X/3, — IMO Res. MSC.36(63)-(1994 HSC code) 13, — IMO Res. MSC.97(73)-(2000 HSC code) 13. | Reg. V/19, — IMO Res. A.694(17), — IMO Res. MSC.36(63)-(1994 HSC code), — IMO Res. MSC.97(73)-(2000 HSC code), — IMO Res. MSC.112(73), — IMO Res. MSC.191(79). | IEC 60945 (2002), — IEC 61108-1 (2003), — IEC 61162 series, — IEC 62288 Ed.1.0(2008). | E F P |
| Glonass equipment | Reg. V/18, — Reg. X/3, — IMO Res. MSC.36(63)-(1994 HSC code) 13, — IMO Res. MSC.97(73)-(2000 HSC code) 13. | Reg. V/19, — IMO Res. A.694(17), — IMO Res. MSC.36(63)-(1994 HSC code) 13, — IMO Res. MSC.97(73)-(2000 HSC code) 13, — IMO Res. MSC.113(73), — IMO Res. MSC.191(79). | IEC 60945 (2002), — IEC 61108-2 (1998), — IEC 61162 series, — IEC 62288 Ed.1.0(2008). | E F P |
| Galileo | Reg V/18 IMO res xxx | ????? |  | E F P |
| Heading control system (HCS) | Reg. V/18. | Reg. V/19, — IMO Res. A.342(IX), — IMO Res. A.694(17), — IMO Res. MSC.64(67) Annex 3, — IMO Res. MSC.191(79). | ISO 11674 (2000), — IEC 60945 (2002), — IEC 61162 series, — IEC 62288 Ed.1.0(2008). | X |
| 9 GHz SAR transponder (SART) | Reg. III/4, — Reg. IV/14, — Reg. V/18, — Reg. X/3, — IMO Res. MSC.36(63)-(1994 HSC code) 13, — IMO Res. MSC.97(73)-(2000 HSC code) 13. | Reg. III/6, — Reg. IV/7, — IMO Res. A.530(13), — IMO Res. A.802(19), — IMO Res. A.694(17), — IMO Res. MSC.36(63)-(1994 HSC code) 8, 14, — IMO Res. MSC.97(73)-(2000 HSC code) 8, 14, — ITU-R M.628-3(11/93). | IEC 60945 (2002), — IEC 61097-1 (2007). | X |
| Rudder angle indicator | Reg. V/18, — Reg. X/3, — IMO Res. MSC.36(63)-(1994 HSC code) 13, — IMO Res. MSC.97(73)-(2000 HSC code) 13. | Reg. V/19, — IMO Res. A.526(13), — IMO Res. A.694(17), — IMO Res. MSC.36(63)-(1994 HSC code) 13, — IMO Res. MSC.97(73)-(2000 HSC code) 13, — IMO Res. MSC.191(79). | IEC 60945 (2002), — ISO 20673 (2007), — IEC 62288 Ed.1.0(2008). | M F P |
| Propeller revolution indicator | Reg. V/18, | Reg. V/19, — IMO Res. A.694(17), — IMO Res. MSC.191(79), | IEC 60945 (2002), — ISO 22554 (2007), — IEC 62288 Ed.1.0(2008). | M F P |
| Pitch indicator | Reg. V/18, | Reg. V/19, — IMO Res. A.694(17), — IMO Res. MSC.191(79). | IEC 60945 (2002), — ISO 22555 (2007), — IEC 62288 Ed.1.0(2008). | M F P |
| Compass for lifeboats and rescue boats | Reg. III/4, — Reg. X/3, — IMO Res. MSC.36(63)-(1994 HSC code) 13, — IMO Res. MSC.97(73)-(2000 HSC code) 13. | Reg. III/34, — IMO Res. MSC.48(66)-(LSA code) IV, V, — IMO Res. MSC.36(63)-(1994 HSC code) 8, 13, — IMO Res. MSC.97(73)-(2000 HSC code) 8, 13. | ISO 613 (2000), — ISO 10316 (1990). | X |
| Voyage data recorder (VDR) | Reg. V/18, — Reg. V/20, — Reg. X/3, — IMO Res. MSC.36(63)-(1994 HSC code) 13, — IMO Res. MSC.97(73)-(2000 HSC code) 13. | Reg. V/20, — IMO Res. A.694 (17), — IMO Res. A.861 (20), — IMO Res. MSC.36(63)-(1994 HSC code) 13, — IMO Res. MSC.97(73)-(2000 HSC code) 13, — IMO Res. MSC.214(81), — IMO Res. MSC.191(79). | IEC 60945 (2002), — IEC 61162 Series, — IEC 61996-1 (2007-11), — IEC 62288 Ed.1.0(2008). | M F |
| Electronic chart display and information system (ECDIS) with backup, and raster chart display system (RCDS) | Reg. V/18, — Reg. X/3, — IMO Res. MSC.36(63)-(1994 HSC code) 13, — IMO Res. MSC.97(73)-(2000 HSC code) 13. | Reg. V/19, — IMO Res. A.694(17), — IMO Res. MSC.36(63)-(1994 HSC code) 13 — IMO Res. MSC.64(67), — IMO Res. MSC.86(70), — IMO Res. MSC.97(73)-(2000 HSC code) 13, — IMO Res. MSC.191(79), — IMO Res. MSC.232(82). [ECDIS back-up and RCDS are only applicable when this functionality is included in the ECDIS. The module B certificate shall indicate whether these options were tested.] | IEC 60945 (2002), — IEC 61162 Series, — IEC 61174 (2008), — IEC 62288 Ed.1.0(2008). | E F P |
| Gyro compass for high-speed craft | Reg. X/3, — IMO Res. MSC.36(63)-(1994 HSC code) 13, — IMO Res. MSC.97(73)-(2000 HSC code) 13. | IMO Res. A.694(17), — IMO Res. A.821(19), — IMO Res. MSC.36(63)-(1994 HSC code) 13, — IMO Res. MSC.97(73)-(2000 HSC code) 13, — IMO Res. MSC.191(79). | ISO 16328 (2001), — IEC 60945 (2002), — EN 61162 Series, — IEC 62288 Ed.1.0(2008). | E F |
| Universal automatic identification system equipment (AIS) | Reg. V/18, — Reg. X/3, — IMO Res. MSC.36(63)-(1994 HSC code) 13, — IMO Res. MSC.97(73)-(2000 HSC code) 13. | Reg. V/19, — IMO Res. A.694 (17), — IMO Res. MSC.36(63)-(1994 HSC code) 13, — IMO Res. MSC.74(69), — IMO Res. MSC.97(73)-(2000 HSC code) 13, — IMO Res. MSC.191(79), — ITU-R M. 1371-3(2007). Note: ITU-R M. 1371-3(2007) Annex 3 shall only be applicable in accordance with requirements of IMO Res. MSC.74(69). | IEC 60945 (2002), — IEC 61162 Series, — IEC 61993-2 (2001), — IEC 62288 Ed.1.0(2008). | E P F |
| Track control system (working at ship’s speed from minimum manoeuvring speed up to 30 knots) | Reg. V/18. | Reg. V/19, — IMO Res. A.694(17), — IMO Res. MSC.74(69). | IEC 60945 (2002), — IEC 61162 Series, — IEC 62065 (2002). | E F P |
| Radar equipment CAT 1 | Reg. V/18. | Reg. V/19. — IMO Res. A.278(VIII), — IMO Res. A.694(17), — IMO Res. A.823(19), — IMO Res. MSC.191(79), — IMO Res. MSC.192(79), — ITU-R M. 628-3(11/93), — ITU-R M. 1177-3(06/03). | IEC 60945 (2002), — IEC 61162 Series, — IEC 62288 Ed.1.0(2008), — IEC 62388 Ed.1.0(2007). | E F P |
| Radar equipment CAT 2 | Reg. V/18. | Reg. V/19, — IMO Res. A.278(VIII), — IMO Res. A.694(17), — IMO Res. MSC.191(79), — IMO Res. MSC.192(79), — ITU-R M. 628-3(11/93), — ITU-R M. 1177-3(06/03). | IEC 60945 (2002), — IEC 61162 Series, — IEC 62288 Ed.1.0(2008), — IEC 62388 Ed.1.0(2007). | E F P |
| Radar equipment CAT 3 | Reg. V/18. | Reg. V/19, — IMO Res. A.278(VIII), — IMO Res. A.694(17), — IMO Res. MSC.191(79), — IMO Res. MSC.192(79), — ITU-R M. 628-3(11/93), — ITU-R M. 1177-3(06/03). | IEC 60945 (2002), — IEC 61162 Series, — IEC 62288 Ed.1.0(2008), — IEC 62388 Ed.1.0(2007). | E F P |
| Radar equipment for high-speed craft applications (CAT 1H, CAT 2H and CAT 3H) | Reg. X/3, — IMO Res. MSC.36(63)-(1994 HSC code) 13, — IMO Res. MSC.97(73)-(2000 HSC code) 13. | IMO Res. A.278(VIII), — IMO Res. A.694(17), — IMO Res. A.820(19), — IMO Res. MSC.36(63)-(1994 HSC code) 13, — IMO Res. MSC.97(73)-(2000 HSC code) 13, — IMO Res. MSC.191(79), — IMO Res. MSC.192(79), — ITU-R M. 628-3(11/93), — ITU-R M. 1177-3(06/03). | IEC 60945 (2002), — IEC 61162 Series, — IEC 62288 Ed.1.0(2008), — IEC 62388 Ed.1.0(2007). | E F P |
| Radar equipment approved with a chart option (CAT 1HC, CAT 2HC and CAT 3HC) | Reg. X/3, — IMO Res. MSC.36(63)-(1994 HSC code) 13, — IMO Res. MSC.97(73)-(2000 HSC code) 13. | IMO Res. A.278(VIII), — IMO Res. A.694(17), — IMO Res. A.820(19), — IMO Res. MSC.36(63)-(1994 HSC code) 13, — IMO Res. MSC.97(73)-(2000 HSC code) 13, — IMO Res. MSC.191(79), — IMO Res. MSC.192(79), — ITU-R M. 628-3(11/93), — ITU-R M. 1177-3(06/03). | IEC 60945 (2002), — IEC 61162 Series, — IEC 62288 Ed.1.0(2008), — IEC 62388 Ed.1.0(2007). | E F P |
| Radar reflector | Reg. V/18, — Reg. X/3, — IMO Res. MSC.36(63)-(1994 HSC code) 13, — IMO Res. MSC.97(73)-(2000 HSC code) 13. | Reg. V/19, — IMO Res. MSC.36(63)-(1994 HSC code) 13, — IMO Res. MSC.97(73)-(2000 HSC code) 13, — IMO Res. MSC.164(78). | ISO 8729 (1997), — IEC 60945 (2002). | X |
| Heading control system for high-speed craft | Reg. X/3, — IMO Res. MSC.36(63)-(1994 HSC code) 13, — IMO Res. MSC.97(73)-(2000 HSC code) 13. | IMO Res. A.694(17), — IMO Res. A.822(19), — IMO Res. MSC.36(63)-(1994 HSC code) 13, — IMO Res. MSC.97(73)-(2000 HSC code) 13, — IMO Res. MSC.191(79). | ISO 16329 (2003), — IEC 60945 (2002), — IEC 61162 series, — IEC 62288 Ed.1.0(2008). | X |
| Transmitting heading device THD (GNSS method) | Reg. V/18, — Reg. X/3, — IMO Res. MSC.36(63)-(1994 HSC code) 13, — IMO Res. MSC.97(73)-(2000 HSC code) 13. | Reg. V/19, — IMO Res. A.694(17), — IMO Res. MSC.36(63)-(1994 HSC code) 13, — IMO Res. MSC.97(73)-(2000 HSC code) 13, — IMO Res. MSC.116(73), — IMO Res. MSC.191(79). | ISO 22090-3 (2004), — IEC 60945 (2002), — IEC 61162 series, — IEC 62288 Ed.1.0(2008). | E F |
| Searchlight for high-speed craft | Reg. X/3, — IMO Res. MSC.36(63)-(1994 HSC code) 13, — IMO Res. MSC.97(73)-(2000 HSC code) 13. | IMO Res. A.694(17), — IMO Res. MSC.36(63)-(1994 HSC code) 13, — IMO Res. MSC.97(73)-(2000 HSC code) 13. | ISO 17884 (2004), — IEC 60945 (2002). | X |
| Night vision equipment for high-speed craft | Reg. X/3, — IMO Res. MSC.36(63)-(1994 HSC code) 13, — IMO Res. MSC.97(73)-(2000 HSC code) 13. | IMO Res.A.694(17), — IMO Res. MSC.36(63)-(1994 HSC code) 13, — IMO Res. MSC.94(72), — IMO Res. MSC.97(73)-(2000 HSC code) 13. | ISO 16273 (2003), — EN 60945 (2002), or — ISO 16273 (2003), — IEC 60945 (2002). | X |
| Differential beacon receiver for DGPS and D Glonass equipment | Reg. V/18, — Reg. X/3, — IMO Res. MSC.36(63)-(1994 HSC code) 13, — IMO Res. MSC.97(73)-(2000 HSC code) 13. | Reg. V/19, — IMO Res. A.694 (17), — IMO Res. MSC.36(63)-(1994 HSC code) 13, — IMO Res. MSC.97(73)-(2000 HSC code) 13, — IMO Res. MSC.114(73). | IEC 60945 (2002), — IEC 61108-4 (2004), — IEC 61162 series. | E |
| Chart facilities for shipborne radar | Reg. V/18, — Reg. X/3, — IMO Res. MSC.36(63)-(1994 HSC code) 13, — IMO Res. MSC.97(73)-(2000 HSC code) 13. | Reg. V/19, — IMO Res. A.694(17), — IMO Res. A.817(19), — IMO Res. MSC.36(63)-(1994 HSC code) 13, — IMO Res. MSC.64(67), — IMO Res. MSC.97(73)-(2000 HSC code) 13, — IMO Res. MSC.191(79), — IMO Res. MSC.192(79). | IEC 60936-3 (2002), — IEC 60945 (2002), — IEC 61162 series, — IEC 62288 Ed.1.0(2008), — IEC 62388 Ed.1.0(2007). | X ?? |
| Transmitting heading device THD (gyroscopic method) | Reg. V/18. — Reg. X/3, — IMO Res. MSC.36(63)-(1994 HSC code) 13, — IMO Res. MSC.97(73)-(2000 HSC code) 13. | Reg. V/19, — IMO Res. A.694 (17), — IMO Res. MSC.36(63)-(1994 HSC code) 13, — IMO Res. MSC.97(73)-(2000 HSC code) 13, — IMO Res. MSC.116(73). | ISO 22090-1 (2002) including Corr.1 (2005), — IEC 60945 (2002), — IEC 61162 series. | E F |
| Simplified voyage data recorder (S-VDR) | Reg. V/20. | Reg. V/20, — IMO Res. A.694(17), — IMO Res. MSC.163(78), — IMO Res. MSC.214(81), — IMO Res. MSC.191(79). | IEC 60945 (2002), — IEC 61162 series, — IEC 61996-2 (2007), — IEC 62288 Ed.1.0(2008). | obs |
| DGPS equipment | Reg. V/18, — Reg. X/3, — IMO Res. MSC.36(63)-(1994 HSC code) 13, — IMO Res. MSC.97(73)-(2000 HSC code) 13. | Reg. V/19, — IMO Res. A.694 (17), — IMO Res. MSC.36(63)-(1994 HSC code) 13, — IMO Res. MSC.97(73)-(2000 HSC code) 13, — IMO Res. MSC.114(73), — IMO Res. MSC.191(79). | IEC 60945 (2002), — IEC 61108-1 (2003), — IEC 61108-4 (2004), — IEC 61162 series, — IEC 62288 Ed.1.0(2008). | E F P |
| D Glonass equipment | Reg. V/18, — Reg. X/3, — IMO Res. MSC.36(63)-(1994 HSC code) 13, — IMO Res. MSC.97(73)-(2000 HSC code) 13. | Reg. V/19, — IMO Res. A.694 (17), — IMO Res. MSC.36(63)-(1994 HSC code) 13, — IMO Res. MSC.97(73)-(2000 HSC code) 13, — IMO Res. MSC.114(73), — IMO Res. MSC.191(79). | IEC 60945 (2002), — IEC 61108-2 (1998), — IEC 61108-4 (2004), — IEC 61162 series, — IEC 62288 Ed.1.0(2008). | E F P |