| Sub Committee on Safety of Navigation. 57:th. session Agenda item 5. | NAV57/5/XXl 25 April 2011 Original: ENGLISH |
| --- | --- |

Clarifications on AIS Navigation Status

Submitted by IALA

| SUMMARY |  |
| --- | --- |
| Executive summary: | This document proposes a change in the use of some values of the Navigational Status parameter in AIS messages. |
| Strategic direction: |  |
| High-level action: |  |
| Planned output: | Liaison with ITU |
| Action to be taken: | Paragraph 3 |
| Related documents: | Recommendation ITU-R M 1371-4 |

# Introduction

At its 56th session, the Navigation Sub-Committee invited interested parties to make proposals for the future use of 3 of the 13 Navigation Status parameters which were available for future definition, as defined in Recommendation ITU-R M.1371-4.  IALA welcomes this invitation and proposes 3 new definitions for these available statuses.  It also wishes to bring to the Organization attention the many requests IALA (and other Administrations) has received from the AIS user community to clarify some of the status provided within Recommendation ITU-R M.1371 and used in Messages 1, 2, and 3 Position Reports.

# Navigation Status parameter

Below are the current NavStatus options and descriptive texts that exist within ITU-R M.1371

0 = under way using engine,

1 = at anchor,

2 = not under command,

3 = restricted manoeuvrability,

4 = constrained by her draught,

5 = moored,

6 = aground,

7 = engaged in fishing,

8 = under,

9 = reserved for future amendment of navigational status for ships carrying DG, HS, or MP, or IMO hazard or pollutant category C, high speed craft (HSC),

10 = reserved for future amendment of navigational status for ships carrying dangerous goods (DG), harmful substances (HS) or marine pollutants (MP), or IMO hazard or pollutant category A, wing in grand (WIG);

11-13 = reserved for future use,

14 = AIS-SART (active)

15 = not defined = default (also used by AIS-SART under test)

Some of these NavStatus descriptions vary slightly from commonly known notion such as what is found in COLREGS72.  To mitigate possible confusion or ambiguity amongst users, IALA proposes the following clarifications denoted by strikeout and underlines.  IALA also proposes to define NavStatus 11-13, which are currently reserved for future use, so as to allow additional pertinent navigation safety information to be provided by a growing number of new users and devices, such vessels engaged in towing and AIS man-overboard devices.

Further, IALA proposes to amend NavStatus 9 and 10 to be “reserved for future use” but not solely reserved for WIG or IMDG vessels.

0 = underway using engine, propelled by machinery,

1 = at anchor,

2 = not under command,

3 = restricted in ability to manoeuvre,

4 = constrained by her draught,

5 = moored or at berth,

6 = aground,

7 = engaged in fishing,

8 = under way sailing

9 = reserved for future use amendment of navigational status for ships carrying DG, HS, or MP, or IMO hazard or pollutant category C, high speed craft (HSC),

10 = reserved for future use amendment of navigational status for ships carrying dangerous goods (DG), harmful substances (HS) or marine pollutants (MP), or IMO hazard or pollutant category A, wing in grand (WIG),

11 = reserved for future use power-driven vessel towing astern,

12 = reserved for future use power-driven vessel pushing ahead or towing alongside,

13= reserved for future use requiring assistance,

14 = AIS-SART (active)

15 = not defined = default (also used by AIS-SART under test)

# Action requested of the Sub-Committee

The Sub-Committee is invited to note the information provided and to liaise with ITU to make the necessary clarifications to ITU 1371-4.