IALA Guideline

DraFT G1111-3

Producing Functional and Performance Requirements for RADAR systems

NOTE: This is a draft copy for submission to VTS 50, representing state of work on February 10, 2021. Work is expected to continue during VTS 50

Prepared by intersessional working group:

- Rostopshin, Dmitry <dmitry.rostopshin@wartsila.com>

- Steve Guest <steve.guest@kongsberg.com>

- Hogendoorn, Rene <rene.hogendoorn@saabgroup.com>

- Jens Chr. Pedersen <JCP@terma.com>

Edition x.x

Date (of approval by Council)

urn:mrn:iala:pub:g1111-3(First draft)

Revisions to this document are to be noted in the table prior to the issue of a revised document.

| Date | Details | Approval |
| --- | --- | --- |
|  | Edition 1.0 This document originated from Guideline G1111 which has been subdivided into 13 sub-guidelines, including this document. Document structure revised, Basic, Standard and Advanced substituted with guidance on specific areas including Inland VTS, Ports, Ports Approach and Coastal VTS. Guidance on off shore related VTS and Acceptance of VTS Radar Systems added. (Note - G1111 originated from annex of Recommendation V-128 Ed 3 in May 2015) |  |

1.	INTRODUCTION	6

1.1.	G1111 guideline series	6

2.	DEFINITIONS	7

2.1.	GENERAL TERMS	7

2.2.	SPECIFIC TERMS	7

2.3.	Speciffic IALA Definitions	9

2.3.1.	Target Types	9

2.3.2.	Range and Azimuth Resolution	10

2.3.3.	Target Separation	10

2.3.4.	Dynamic range	10

3.	References	10

4.	Abbreviations	11

5.	Operational Areas	12

5.1.	Inland Waterway	12

5.2.	Port	12

5.3.	Port Approach	12

5.4.	Coastal Area	13

5.5.	Off-Shore	13

6.	VTS Radar Considerations	14

6.1.	Radar Types	14

6.1.1.	Pulse Radar	14

6.1.2.	Pulse Compression Radar	14

6.1.3.	Frequency Modulated Continuous Wave Radar	15

6.2.	Frequency Bands	16

6.3.	Antennas	16

6.3.1.	Antenna Principles	16

6.3.2.	Polarisation	17

6.4.	Radar capabilities and constraints	17

6.4.1.	Radar coverage	17

6.4.2.	Target separation	18

6.4.3.	Target Positional Accuracy	20

6.4.4.	Dynamic characteristics	21

6.4.5.	Sidelobes	22

6.5.	Target caracteristics	23

6.5.1.	Radar Cross Section	23

6.5.2.	Target Models versus Detailed Characteristics	24

6.5.3.	Target RCS Fluctuations	25

6.5.4.	Target Speed and Manoeuvrability	25

6.5.5.	Multipath Effects	25

6.5.6.	Influence on Point Targets (Lobing)	26

6.6.	Environmental Influence	26

6.6.1.	Precipitation	27

6.6.2.	Sea State	27

6.6.3.	Reduced Visibility, Fog, sand, Dust	27

6.6.4.	Air Mass (Propagation)	27

6.7.	Other Influencing Factors	31

6.7.1.	Shadowing Effects	31

6.7.2.	Interference	31

6.7.3.	Radar Susceptibility	32

6.7.4.	Radar Compatibility with Other Users	32

6.7.5.	Influence from Wind Farms	32

6.8.	Signal Processing and Tracking	33

6.9.	Analysis of Radar Detection Performance	33

6.9.1.	Probability of Detection and False Alarm Rate	33

7.	Producing requirements	35

7.1.	Radar Coverage	35

7.2.	Target Detection and Range Performance	35

7.2.1.	Range performance	35

7.2.2.	Target Separation and Accuracy	36

7.2.3.	Target Separation	36

7.2.4.	Range Separation for Small Point Targets	36

7.2.5.	Azimuth Separation for Small Point Targets	37

7.2.6.	Separation of Larger Targets	37

7.3.	Target Positional Accuracy	38

7.4.	Target Positional Update rate	39

7.5.	Elimination of False Echoes, Side lobes & Dynamic range	39

7.5.1.	Elimination of False Echoes	39

7.5.2.	Acceptable side lobe levels	39

7.6.	Handling of Dynamic Range, targets and surroundings	40

7.7.	Handling of environmental Conditions	40

7.7.1.	Precipitation	40

7.7.2.	Sea Conditions	40

7.8.	Functions and other Features	41

7.8.1.	Clutter and Noise Reduction / Management	41

7.8.2.	Operator Functions	41

7.8.3.	Operational Outputs	41

7.9.	Radar Design, Installation and Maintenance Considerations	41

7.9.1.	Service Access	42

7.9.2.	Antenna Accessibility	42

7.9.3.	Tower Considerations and Antenna Robustness	42

7.9.4.	Choice of Upmast versus Downmast Transceivers	42

7.9.5.	Built In Test Equipment	42

7.9.6.	Protection against Extreme Events	42

8.	Acceptance of VTS Radar Systems	43

8.1.	Verification of Coverage and Target Detection Performance	43

8.2.	Dynamic Capabilities	45

8.3.	Side Lobe Performance	45

8.4.	Target separation	45

8.4.1.	RANGE SEPARATION	45

8.4.2.	BEARING SEPARATION	45

8.5.	Target Positional accuracy	46

8.6.	Test Targets and their Callibration	46

List of Tables

List of Figures

# INTRODUCTION

This Guideline presents a common source of information to assist competent authorities and VTS authorities in the understanding of radar performance, supporting the design of radar service and its contribution to the VTS traffic image (situational awareness) as well as guidance of how the VTS Authority should specify the Functional, Operational and associated Validation Requirements.

The guideline considers application of radar to different operational areas (e.g. inland waterways, Harbours, Coastal regions and offshore).

Environmental conditions such as weather, sea conditions and geographical constraints pose challenges to the detection and coverage of radar sensors.

In some cases, there could be a need to detect small targets in heavy clutter conditions and small versus large target discrimination may be essential.

The use of multi-sensor integration, including radar, AIS and other sensors also needs to be considered.

Specific maritime security requirements possibly identified by the International Ship and Port Security code and other requirements from allied services may introduce additional challenges.

## G1111 guideline series

This Guideline is one of the G1111 series of guideline documents. The purpose of the G1111 series is to assist the VTS authority in preparing the definition, specification, establishment, operation, and upgrades of a VTS system.  The documents address the relationship between the operational requirements and VTS system performance (technical) requirements and how these reflect into system design and sub system requirements.

The G1111 series of guideline documents present system design, sensors, communications, processing, and acceptance, without inferring priority. The guideline documents are numbered and titled as follows:

- G.1111 Establishing Functional and Performance Requirements for VTS systems

- G.1111-1 Producing Functional and Performance Requirements for the Core VTS system

- G.1111-2 Producing Functional and Performance Requirements for Voice Communications

- G.1111-3 Producing Functional and Performance Requirements for RADAR systems

- G.1111-4 Producing Functional and Performance Requirements for AIS and VDES systems

- G.1111-5 Producing   Functional   and   Performance   Requirements   for   Environment   Monitoring systems

- G.1111-6 Producing Functional and Performance Requirements for Electro Optical systems

- G.1111-7 Producing Functional and Performance Requirements for Radio Direction Finder systems

- G.1111-8 Producing Functional and Performance Requirements for Long Range Sensor systems

- G.1111-9 Producing Acceptance Requirements for VTS Systems

# DEFINITIONS

## GENERAL TERMS

For general terms used throughout this section refer to IEEE Std 686-1997 IEEE Standard Radar Definitions.

## SPECIFIC TERMS

Specific terms are defined as follows:

Azimuth (Antenna) Side Lobes - antenna responses (in azimuth) outside the intended radiation beam.  Weighting of the illumination function allows a significant reduction of these lobes, but some response outside the intended direction is unavoidable, normally presenting an irregular pattern with 'peaks' and 'nulls'.  The side lobes may produce responses from targets in unwanted directions, allowing disturbing signals (intentional or not) to enter the receiver, and in addition makes the radar detectable by receivers, which are not illuminated by the main beam.

Availability - is the probability that a system will perform its specified function when required.

Blind Spots – typically resulting from either blind range (the Range corresponding to an echo delay of one or more pulse repetition intervals: the echo then arrives at the receiver while the radar is transmitting a new pulse and the receiver is blanked) or Blind speed (target speeds which produce Doppler shift which are integer multiples of the radar pulse repetition frequency (PRF), which are therefore aliased to zero Doppler and cancelled by the clutter rejection filtering).  Blind spots can also arise behind significant obstructions in the field of view (buildings, land masses, oil tankers).

Chirp - frequency modulation of the carrier frequency applied within the radar pulse to increase its bandwidth and therefore the range resolution (see also pulse compression).

Coherence - capability of a system to keep a stable phase reference during the target illumination time in order to properly exploit the received phase information for moving target indication (MTI), pulse Doppler processing or other purposes.

Doppler Shift - shift in frequency of a wave due to the relative motion between the transmitter and the receiver.  Frequency shift is relative target velocity/wavelength.  Radar echoes are shifted twice this value because this shift must be accounted for in both the forward and the return path.

Doppler Side Lobes - when using Doppler processing (or MTI) the integrated ideal pulse always presents a response outside the integration peak (across all Doppler filters) known as Doppler side lobes.  Their main effect is to limit the capability to discriminate weak returns in proximity of strong returns (with side lobes of the same order of magnitude as the primary response of the weak return).

FMCW - Frequency Modulation - Continuous Wave - A type of radar where a continuous wave instead of pulse is transmitted.  The range information is derived by frequency modulating the carrier with a saw tooth waveform and comparing the echo FM modulation envelope with the reference.

Gap Filler -

Ghost Targets (Ghost Echoes) – undesirable radar echoes resulting from a number of sources.  For example, multipath related wave reflections caused by large structures or surface reflections, time sidelobes, antenna azimuth sidelobes, and Doppler sidelobes.

Interference Rejection - this function is included to seek to reduce or eliminate interference received from transmitters utilising the same or nearby frequencies.  One common technique is to compare adjacent range cells in the present 'live' video signal with the video signal from the previous sweep.  The output video signal to the display device is inhibited should the comparison indicate the presence of interference.

Normal Weather and Propagation Conditions - are the conditions not exceeded 99% to 99.9 % of the time as defined by the individual VTS Authority.  The rest of the time is considered having adverse weather and propagation conditions.

Operational area -

Plot Extraction – the process of determining the likely target related radar returns from the radar video signal.  This typically consists of comparing the video level with a threshold which can be (dynamically) adapted to local background noise and clutter conditions.

Polarisation – of a radar signal is determined by the orientation of the electrical field.  In the case of circular polarisation, the field rotates left or right.

Pulse – typically a pulse (which is modulated in the case of pulse compression radar) of RF energy transmitted from the radar.

Pulse Compression – A technique used to achieve a wide pulse bandwidth (and, therefore, enhanced range resolution) using long pulse (for high pulse energy with limited peak power) by introducing an intra-pulse modulation (e.g., chirp frequency modulation or Barker discrete phase modulation) and performing a correlation on the received echo.

Radar Cross Section (RCS) – an assessment of the cross-sectional area presented by a reflector (typically a target or unwanted “clutter”) to the transmitted radar energy.  The RCS can vary with frequency and target attitude.

Radar Information – a generic term potentially referring to the radar picture/video, target data, clutter data, topographical data, aids to navigation SARTs etc.

Radar PD – is the probability of target detection at the output of a radar, subsequent to plot extraction, but prior to tracking, and presentation.  Note, in some systems the boundary of the radar and its achieved PD complicate this definition – clarification may be required to avoid misunderstanding arising from, for example, data compression or video processing.

Radar PFA – is the probability of false alarm at the output of a radar, subsequent to plot extraction, but prior to tracking, and presentation.  In this context, the PFA is defined as relating to the number of false target declarations per radar cell (range cell x azimuth cells), arising from a noise plus clutter environment (only).  Note, in some systems the boundary of the radar and its achieved PFA complicate this definition – clarification may be required to avoid misunderstanding arising from, for example, noise related threshold crossings vs. unwanted radar energy reflections (unwanted targets, ghost targets etc.).

Radar Plot – is the generic term to describe the report resulting from a radar sensor observation. Each report contains positional information, possibly supplemented by other data.

Radar Target – an object about which information is sought with radar equipment.

Radar Video – a time-varying signal, proportional to the sum of the radio frequency (RF) signals being received and the RF noise inherent in the receiver itself.  Traditionally, radar video is an analogue signal with associated azimuth reference information.  Recently, radar systems have become available which provide equivalent data in digital format.

Radar Sensor – the transmitting, receiving and signal handling apparatus, delivering radar information to the tracking and presentation features of VTS.

Radar Service – a service that delivers all radar-derived data, such as radar image, radar plots and radar tracks.

Radar Tracks – a target report resulting from the correlation, by a special algorithm (tracking) of a succession of radar-reported positions (radar plots) for one object.  The report normally contains filtered position, speed vector information, identity, (e.g. track number).  Additional information may include, for example, track uncertainties, the associated plot, timestamp, track quality.

Range Ambiguous Returns – the measured range of a target typically assumes that the target true range is less than the first range ambiguity (the Range corresponding to an echo delay of one pulse repetition interval) whereas large targets beyond this range can be detected but typically with (incorrect) ambiguous range measurement.  Techniques exist for the resolution of range ambiguity if required.  See also blind spots above.

Range Side Lobes – see Time Side Lobes (below).

Receiver Dynamic Range – essentially the range of signal levels over which a receiver can operate.  The low end of the range is governed by its sensitivity whilst, at the high end, it is governed by its overload or strong signal handling performance.

Reliability – the probability that a system, when it is available performs a specified function without failure under given conditions for a given period of time.

Sea Characteristics – often described by sea state but additional parameters can also be of interest.  Sea characteristics include wave/swell height, direction and speed of waves/swell, distance between waves/swell, salinity etc.

Standard Atmospheric Condition – the International Commission of Air Navigation uses a definition for a standard atmosphere, defining temperature and pressure relative to the height.  In the troposphere (0 metres to 11,000 metres), the temperature is defined to be 15 °C at the surface and changing -6.5 °C/km.

Squint – the potential angular difference between antenna broadside and the antenna beam pointing direction.  This angular difference may change with transmission frequency.  The effect can be fully compensated.

Swerling Cases – a series of mathematical models representing RCS fluctuations to characterise the statistical behaviour of reflected radar signals from a target (see also target fluctuations).

Target Fluctuations – (also known as Glint or Swerling characteristic) - Fluctuation of a target radar cross section (RCS) (and, therefore, of the received echo amplitude) due to changes in the target attitude and illuminating frequency.  For complex targets (consisting of a number of reflecting surfaces), RCS is normally strongly dependent on the angle of observation.

Target Separation – (also known as Target resolution) – the ability to successfully identify two discrete detectable, similarly sized targets when closely spaced in either range or azimuth.

Track Swapping – the (usually unwanted) transfer of a track identity (track label) to another track.  This can break the intended association between a track and a physical object.

Time Side Lobes – when using pulse compression, the correlated pulse always presents responses outside the correlation peak (before and after it) known as time (or range) side lobes.  Their main effect is to limit the capability to discriminate weak returns in proximity of strong returns (with side lobes of the same order of magnitude as the primary response of the weak return).

## Speciffic IALA Definitions

### Target Types

For calculation purposes, the IALA simplified target types are defined in Error! Reference source not found..

Typical targets of interest are modelled as point targets with conservative estimate of Radar Cross Section and height.  This is normally sufficient for estimation of detection range for consideration in VTS radar sensor coverage.  However, further considerations are required for the overall design of radar systems as discussed later in this section.

Radar performance estimation, including concerns regarding fluctuations, is discussed later in this section.

Table 1 IALA Target Types

| IALA Point Target Types |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Target Type | Typically Representing | Radar Cross Section |  |  | Height (ASL) | Fluctuation |
|  |  | S‐Band | X‐Band | Ku-Band |  |  |
| 1 | AtoN without radar reflector. Small open boats, fiberglass, wood or rubber with outboard motor and, at least, 4 meters long. Small speedboats, small fishing vessels, and small sailing boats. | <<1 m2 | <<1 m2 | <<1 m2 | 1 m | Rapid, depending on sea state and target movement |
| 2 | In‐shore fishing vessels, sailing boats and speedboats. | <1 m2 | 3 m2 | <4 m2 | 2 m |  |
| 3 | Aids to Navigation with radar reflector. | 4 m2 | 10 m2 | 12 m2 | 3 m |  |
| 4 | Small metal ships, fishing vessels and patrol vessels. | 40 m2 | 100 m2 | 120 m2 | 5 m | Moderate |
| 5 | Small coasters and large fishing trawlers. | 400 m2 | 1,000 m2 | 1,200 m2 | 8 m |  |
| 6 | Large coasters, bulk carriers, and cargo ships. | 4,000 m2 | 10,000 m2 | 12,000 m2 | 12 m | Negligible |
| 7 | Container carriers and tankers. | 40,000 m2 | 100,000 m2 | 120,000 m2 | 18 m |  |

Note: RCS values are average values for the distribution of single pulse radar echoes.  The indicated values include allowance for the RCS-limiting effect of the cell size in the case of radars with high-resolution. See Table 2 for detailed target characteristics.

### Range and Azimuth Resolution

Within this document …..

### Target Separation

Within this document …..

### Dynamic range

Within this document …..

# References

- IEEE Std 686-1997 - IEEE Standard Radar Definitions

- Merrill I. Skolnik - Introduction to Radar Systems, McGraw-Hill Higher Education, ISBN 0-07-290980-3

- P.D.L. Williams, H.D, Cramp and Kay Curtis - Experimental study of the radar cross section of maritime targets, ELECTRONIC CIRCUITS AND SYSTEMS, July 1978. Vol. 2. No 4

- Ingo Harre - RCS in Radar Range Calculations for Maritime Targets (http://www.mar-it.de/Radar/RCS/RCS_18.pdf)

- International Maritime Organisation (IMO) - Performance Standards for radar reflectors

- ITU-R  SM.1541 - Unwanted emissions in the out-of-band domain

- ITU-R  SM.329-9 - Spurious emissions

- ISO 8729 Ships and marine technology – Marine radar reflectors

- International Commission for Air Navigation - Definition of the Standard Atmosphere

- AREPS: Advanced Refractive Effects Prediction System, Space and Naval Warfare Systems Center, San Diego, http://sunspot.spawar.navy.mil

- ISO 5725-1 ……….

# Abbreviations

Please refer to IALA G.1111 Establishing Functional and Performance Requirements for VTS systems for an extensive list of abbreviations and acronyms covering the entire G1111 series

# Operational Areas

The VTS Authority should specify Operational Requirements to derive the Technical Specifications of radar sensor(s). These Operational Requirements are highly dependent on the operational area, on the vessels of interest and, to a lesser extent, on the services provided the VTS. Operational areas may be characterised as follows:

- Inland waterways, like rivers and canals

- The port area

- The port approach area incl. anchorages

- Coastal areas, including shipping lanes

- Off-shore installations, like platforms and windfarms

These areas vary in the types of vessels and their interactions, the required sensor ranges and the types of services. Also, note that, within the total VTS area, there are generally several different operational areas.

## Inland Waterway

Inland waterways, like rivers and canals, are confined waters that are used by a large variety of vessels, ranging from sea-going vessels (if it is part of the port approach), river-trade, allied services vessels, ferries and even recreational vessels. Traffic separation may be as low as a few metres. This requires a high sensor position accuracy and target resolution performance. Locks, bridges and waterway intersections may be present that will affect traffic flow.

Although the required sensor range is limited, the sensor may not have a full view of the waterway due to obstructions. Sensor siting will require proper attention (especially when considering siting limitations).

Radar Characteristics – detection – accuracy - resolution

Detection ranges may vary from 1km for a gap-filler up to 10km. At larger ranges, azimuthal accuracy becomes the limiting factor. Table/graph XXX provides accuracy and resolution performance figures

Given the required accuracy and resolution for this operational area, the use of S-band radar is not recommended.

## Port

The port area consists of confined waters. Traffic consists of sea-going vessels, allied services vessels and, possibly, river-trade vessels (trans-shipping). Position accuracy and target resolution performance are comparable to those of inland waterways. The required sensor range is very limited, but the sensor view will be obstructed, even if temporarily, by cranes, moored ships, cargo and buildings. Special attention should be given to sensor siting and the possible occurrence of unwanted reflections.

Radar Characteristics – detection – accuracy – resolution – false target suppression

For port areas, required accuracy and resolution performances are similar to those for inland waterways (refer to Table/graph XXX).

## Port Approach

The port approach area is usually within the 12NM zone but may extend beyond this 12NM. There may be several types of operations within this area

- Vessels entering and leaving port, possibly crossing coastal traffic lanes

- Pilot (dis-)embarkation

- Anchorages with supply vessel traffic and, possibly, trans-shipping operations

- Fishery

- Renewable energy harvesting, fish farms, etcetera. In general, areas not allowed for ordinary maritime traffic

- Recreation

Coverage of this area translates into a medium required sensor range. Traffic of interest may range from small sailing boats up to >20k TEU container vessels. In dense traffic areas, e.g. pilot boarding areas, sensor resolution performance is important to be able to separate individual vessels.

Radar Characteristics – detection – accuracy – resolution

## Coastal Area

The coastal area may extend well beyond the 12NM zone. Typical operations in this zone are traffic monitoring for

- Pollution prevention

- Protection of Particularly Sensitive Sea Areas

- Maritime Assistance and Search and Rescue (SAR)

Coverage of coastal areas usually requires long-range sensors. Traffic of interest may include fishing vessels and larger vessels, but also smaller vessels in case of SAR operations.

Radar Characteristics – detection, accuracy

## Off-Shore

Off-shore areas may need traffic monitoring to protect oil/gas platforms and renewable energy harvesting, like windfarms. This requires a short to medium sensor range. Traffic of interest may include fishing vessels and larger vessels.

Radar Characteristics – detection, accuracy

# VTS Radar Considerations

This section is descriptive and intended to introduce the reader to common radar topics and knowledge specific to VTS radar.  Guidance for specifying VTS radar follow in subsequent sections.

## Radar Types

VTS radars could be of the following types:

- pulse radar (usually Magnetron based);

- pulse compression radar (usually Solid State);

- Frequency Modulated Continuous Wave, FMCW (usually Solid State).

A general explanation of each radar type will follow.

### Pulse Radar

A Magnetron based pulse radar typically transmits high peak power RF pulses (10 to 50 kilowatt (kW)) of very short duration (50 to 1000 nanoseconds).  The transmission is made with a pulse repetition frequency (PRF) of typically 1000 to 4000 pulses per second.  Upon reception, the returned signal is amplified, demodulated, and processed.

Main characteristics include:

- it is a well-known and proven technology.

- it has fixed pulse lengths.

- increased pulse duration translates into longer-range detection, but reduced range resolution and reduced ability to penetrate precipitation due to increased backscatter.

- normally with a fixed transmission frequency (or frequencies).

- requires wide frequency band allocation for compatibility with adjacent equipment.

- the need to reduce out-of-band transmissions. Where ITU requirements for land-based radar more stringent than for shipborne radar.

### Pulse Compression Radar

A pulse compression radar transmits low peak power modulated chirps (typically up to 200 Watt (W) and eventually higher) with a typical pulse duration of up to 100 microseconds (s).  The transmission is made with a chirp repetition frequency of typically 1000 to 20000 chirps per second.  Upon reception, the returned signal is amplified, pulse-compressed and processed.

The energy in the chirp of a pulse compression radar is comparable to the energy emitted in a pulse from a magnetron radar.  The longer chirps are converted into short pulses upon reception by the process of pulse compression; therefore, improved range resolution can be achieved at all ranges within a single radar mode.

Main characteristics include:

- it is based on well-known and proven principles, but at time of writing (2015), the high power at high frequency solid state amplifier technology relies on recent transistor developments.

- no need for magnetron replacement due to solid-state power amplifier, reducing the need for periodic maintenance.

- increased ability to penetrate adverse weather conditions facilitating smaller target detection.

- transmission frequencies can be programmed.

- cleaner spectrum than magnetron radars, with reduced emissions outside the allocated frequency band(s).

Additional challenges compared to magnetron radars include:

- the need for sophisticated interference rejection due to the longer chirps transmitted.

- the need for simultaneous short- and long-range detection increases complexity.

- high-power solid-state amplifiers operate with large currents therefore requiring careful design to obtain high reliability.

- by nature, the pulse compression radar creates so-called time (or range) side lobes.  Avoiding such side lobes, requires sophisticated techniques, alternatively side lobes suppression may imply a reduced detection of small targets in the vicinity of larger targets.

- additional focus on dynamic range to avoid saturation which will create very distinct sidelobes in range and reduce sensitivity in front of and behind large objects.

- inability or less sensitivity in the detection of RACONS and SART transponders.

Note: There may be legal restrictions (dual use, catch-all etc.) limiting the compression ratio and other parameters when importing (pulse compression) radars to certain countries.  This may limit the availability of this technology to some VTS Authorities.

#### Doppler Separation

The use of Doppler separation (based on target radial velocity) to achieve successful target separation, can improve performance, however, single targets can create multiple, complex Doppler returns which can lead to confusion when associating track updates with existing tracks.  Therefore, it is generally suggested that there should be a separation of at least one Doppler resolution cell when relying on Doppler separation.

Radars incorporating Doppler or MTI processing are subject to Doppler side lobes which in addition can limit Doppler separation of smaller, fast moving targets from large targets at the same range.

### Frequency Modulated Continuous Wave Radar

Frequency modulated continuous wave radar transmits low peak power continuous wave forms (typically up to 50 W).  The waveforms are repeated with a typical rate of 500 to 2000 per second.  Upon reception, the returned signal is amplified, compressed, and processed.

The energy in a frequency modulated continuous wave radar is comparable to the energy emitted in a pulse from a magnetron radar.  The waveforms are converted into pulses upon reception, therefore high range resolution can be maintained at all ranges.

Main characteristics include:

- well-known and proven principles.

- no need for magnetron replacement due to solid-state power amplifier, reducing the need for periodic maintenance.

- the ability to detect from very short range.

- transmission frequencies can be programmed.

- cleaner spectrum than magnetron radars, with reduced emissions outside the allocated frequency band(s).

Additional challenges compared to other types of radar include:

- dynamic limitations restrict the ability to handle small and large targets simultaneously and also affects long range detection.

- target revisit rate is low compared to typical VTS target kinematics.

- the need for sophisticated interference rejection, even more than for pulse compression radars.

- by nature, FMCW creates so-called time side lobes and suppressing the side lobes may reduce detection of small targets in the vicinity of larger targets.

- more complicated antenna system, 2 antennas or complicated antenna feed.

- the inability to detect RACONS and SART transponders.

## Frequency Bands

S-band, 2.0 – 4.0 GHz: Frequency band occasionally used for VTS Coastal radar to minimize weather on longer distance radar, e.g. for Coastal VTS– Frequency allocation (sub-bands for radiolocation and radionavigation services) predefined by ITU. However, frequency allocation is challenged by other users, e.g. Cellular phone operators aiming to expand their allocation.

Weather penetration used to be superior in S-band but is for modern radar a less dominating argument as X-band radars improve in capabilities.

C-band, 4.0 – 8.0 GHz: Not used for VTS

X-band, 8.0 – 12.0 GHz: The commonly used frequency band across VTS operational areas – Frequency allocation (sub-bands for radiolocation and radionavigation services) predefined by ITU. X-band is often a good compromise for balance between resolution, weather penetration and cost across application areas.

Ku-band, 12.0 – 18.0 GHz: Frequency band occasionally used for close range to optimise resolution and minimize antenna size. More sensitive to weather than S and X-band radar and typically limited to shorter distances by that reason (Inland and Port applications).

There is no Ku Frequency allocation for VTS predefined by ITU, meaning that frequencies shall be allocated by individual countries. It can be troublesome and expensive to obtain allocation in some countries, if possible!

## Antennas

The selection of antenna parameters (height, gain, side lobes, rotation rate, polarisation etc.) for a given installation, is integral to the resulting radar performance. VTS authorities are, however, advised to avoid specifying detailed antenna parameters in favour of identifying operational requirements such as coverage area  (based on risk assessments), track update rates (based on tracker design and typical target manoeuvres in high density areas), range performance, overlapping and redundant coverage.  The identified operational requirements will allow the radar expert some flexibility to achieve the best solution within the constraints of cost and location options.

### Antenna Principles

Typically, the VTS radar design includes an antenna, which provides a narrow beam in azimuth and a wide beam in elevation.  Thus, the VTS antenna is not designed to measure the target elevation (from which target height might be determined) or to separate targets based on elevation angle difference.

The installed antenna height is determined based on avoidance of physical obstructions, and the compromise between the need for close range visibility vs. long-range performance.

The radar designer, in his selection of antenna characteristics, needs to optimise the compromise between antenna size (and cost), track update rate, integration time on target (related to rotation rate and azimuth beamwidth and contributing to target detection range) and azimuth target separation and accuracy.  In addition, the choice of transmission frequency influences the size vs. beamwidth compromise.

Flat face electronic scanning antennas (phased array) may have advantages in terms of no rotating parts, flexible beam management, and the possibility to focus attention on some key directions.  Note this antenna technology may also have disadvantages in terms of technical complexity and cost.  Performance (e.g. variation in gain, blind coverage directions, beamwidth, and update rates) at all applicable scan angles should be considered when assessing radar antenna performance.

### Polarisation

Radio waves are polarised, and objects (target and clutter) will often reflect differently for the polarisation used.  This can be utilised by radar system designers, where rules of thumb are that:

- target returns from linear polarisation, (horizontal or vertical) in general will be stronger than returns from circular polarisation.

- non-metallic and natural objects, such as human beings will return linear polarised radio waves much better than circular polarised radio waves.

- most radar reflectors will be poor reflectors for circular polarised radio waves.

- distant ships with vertical masts tend to give the strongest return for vertical polarisation, whereas the opposite tends to be the case for modern ship designs without tall masts.

- In addition, target detection in clutter can be affected by the polarisation.  For instance:

- linear polarisation (horizontal or vertical) will result in higher rain clutter returns than circular polarisation.

- vertical or circular polarisation will result in higher sea clutter returns than horizontal polarisation, especially for lower sea states.

In summary complex designs are possible in which VTSOs may select the polarisation.  However, this adds to equipment costs and adds to the VTSO workload.

In general, the best cost/performance combined with ease of operation is achieved by horizontal polarisation.  Circular or switchable may however be appropriate to achieve acceptable performance for operation in areas with extreme (tropical) rainfall.

Note that, in the case of a radar station also being used for oil spill detection, the preferred polarization is vertical.

## Radar capabilities and constraints

### Radar coverage

Careful consideration should be given to optimise radar location(s) and antenna height(s) to ensure appropriate radar coverage and radar target separation.

To provide coverage over large areas and/or to mitigate shadow effects of other vessels, multiple radar sensors may be utilised.  Combined processing of images from two or more radars may also be utilised for elimination of false (ghost) echoes and to improve target discrimination.

#### Detection range

The below figure illustrates the geometry associated with antenna height above sea level (ASL) in target detection and the effect of the earth curvature. The calculated Target horizon for each site is shown in Table 4.5-1 below.

Target Horizon=2.2×htarget0.5+hantenna0.5NM

Figure 1 Geometry, Antenna height and target height

Lobing, or multipath, limits the detection range for targets lower than the first elevation lobe to a range somewhat smaller than the geometric target horizon summarized in  Table 3.

Table 2 Examples of target horizon versus antenna and target height in standard atmospheric conditions

|  | Target height ASL [m] |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Tower Height ASL [m] | 1 [NM] | 2 [NM] | 3 [NM] | 5 [NM] | 8 [NM] |  |
| 20 | 12.2 | 13.1 | 13.8 | 14.9 | 16.2 |  |
| 50 | 17.9 | 18.8 | 19.5 | 20.7 | 22.0 |  |
| 100 | 24.4 | 25.3 | 26.1 | 27.2 | 28.5 |  |

Abnormal propagation phenomena’s (Ducting) may further either reduce or significantly extend the target horizon.

#### Minimum detection range

Minimum detection range depends of antenna height and antenna vertical beam. Example values for minimum detection range can be seen in Table 3

Table 3 Examples of minimum versus antenna and target height

|  | Target height ASL [m] |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Tower Height ASL [m] | 1 [m] | 2 [m] | 3 [m] | 5 [m] | 8 [m] |  |
| 20 | 171 | 162 | 153 | 135 | 108 |  |
| 50 | 441 | 432 | 423 | 405 | 378 |  |
| 100 | 891 | 882 | 873 | 855 | 828 |  |

In case of need of reduction of these values special measures must be considered (using antennas with inverse cosecant beam or tilt mechanism)

### Target separation

Target separation (or discrimination) is a function of radar azimuth and range resolution as illustrated below.

Figure 2  Azimuth and range resolution

Where the range for a modern radar is determined by the frequency of range samples (cell size) and the effective bandwidth of the radar. The azimuth separation is determined by the number of azimuth samples and the characteristics of the antenna More to be done on this

Range:

| Figure 3 Two separate echoes separated in range by 11m | Figure 4 The power sum (RED) of the two echoes separated 11m |
| --- | --- |

Figure 5 The power sum (RED) of the 18m separation achieving a -6dB dip

Azimuth:

| Figure 6 Graphs depicting the return from two identical targets, sampled every 360o/4096 | Figure 7 The power returns summed in the red curve |
| --- | --- |

Figure 8 The power returns spaced to obtain -6dB separation

### Target Positional Accuracy

Target Detection Accuracy should be based upon the results of the risk assessment and the purpose of the VTS.

The operational requirements in terms of accuracy should consider:

- limitations of ships in the VTS area that may impose restrictions on the navigation of other ships (e.g. manoeuvrability), or any other potential hindrances.

- VTS space allocation.

- route advice.

- navigation assistance.

- responding to unsafe situations.

- timely manner

Several elements of a VTS system contribute to the track accuracy and these should be appropriately budgeted for before deriving the radar sensor accuracy requirements.  The target tracking requirements contained in Section Error! Reference source not found. should be used in combination with knowledge of the design of the target tracking function to derive the individual radar sensor measurement accuracy requirements.  Typical target position accuracy is provided in Error! Reference source not found.. This includes the effects of quantisation noise (related to radar cell size), plot extraction, calibration, and installation inaccuracies.

The “accuracy” is characterized in accordance with ISO 5725-1 by two figures; trueness (systematic error) and precision (random error), which are illustrated in Figure 10.

Figure 9 Definition of trueness and precision

The “Overall Accuracy” is the average deviation between sets of single measurements and the corresponding set of reference values. Overall accuracy thus equals the definition of trueness provided that enough measurements are averaged.

The overall RMS accuracy is calculated by:

AccuracyRMS=Trueness2+Precision2

The azimuth accuracy at video level is dependent on the turning gear and its associated azimuth bearing encoder resolution and accuracy e.g.

- Trueness, azimuth:	≤ 360° / 4096 = 0.088°

- Precision, azimuth: 	≤ 2 x Trueness i.e.  ≤ 0.176°.

The range accuracy at video level is dependent on cell size and the instrumented range e.g:

- Precision, range:	≤ 2 range cell size

- Trueness, range:	≤ (Range cell size) + 0.0005 x Instrumented range.

The impact of the antenna height on the measuring accuracy (slant range vs. plan range) is additional to the figures.

#### Target Positional update rate

The required radar update rate is determined by the behaviour of the expected target types.  Typical update rates for VTS lie between 2 and 4 seconds, however, values outside this range may offer alternative benefits to the overall design.

For short range applications with requirements to follow fast manoeuvring targets, update rates of 1-2 seconds may be beneficial.  Up to 10 seconds may be preferable for very long-range applications.

Note that tracking performance, especially the continuity of tracks, can be highly dependent on the radar update rate and there is a trade-off between radar sensitivity, accuracy, and the ability to follow fast manoeuvring targets.

### Dynamic characteristics

The dynamic range of the radar should, in normal weather and propagation conditions, detect and process the surface objects specified by the VTS Authority.  This should be achieved without significant side lobes, degradation of target appearance, degradation of detection or degradation of separation capability.

The required dynamic range is determined by:

- The ratio between the largest nearby objects expected and the smallest distant objects to be detected.

- Target return signal fluctuations including multipath.

Requirements for the radar(s) can be determined from the characteristics of the objects in the coverage area of the individual radar.  The characteristics of objects typically considered and the corresponding dynamic range, as a function of RCS and detection range, can be determined from Figure 11

The figure represents targets in free space.  This is normally sufficient for the determination of VTS radar requirements when combined with 10 dB allowance for target fluctuations.

Notes: If more accurate determination than that from the graph in Figure 11 is deemed necessary, this can be performed using performance evaluation tools, combined with evaluations of near range effects.

Technology limitations may restrict compliance with the extreme dynamic range that could be derived from Figure 11

Figure 10 Dynamic characteristics of signal received versus target RCS and target range (in NM) for point targets in free space

### Sidelobes

Side lobes (see Figure 12) are unwanted, as they will limit the size of a small Radar Cross Section (RCS) target that can be detected next to a large RCS target.

Range (time) side lobes

Antenna side lobes

Target

Target

.

Unwanted side lobes

Clean target echo

Figure 11 Range and Antenna (Azimuth) side lobe effects

The ratio between the peak level of the target and the highest side lobe is called the Peak Side Lobe Ratio (PSLR).

Typically, azimuth (antenna) side lobes are the only contributor in traditional magnetron pulse radars.  FMCW and pulse compression radars are subject to range as well as azimuth side lobes.

Side lobes should be sufficiently low to avoid masking of smaller targets in the proximity of large returns from targets or clutter.  In addition, low side lobes minimise the probability of false targets arising from other large returns.

#### Antenna Side Lobes

The antenna designer uses an amplitude weighting function to control the azimuth side lobe levels to a level which recognises the requirements for close in (< 10° from the main lobe peak) azimuth side lobes and the requirements for side lobes beyond this region.  Side lobes are specified as a ratio (in dB) relative to the antenna beam peak.

The antenna gain defined from a specific point in the radar system is specified as a ratio above ‘isotropic’ (dB or dBi) and can be increased by increasing the physical size of the antenna.  This will also reduce the beamwidth (azimuth, elevation or both).

Sidelobes exist booth in Azimuth and elevation, however, elevation side lobes are unlikely to be a major contributor to the performance of the VTS radar system. So called grating lobes are of special concern for array antennas and these should be included in consideration and measurement of side lobe levels.

Also note that structures near antennas (in some cases up to 100 metres) tend to distort wave propagation and, thereby, increase the azimuth side lobe level in the direction of such structures.

#### Range Side Lobes

Can be very disturbing in the case of modern radars, short description to be developed

#### Doppler Side Lobes

Radars incorporating Doppler or MTI processing are also subject to Doppler side lobes which can limit detectability of smaller targets competing with large clutter, but these radars offer performance benefits when compared to radars without Doppler or MTI processing.

Currently (2015), typical VTS radars do not achieve Doppler side lobe levels which are comparable to the figures in Error! Reference source not found. due to cost constraints.  30 to 40dB is a realistic expectation and this imposes a corresponding limitation on the visibility of small targets competing with Doppler side lobes from large returns at the same range.

## Target caracteristics

VTS radar targets are defined by their height above sea level, their speed and manoeuvrability, their polarisation characteristics, their radar cross section (RCS) and the fluctuations in RCS.

### Radar Cross Section

The amount of radar energy reflected by a target is proportional to its effective area called the Radar Cross Section (RCS). The RCS defined as the ratio between the power (in W) scattered by the target back towards the radar receiver and the power density (in W/m2) hitting the target. Thus, the RCS has the dimension of an area, being measured in m2. In practical terms, the RCS can be interpreted as the cross-sectional area of a perfectly reflecting metal sphere that would give rise to the same reflected energy as the target in question.

Being an effective area, the RCS of a target is not necessarily related to the physical cross-sectional area of the target, but rather depends strongly on the nature and shape of the surfaces exposed to the radar beam. Consequently, targets of similar physical sizes may exhibit vastly different RCS, and additionally, their RCS may vary drastically with aspect angle (see [2]) as the exposed surfaces change with orientation. Finally, the apparent RCS of a target is still prone to fluctuate over time, for instance due to minor changes in orientation between exposures to the radar beam.

To obtain quantitative and reproducible results during performance testing, it is therefore necessary to use physically small and rather simple targets and to measure and calibrate their RCS against that of a well-known reference target. During these measurements, the dependence of the RCS on the aspect angle can additionally be mapped out. A method to perform such a measurement is described in section 7.

### Target Models versus Detailed Characteristics

The point target characteristics in  will normally be sufficient for range calculation of specific targets of interest in VTS.

However, the design of a radar system should consider the overall characteristics of all objects within coverage range of the individual radar and Table 2 provides an overview of such characterises for targets, typically of interest to VTS radar.  Data is primarily based on references [3] + [4] , supplemented by data obtained from the experiences of IALA VTS committee members.

Table 4 Detailed Target Characteristics

| Target | Typical characteristics at X-band |  |  |
| --- | --- | --- | --- |
|  | RCS | Height | Fluctuations etc. |
| Aids to Navigation without radar reflector. | Up to 1 m2 | 1 to 4 m ASL | Rapidly fluctuating, highly dependent on sea characteristics. Polarisation characteristics will often vary depending on wind. |
| Aids to Navigation with radar reflector. | 10 – 100 m2 |  | Rapidly fluctuating, wind and currents may tilt to blind angles and lobing may cause reflectors to be in blind spots. Most radar reflectors will be poor radar targets in case of circular polarisation |
| Small open boat, fibreglass, wood or rubber with outboard motor and at least 2 persons on board, small speedboat, small fishing vessels or small sailing boats. | 0.5 – 5 m2 | 0.5 to 1 m ASL | Rapidly fluctuating may be hidden behind waves up to 50% of the time. Slow moving targets tend to lie lower in the water than fast moving ones and therefore RCS visible to the radar tends to increase with speed. Humans and non-metallic targets will give poor radar return in case of circular polarisation |
| Inshore fishing vessels, sailing boats and speedboats, equipped with radar reflector of good quality. | 3 – 10 m2 | 1 to 2 m ASL | Rapidly fluctuating. |
| Small metal ships, fishing vessels, patrol vessels and other similar vessels. | 10 – 100 m2 | 2 to 4 m ASL | Moderately fluctuating. |
| Coasters and other similar vessels. | 100 – 1000 m2 | 6 to 10 m ASL | RCS is highly dependent on aspect angle of the individual vessel. Rate of fluctuations is typically moderate. |
| Large coasters, Bulk carriers, cargo ships and other similar vessels. | 1000 – 10,000 m2 | 10 to 25 m ASL |  |
| Container carriers, tankers and other similar vessels. | 10,000 – 2,000,000 m2 | 15 to 40 m ASL |  |
| Buildings, cranes. Stacks of containers and other large structures. | Up to 1,000,000 m2 | Depends on site | Insignificant. |
| Floating items, oil drums and other similar items. | Up to 1 m2 | 0 to 0.5 m ASL | Rapidly fluctuating, highly dependent on sea characteristics and target movements. |
| Birds, floating or flying. |  | Sea level and up |  |
| Flocks of birds. | Up to 3 m2 | Sea level and up | Rapidly fluctuating, flight paths tend to be characteristic of given species in given areas of interest. |
| Jet Skis and other personal water craft | Up to 0.5 m2 | 0 to 1 m ASL | Rapidly fluctuating but virtually independent of aspect angle |
| Wind turbines (onshore and offshore) | Up to 10,000,000 m2 | Up to 300 m ASL | Fluctuations for towers are insignificant. Rotating parts give a wide spectrum of Doppler shifts with RCS up to hundreds of m2 |

Note: Modern warship design seeks to minimise RCS and, as a result, the above figures cannot be related to such vessels.

In case the physical size of the target exceeds the size of the resolution cell of the radar, the target gets extended into other resolution cells. In such a case, the RCS values, as mentioned above, may be incorrect (as the target reflection is now distributed across more than one resolution cell).  Additionally, the extension of the target poses extra challenges on detection and tracking performance. Extension of targets and the consequences thereof should especially be considered in situations where the radar is positioned close to the targets to be detected and tracked and/or in case of high-resolution radars.

RCS on targets using S-band is typically 40% and Ku-band is typically 120% of the RCS in X-band except for small non-metallic targets where the RCS difference between the 3 bands can be much higher.

### Target RCS Fluctuations

For VTS target types, statistical RCS fluctuations can adversely affect target detection.  To predict range performance more realistically, such fluctuations can be mathematically modelled using the different Swerling cases.  The radar design can to some extent compensate for the consequences of target RCS fluctuations.

### Target Speed and Manoeuvrability

The VTS Authority should ensure that the range of target speeds and manoeuvres is specified as part of the operational requirements.

### Multipath Effects

#### Reflections from large objects

Multipath conditions resulting from reflections from large buildings and large vessels can still impact on VTS radar performance resulting in the possibility of target signal cancellation or enhancement.  This effect is hard to predict and it is unreasonable to expect to model multipath affected performance unless a (potentially expensive) site-specific radar model is developed.

In addition, ghost targets can exist.  These derive from undesirable radar echoes resulting from multipath related wave reflections caused by large structures, buildings or vessels.

### Influence on Point Targets (Lobing)

One of the most common pitfalls during verification of radar systems is that the echo intensity from a point target can vary drastically with height. As illustrated in Figure 1, the radar signal will reach the target by travelling along different paths of which the most important ones are the direct path and the path where the radar signal has been reflected by the sea surface. The radar signals travelling along the two paths are added to form a resultant signal, a phenomenon known as multipath reflections. At some target heights, the two waves interference constructively, leading to increased signal levels (top left pane of Figure 1), whereas the waves interference destructively leading to decreased or even vanishing signal levels at other target heights (top right pane of Figure 1).

As a result, the coverage area exhibits a “lobed” structure where the detection probability alternates between high and low with target height, as illustrated in the bottom pane of Figure 1. Hence, if two point targets of equal RCS are placed at the same distance from the radar, their echoes can differ significantly if their heights happen to picked such that one is situated in a multipath peak (top left pane) while the other is at a multipath minimum (top right pane). Thus, the appearance of multipath reflections must be accounted for when selecting target heights and positioning the targets. Multipath reflections are severe at low sea states, elevated heights, and at long ranges. Furthermore, the lobing will depend on atmospheric conditions making range detection tests with point targets, e.g. traditional radar reflectors, very difficult and in practice impossible to reproduce. The practical solution is to use targets distributed in height, such as small vessels with measured RCS.

Figure 12 The effect of multi-path reflections on the detection probability of a target.

## Environmental Influence

There is a strong and complicated relationship between radar performance, geographical constraints and environmental conditions and it is highly recommended that individual assessments, involving radar and meteorological experts, are made for each VTS site separately.  The sections below indicate typical conditions covering the majority of VTS installations.

### Precipitation

Bring or develop textbook phrases of guideline to here

### Sea State

Bring or develop  textbook phrases of guideline to here

### Reduced Visibility, Fog, sand, Dust

Bring or develop phrases

### Air Mass (Propagation)

The performance of surface-based radar systems is strongly influenced by the electromagnetic properties of the atmosphere and the surface of the Earth.  In free space, electromagnetic waves propagate in straight lines from the antenna toward the targets and back.  However, radars located near the Earth’s surface should deal with, and adjust to the diffraction and refraction of the propagating wave.

Performance should, in all cases, be evaluated assuming Standard Atmospheric Conditions, combined with precipitation and sea state information for the individual location.  Evaluation of the effects from adverse propagation should in addition be included for hot, dry areas of the world, e.g. the Arab Gulf.

#### Propagation in the Standard Atmosphere

An electromagnetic wave observed at a target consists of a summation of an infinite number of contributions from different paths to the target leading to constructive and destructive contributions at the target.  The return path suffers from similar effects.  For small low targets this results in shorter detection ranges, than the distance calculated by simple line of sight calculations.

In addition, the barometric pressure and water vapour content of the standard atmosphere decreases rapidly with height, and the temperature decreases slowly with height.  This causes the electromagnetic waves to bend a little towards the Earth’s curvature.

Radar parameters, losses in transmission lines (not only waveguide), processing losses, clutter and precipitation add to the complexity and, even for the Standard Atmosphere, it is necessary to combine this with propagation factors by radar calculation tools in order to determine the predicted performance for a VTS radar sensor.  This is typically modelled by increasing the radius of the Earth by a multiplier (typically 1.33) and assuming straight-line propagation.

#### Sub-refraction and Super-refraction

Sub-refraction, bending the electromagnetic waves up, and super-refraction, bending the electromagnetic waves down, exists when the atmosphere deviates from the standard.

Sub-refraction can be caused by fast reduction of temperature and slower reduction of water vapour content with height, bending the electromagnetic waves towards space.  However, this phenomenon occurs rarely in nature.

Super-refraction can be caused by temperature increase with height (generally by temperature inversion) and/or rapid decrease of water vapour with height, decreasing the refractivity index (N).  Decreasing the refractivity gradient will eventually cause it to reach the critical gradient, at which point an electromagnetic wave will travel parallel to the Earth’s curvature.

#### Ducts and Trapping Layers

Super-refraction will develop into trapping layers, if the refractivity gradient decreases beyond the critical gradient, at which point the electromagnetic wave will be trapped and follow the Earth’s curvature.

Ducts act like waveguides for propagating waves bordered by trapping layers or the Earth’s surface.  To couple into a duct and remain in a duct, the angle of incidence must be small, typically less than 1°.

Ducting can be categorized into three main types:

- Evaporation duct:

- Weak, caused by evaporation from the sea surface, and only at low levels (maximum of 40 metres ASL).

- Generally increasing the radar horizon, especially for low mounted antennas.

- Surface-based duct:

- Surface ducts caused by low level inversion (increase of temperature /decrease of humidity with height), up to 500 metres ASL.

- Increase of radar range, depending on duct and antenna height.

- Elevated duct:

- 0.2-2 km above the surface.

- Typical no effect on surface-based antennas.

The effects are typically increased range but also increased amounts of noise and 2nd / multiple time around returns which may appear as false radar returns.

Notice that the electromagnetic waves are refracted (bent) and not reflected by the trapping layers.

#### Evaporation Ducts

Evaporation ducts exist over the ocean to some degree, almost all the time.  A change in the moisture distribution without an accompanying temperature change will lead to a trapping refractivity gradient.  The air in contact with the ocean surface is saturated with water vapour, creating a pressure that is decreasing to some value above the surface.

This will cause a steep refractivity gradient (trapping) near the surface but will gradually equalise towards normal refractivity gradient at a certain height, which is defined as the evaporation duct height.

Evaporation ducts are generally increasing the radar detection range and the antenna can be located above the duct and still have extended propagation strength.  The effect of evaporation ducts is usually more noticeable for higher frequency radars. This means that the radar range extension caused by an evaporation duct will in general be more noticeable for a X-band radar than for a S-band radar.

For typical coastal radar installations, evaporation duct heights of 6 – 15 metres lead to the longest detection ranges.  Evaporation duct heights of more than 10 metres will also introduce an increased amount of clutter, setting additional demands to clutter processing and noise reduction capabilities.

For example, investigations of weather data from the Arab Gulf area show that evaporation ducts exist all the time with typical duct heights of 5 to 25 metres, resulting in increased radar range in 80% of the time and increased clutter in 50% of the time.

Figure 14 Coverage diagram, in normal atmosphere (left) and including an evaporation duct (right).

#### Surface-based Duct

Surface based ducts can be much stronger than evaporation ducts.  They occur when the air aloft is hot (and dry) compared to the air at the Earth’s surface.  Over the ocean and near land masses, dry continental air may be advected over the cooler water surface; either directly from leeward side of continental land masses or by circulation associated with sea-breeze.

Figure 15 Example of simulated radar coverage in surface based + evaporation ducting conditions

In addition to the temperature inversion, moisture is added to the cool marine air by evaporation, increasing the trapping gradient.  In coastal areas, this leads to surface trapping ducts.  However, away from land, this trapping layer may well rise from the surface thereby creating an elevated duct.

The electromagnetic wave will be refracted towards the surface of the Earth and be trapped in the duct like in a waveguide.  This kind of trapping condition greatly increases the surface detection range – and the amount of noise received.  Note that surface detection may occur far beyond the radar horizon with a ‘dead zone’ in between.

Surface based ducts are often combined with evaporation ducts and examples of radar performance in such conditions, as illustrated by Figure 14

#### Elevated Duct

Generally, elevated ducts occur from descending, compressed and thereby heated air, from anticyclones, approaching the marine boundary layer and causing ducts.  Elevated ducts may also occur from elevating surface-based ducts.

Surface detection might also occur in this case far beyond the radar horizon with a ‘dead zone’ in between, adding noise to the radar image.

Figure 16 Coverage diagram, elevated duct

#### Severe Ducting at Coastlines Adjacent to Hot Flat Deserts

The large temperature variation between night and day in desert areas and the associated pressure differences between land and sea tend to cause very strong temperature inversion during night-time and result in strong sea breeze in the afternoons.  This can result in very severe ducts.  This type of duct can be very low, as little as 15 metres has been experienced

Range performance is very different for an antenna positioned inside or above these ducts, and radar systems with an antenna positioned within such a duct will have substantial increase in the detection range for surface targets.  If the antenna is positioned above the trapping layer (outside the duct), the electromagnetic wave will be refracted and the detection range for surface targets will be substantially reduced.

Figure 17 Coverage diagram based on a measured condition at a coastline adjacent to hot flat deserts.

Note.  The radar detection using antennas positioned inside the duct (left) and above the duct (right) corresponded to the simulated coverage diagram.

The sea breeze also causes eddies over the sea, forming distinctive sea clutter patterns.  The eddy results in a ‘snake’ like pattern moving forth and back for a few hours in the afternoon on hot days with strong sea breezes (see Figure 7). Of course, this may disturb display and tracking.

Eddy (yellow)

History (red)

Eddy (yellow)

History (red)

Figure 18 One hour of recordings with trials (snail tracks) shown in red.

Note.  The yellow ‘snake’ at sea is an eddy moving back and forth with a speed of approximately 4 knots.

## Other Influencing Factors

Obstructions, e.g. topography, buildings and other man-made structures may block or reflect radar signals.  Other radars and sources of electromagnetic radiation may cause interference.

Inland and harbour VTS will often require special considerations as the number of structures, their density and their close ranges can create very complex distortions.  Additional care should be taken to assess and mitigate effects caused by natural and man-made structures such as bridges, buildings, riverbanks, sheet metal pilings, and steep bends.

### Shadowing Effects

Radar detection may be blocked by shadowing effects that, to the extent possible, should be avoided.  Such effects include:

- Targets being hidden by larger targets or other obstructions;

- Masking of small targets by the effects of range and time side lobes.

These effects can be minimised by the appropriate siting of radars and selection of equipment with low side lobes.

### Interference

Interference can be split into susceptibility (received interference) and compatibility (transmitted interference).

To minimise interference, separation between wanted and unwanted transmissions has to be optimised – this can be achieved by a combination of frequency separation, physical separation of transmission sites, antenna directionality, sector blanking, separation by time and also by ensuring that all the systems competing for the same or adjacent spectrum do not transmit excessive and unnecessary transmit power levels or transmit time periods.  The radar receiver design will typically be very sensitive (to achieve the required performance), although gain control techniques (swept gain or Sensitivity-Time Control (STC)) offer further resistance.  Waveform designs incorporating staggered PRFs and processing schemes designed to reject known interference patterns can also aid the radar receiver to suppress unwanted returns.

### Radar Susceptibility

In the case of any radar installation, (e.g. a permanent VTS installation of a radar or of each radar in a VTS network), the performance of that radar can be detrimentally affected by received emissions from other radiating sources (physically adjacent VTS radars, maritime shipborne radars, and other users (legitimate or otherwise) of the electromagnetic spectrum.  Typically, local legislative regulations and restrictions will control and minimise unwanted received signals, but elimination of such signals is likely to be impossible.  National spectrum allocation authorities should always be approached by a VTS integrator when considering any changes to a VTS network (radar, microwave link, communications etc.) to enable a holistic view of the changes and how they might affect all users.

The radar design can assist in minimising the susceptibility to unwanted received interference, e.g. by utilising low antenna side lobes, avoiding large reflecting surfaces, minimising receiver front end bandwidth etc.

Note that FMCW and pulse compression radars may typically require larger front-end receiver bandwidths than conventional magnetron systems.  The multi-pulse waveforms transmitted (and consequently received) by pulse compression radars have to achieve a compromise between pulse chirp bandwidth (related to range cell size and hence range resolution), use of frequency diversity (to optimise performance in clutter), unwanted pulse to pulse interaction and so on versus spectrum usage and hence unwanted susceptibility with other transmitting spectrum users.  FMCW radars transmit and receive (at low levels) for 100% of the time across a swept bandwidth.

### Radar Compatibility with Other Users

In the case of any radar installation (e.g. a permanent VTS installation of a radar or of each radar in a VTS network), the performance of adjacent systems can be detrimentally affected by transmitted emissions from the radar in question (physically adjacent VTS radars, maritime shipborne radars, and other users (legitimate or otherwise) of the electromagnetic spectrum).  Typically, local legislative regulations and restrictions will control and minimise unwanted transmitted signals but elimination of the influence of such signals is likely to be impossible.  As with susceptibility above, National spectrum allocation authorities should always be approached by a VTS integrator when considering any changes to the RF sub- systems within a VTS network.

The radar design can assist in minimising the impact of transmitted signals, for example by utilising low antenna side lobes, avoiding large reflecting surfaces, minimising transmit power, consideration of peak and mean power levels, sector blanking, physical location of the radar etc.

Note that conventional magnetron radars require larger peak power levels than pulse compression and FMCW radar systems.  The magnetron technology can result in unnecessary wideband spectral emissions unless steps are taken to include frequency band pass filtering (which has an inherent loss to the wanted signal).  However, pulse compression radars and FMCW radars, although utilising lower peak powers, use techniques, which may include frequency modulation, pulse-to-pulse frequency variation, frequency diversity etc. all of which increase the use of the spectrum and increase the chances of unwanted degradation of adjacent systems.

### Influence from Wind Farms

Wind turbines produce large static target-like returns which, from a VTSO’s perspective, are normally easy to distinguish from vessel traffic.

The complex return from a wind turbine is made up of two key elements:

- The tower and generator housing offering a large static zero-Doppler RCS, in some cases up to 1 million m2;

- The rotating blades of the turbine offering a complex spread of non-zero-Doppler RCS, typically up to 100 m2, which will vary with wind direction and speed.

This composite return will be seen as a large static target by a conventional pulse radar, whereas FMCW and coherent radars using Doppler processing will see a complex target spread across the Doppler domain.

The influence, independent of the applied radar technology, will be reflections causing unwanted ghost echoes and suppression of nearby targets.  The large RCS may also result in antenna side lobe returns, resulting in reduced detectability.  The symmetrical layout of wind farms may add further to the disturbances.

## Signal Processing and Tracking

## Analysis of Radar Detection Performance

The recommended method for determination of radar coverage and range detection performance is a combination of site inspections and radar system performance predictions.

The evaluation should include:

- range detection performance focused on the smallest targets of interest in poor weather conditions. All applicable losses should be included in calculations

- evaluation of the effects from propagation conditions and obstructions

- an evaluation of dynamic requirements.

The calculations may be supplemented by comparison and/or validation test.

It will typically not be possible to encounter all combinations of variables, and calculations are therefore made based on a simplified model of the targets and the environment based on statistical information with associated limitations and tolerances.

Furthermore, in the design of radar systems it is very important not only to focus on maximum detection range but also on radar quality at all ranges, the ability to handle clutter, the ability to suppress interferences and the ability to simultaneous handling of defined (small and large) targets in the VTS area covered by radar.

Calculation for magnetron radars can by radar modelling.

Warning: The cumulative detection can be very misleading if used for determination of VTS detection performance.

Radar models may not be sufficient for determination of performance for pulse compression radars, where it may be necessary to rely on vendor furnished information, possibly supported by performance tests.

It should also be noted that radar performance predictions are indications and not guarantees of real-world performance.  There are many variables within a CARPET-based prediction that can only be considered to be approximations of target, radar and environment behaviour.

Warning: The use of radar prediction models requires full understanding of their validity and limitations.  Their use as a comparative tool should be performed by radar experts and any predictions may not represent achievable real-world performance.  It is recommended that performance validation against expectations includes live testing.

### Probability of Detection and False Alarm Rate

The probability of detection and the false alarm rates, used for calculations, should comply with those required to meet the operational performance.  Please note the definitions of radar PD and radar PFA (refer to section Error! Reference source not found.).

It should also be noted that improved system performance may be obtained by allowing a higher radar PFA  in combination with subsequent, enhanced plot extraction and tracking.

At specified maximum coverage ranges, the single-scan probability of detection values for VTS will typically lie in the range from 0.7 to 0.9.

It is normally desirable not to have noise and clutter spikes presented to the VTSO in each scan.  Classically, optimal false alarm rates for VTS applications normally lie in the range from 10-4 to 10-5 for the radar video display.  Different values may apply for the tracking, on the condition that tracking requirements are met, however, with modern high resolution technology there is a tendency to allow higher false alarm rates and let the extraction and tracking discriminate between noise and targets of interest.

The false alarms, taken into account in the calculations, should include unwanted information from noise and clutter, as presented to the VTSO or to the tracker (after signal processing), but not signals from other unwanted objects.

# Producing requirements

- Producing requirements for VTS radar is an interactive task involving iterations including evaluation of achievable performance versus overall system cost.  It might, for example, be better to start with simpler solutions, meeting the available budget, than to be left without any radar coverage.

- The task will consider one or more operational areas with specific characteristics and for such area(s) include definition of requirements to:

- radar coverage

- target detection

- target separation

- target positional accuracy

- target positional update rate

- elimination of false echoes, sidelobes and handling dynamic characteristics of targets and surrounding

- handling of local environmental conditions including environmental impact on targets detection

- handling other influencing factors, possible radar location(s), obstructions

- functions and other features, tracking included

- robustness, availability, and serviceability

Furthermore, the radar coverage required for a VTS may consist of multiple radar sensors either co-located or distributed to optimise availability, radar data integrity, security constraints and equipment access considerations.  There are several design decisions regarding the cost, complexity, location, and detailed design of the radar sensors within a VTS.

In addition, there are design decisions and trade-offs to be considered for each radar sensor including frequency band (Ku, X and/or S), magnetron vs. solid state, transmission power, antenna size, receiver technology, signal processing.  All of which need to be considered together in the specifications of each radar sensor within the Radar service.

Different vendors is likely to have different solutions to the same problems, but with different price tags. Therefore, the VTS Authority should keep options open, avoiding detailed technical specifications. VTS vendors should propose solutions to meet the VTS Authority specified operational and functional requirements.

## Radar Coverage

## Target Detection and Range Performance

The operational requirements shall identify the target types found within the VTS area, refer to Table xx for details. For each target type the operational range should be specified.

In addition, any special objects of interest should be specified separately.

It should be noted that the environmental conditions will impact on target detection.

The achievable target detection range is highly dependent on several factors including antenna height, target characteristics, weather and atmospheric propagation conditions.

### Range performance

The graph below includes typical examples of detection and tracking ranges for X-band radar systems, in standard atmospheric propagation conditions.

It is normally sufficient to calculate detection range for small and medium size targets, therefore IALA targets type 6 and 7 typically not used for calculations.

The minimum ranges stated in the tables are those typically achievable from a pulsed radar with a fan beam antenna.  Geographical conditions may set other requirements and other values may offer benefits to the overall design.

### Target Separation and Accuracy

Target separation (or discrimination) and target accuracy requirements should be considered separately.

### Target Separation

The required target separation should be based on a risk assessment. The risk assessment should specify the minimum separation requirement between surface objects or vessels at defined locations. These objects should be individually detected.

At long range, the impact of the height and type of antenna on the resolution performance should be taken into account.  The system should also be capable of displaying and tracking all detectable targets of interest simultaneously in normal conditions, preferably without the need for manual adjustments by the VTSO.

Typically, target separation is achieved by a combination of range and azimuth (and Doppler, when available). The following sections assume separation is achieved individually in each of these dimensions.

### Range Separation for Small Point Targets

The range separation is, for traditional magnetron radars, linked to the transmitted pulse length, see Error! Reference source not found..  Substantially better range separation may be achieved by pulse compression radars.

Typical Range Separation

| Typical Range Separation of Small Point Targets [m] |  |  |  |  |
| --- | --- | --- | --- | --- |
|  |  | Basic | Standard | Advanced |
|  |  | -6 dB points |  |  |
| Minimum Range Separation | Less than 5 NM instrumented range | 25 | 20 | 15 |
|  | 5-20 NM instrumented range | 75 | 60 | 50 |
|  | More than 20 NM instrumented range | N/A | 100 | 80 |

### Azimuth Separation for Small Point Targets

Separation for similar sized, point targets are given by ……………

### Separation of Larger Targets

Beyond the separation of point targets, the VTS Authority is recommended to express target separation requirements in operational terms rather than radar subsystem parameters.

For larger (non-point) targets and for separation of dissimilar sized targets, the definition of separation is highly dependent on physical size, aspect angles, relative return amplitude, pulse stretch, and other radar characteristics.

## Target Positional Accuracy

Figure 19 Obtainable azimuth accuracy versus typical antenna beamwidths used for inland waterways and port areas  (shall be revisited to allow for tolerances, azimuth sample as well as values typical for Ku Band)

Figure 20 Obtainable azimuth accuracy versus typical antenna beamwidths used for port approach and Coastal areas  (shall be revisited to allow for tolerances, azimuth sample as well as values typical for S Band)

## Target Positional Update rate

Table 5 Update rate

| VTS area | Typical Positional Update Rate |
| --- | --- |
| Inland waterways, like rivers and canals | 1.5 – 3 seconds |
| Port with berths and anchorages | 1.5 – 3 seconds |
| The approach | 1.5 – 3 seconds |
| Coastal, including shipping lanes | 3-6 seconds |
| Off-shore installations, like platforms and windfarms | 2-4 seconds |

## Elimination of False Echoes, Side lobes & Dynamic range

### Elimination of False Echoes

The radar should also be designed and installed so as to eliminate, to the maximum extent possible, false echoes caused by side lobes, reflections from nearby structures or second/multiple time around echoes.

### Acceptable side lobe levels

Error! Reference source not found. recommends the maximum allowed signal presented to the display and plot extractor, originating from antenna and range (or time) side lobes.

The figures account for two-way propagation, therefore the antenna side lobe requirements (one way) equal half the values indicated (dBs divided by 2).

Maximum Side Lobe Level Relative to Non-saturating Target Signals

|  | Level |  |  |
| --- | --- | --- | --- |
| Maximum near side lobe level (within +/- 10 in azimuth and +/- 250 m in range from any target) | - 52 dB (-26 dB one way) | - 54 dB (-27 dB one way) | - 56 dB (-28 dB one way) |
| Maximum far side lobe level (outside +/- 10 in azimuth and +/- 250 m in range from any target) | - 66 dB (-33 dB one way) | - 68 dB (-34 dB one way) | - 70 dB (-35 dB one way) |
|  | Level |  |  |
| Maximum near side lobe level (within +/- 10 in azimuth and +/- 250 m in range from any target) | - 52 dB (-26 dB one way) | - 54 dB (-27 dB one way) | - 56 dB (-28 dB one way) |
| Maximum far side lobe level (outside +/- 10 in azimuth and +/- 250 m in range from any target) | - 66 dB (-33 dB one way) | - 68 dB (-34 dB one way) | - 70 dB (-35 dB one way) |

## Handling of Dynamic Range, targets and surroundings

## Handling of environmental Conditions

Special local conditions such as heavy rainfall should also be considered.  Additional factors affecting the detection performance of radar systems include noise, interference, clutter and propagation.

The environmental conditions of the VTS area should be described, including the annual cycle.  This should include conditions to be expected as normal over a 12-month period as well as extreme events.

Restrictions with respect to operation and access to site(s) due to weather should also be included.

### Precipitation

Precipitation can significantly affect on the radar coverage of the VTS system due to reduction of radar signal propagation.

Typical example of the degradation of radar coverage is displayed on the picture XXX illustrated detection range of Type 3 target in a good weather conditions and 10mm/h rain.

Information about precipitation over the VTS areas should be obtained from meteorological services, and the VTS Authority should define the required radar performance requirements in clear conditions as well as for normal precipitation conditions.

Note that rainfall is rare in dry/hot regions, maybe only once or twice per year and the VTS Authority should consider if rain should be specified at all.

Also note that designing a system to perform in tropical rain showers will typically call for S-band radars.  However, the worst rain conditions may only be present for a few hours per year, and reduction in performance on other parameters, may not justify the additional investment.

### Sea Conditions

Information about the Sea State over the VTS areas should be obtained and the VTS Authority should define the required radar performance requirements in such conditions.

## Functions and other Features

The Radar service in a VTS should, as a minimum, support the operational functions specified by G.1111-1 (Target tacking included) as well as the functions listed below.

These functions may be hosted by the radar sensors or other parts of the VTS system.

### Clutter and Noise Reduction / Management

Appropriate, clutter reduction or clutter management should be available to meet the performance requirements.

This will typically include:

- White noise suppression

- Interference rejection

- Sea and rain clutter processing

- Adaptation to varying propagation conditions.

The features should preferably be automatic. These functions may be hosted by the radar sensors or other parts of the VTS system

### Operator Functions

Radar functions should be designed and implemented to optimise performance and minimize VTSO workload to the level practical.  Ideally, the VTSO should only need to control basic functions such as start and stop.

Each radar site should be designed and equipped to reduce the adverse effects of rain and sea clutter and enhance the probability of target detection and it is recommended to make adaptation to changing weather conditions, etc. automatic to reduce VTSO workload.  However, it might be necessary to implement dedicated operational modes e.g. for adaptation to weather.

Manual override of automatic functions should always be possible.

### Operational Outputs

The output from a radar service should include radar image data and track data.  In addition, the output from the radar service may include clutter data to enable identification of, for example, squalls, oil spills, ice detection, wave height, etc.

## Radar Design, Installation and Maintenance Considerations

The radar systems should in general be designed taking the general considerations in Section Error! Reference source not found. into account.

Special safety precautions for radar should include but not be limited to those applicable for Rotating Machinery, Radiation Hazards and Electrical Shock.

Special precautions should also consider lightning protection, wind load on antennas and access to the systems, including antennas for installation and maintenance.  Turbulence, asymmetrical wind and vertical wind components should be considered with reference to the descriptions in Section Error! Reference source not found..

Waveguides should in general be kept as short as possible.  Their length and associated losses need to be included when determining system performance.  Waveguides should always be equipped with dehumidifiers or simple desiccators.

Concerning the lightning rod, it is recommended to place this in a blanked sector or in a direction of low practical importance.

### Service Access

Service access should be possible at the individual radar location.

A local service display, at each radar sensor, providing radar control, BITE results and other specified radar information is recommended.

To the extent practical, service access should be possible remotely, for example, at the central monitoring location for the VTS network.

### Antenna Accessibility

When siting a radar, the accessibility should be carefully considered – for example an access ladder and maintenance platform may be required to ease maintenance on a tower or tall building.

### Tower Considerations and Antenna Robustness

The installation of a radar can introduce problems resulting from high winds.  In some cases, it may be appropriate to separately specify both the survival wind limits and a lower operational wind limit within which the system should not be degraded due to the normal weather conditions specified for that location.  High winds can affect the motor and gearbox design and can affect the instantaneous rotation rate at varying angles to the predominant wind direction.  The build-up of ice in some climates should also be a consideration.

Plot accuracy can be affected by high winds and the overall system azimuth accuracy should consider torsional errors arising from high winds.

In extreme conditions, it may be appropriate to house the rotating radar antenna within a static radome although this can increase costs and RF losses and complicate maintenance of some components.

### Choice of Upmast versus Downmast Transceivers

The radar designer may also have a choice of whether to locate the transceiver upmast or downmast.  In the latter case, a waveguide run may be required to link the RF output / input of the transceiver to the antenna.  Such a run of waveguide will introduce losses which should be considered as a part of the evaluation of the predicted performance of a given radar installation.

Conversely, an upmast transceiver installation may be more difficult to access for maintenance and servicing than a downmast transceiver which might also benefit from an environmentally controlled location and consequent improvement in equipment reliability.

Further issues might include the need to transfer high bandwidth video (on copper, fibre or microwave link etc.) which can influence the radar designer’s selection of whether to use an upmast or downmast transceiver.  Incorporation of transceiver redundancy can also influence the choice of transceiver location.

### Built In Test Equipment

BITE should include monitoring of functions and performance.  Communication of summary alarms and system status to a central monitoring system may be required.  It is recommended that detailed BITE results are made accessible for remote monitoring, especially for radars installed in locations that are difficult to access.

### Protection against Extreme Events

Authorities responsible for VTS areas subject to adverse weather such as cyclones, typhoon, hurricane, and tornado should consider the potential impact and specify requirements to equipment survival.  Radar operation is normally suspended in such conditions.

Refer to Section Error! Reference source not found., Section Error! Reference source not found. for further guidance

# Acceptance of VTS Radar Systems

It is recommended to base acquisition of radars and subsequent verification after installation based on measured performance data using real targets.

Such measurements should be carefully analysed including the influence from weather and propagation.

Note that verification of radars using floating point targets, such as corner reflectors or Lunenburg reflectors, is subject to large inaccuracies due to sea surface movements and variations in propagation. It is there suggested to measure the radar cross section of real targets and use those for actual measurements.

Radar cross section measurement of targets should be made in calm sea conditions, at close range and using stable (not moving) Luneburg reflectors as reference.

Note: Measurement of accuracy using small floating targets can be unreliable due to target fluctuations.  Instead it is recommended to use small radar reflectors placed in a clutter free land area, e.g. a paved area or a beach.  If a suitable land area is not available, the measurement of accuracy should be carried out in a calm sea area.

It is normally sufficient to measure at one or two points in the distance, typically with reflectors placed in the far field beyond 1 nautical mile from the radar.  For short range applications a similar test at the range(s) of interest may be more appropriate.

## Verification of Coverage and Target Detection Performance

A qualitative measurement can be carried out using targets of opportunity. A relatively easy way to examine the coverage is to use several hours of radar video recordings and/or tracking information and superimpose all tracks to obtain so-called “snail tracks” as shown in Figure xx.

Figure 21 Example of a video capture with “snail tracks” lasting several hours and showing uninterrupted vessel traffic within the coverage area of a radar sensor

Naturally, this approach only reveals the radar coverage area within areas of normal shipping routes. If necessary, coverage outside these areas can be examined by use of controlled targets.

Validation of detection performance is mainly concerned with determining the maximum range at which a target of a specific RCS is detected with specified probability of detection, typically 90 %.

These measurements should be carried out using controlled targets with predetermined RCS as described in section xx and at least one target should exhibit an RCS close to the minimum RCS value specified for the actual site. The weather plays a significant role for target detection and conditions during testing should preferably resemble those described for the specific target type in requirements. However, tests might be scheduled to a fixed time span, implying that the test must carried out under the actual weather conditions at the given time. In this situation, the test results can be used to benchmark simulation tools which then can be used to predict range detection performance under the specified weather conditions.

The planned test trajectory should preferably be in a radial direction along free line of sight and extending from close to the radar to at least 10% beyond the expected detection range. An example of such a trajectory is shown in Figure xx. Furthermore, the test target can be asked to perform a manoeuvre, like sailing along a large circle, after traversing a certain distance as also illustrated in Figure 5. The manoeuvre serves a dual purpose: it helps to identify the controlled target in the radar video, and the radar echo as a function of aspect angles is re-evaluated.

Figure 22 Example of a test trajectory for range detection testing

Test sequence: Observe the radar performance from the radar display and establish tracking of the test target. The following steps are then carried out:

1) Ask the controlled target to move along the predetermined test trajectory including the manoeuvres repeated every 1-2 NM

2) For each antenna scan, observe whether or not the target is visible and/or tracked in the radar video. Record the result

3) Re-establish tracking if target tracking is lost, e.g. as a result of increased clutter levels, shadowing effects due to large ship, or similar

4) Continue the test until target tracking is permanently lost, and until video plots of the test targets are seen in less than 3 out of 10 antenna scans

5) Repeat the test with the target sailing inbound

If necessary, the test can be repeated at other selected bearing and/or for different target types. Verify that the observed detection range is at least the value stated in the requirements.

## Dynamic Capabilities

## Side Lobe Performance

## Target separation

Verification of target separation aims to determine the minimum distance, in range or bearing, by this guideline defined at which a 6 dB notch appears between the returns from two close targets. It is important NOT to use extended targets for the test, but rather use point targets of, preferably, equal RCS. As the distance between the two targets must be changed during testing, at least one of the targets must be moveable. It is extremely cumbersome to perform the test at sea, as it requires floating supports for the reflectors which additionally must be moved relative to each other on the sea surface. Furthermore, if performed at sea, the test should not be performed in higher sea states than 2. A much easier, but just as valid, approach is to perform the test on land, e.g. at a beach or flat paved area, thus providing much better control of the positioning of the reflectors. Consequently, it is preferable to perform the test on land.

### RANGE SEPARATION

Figure 23 Measurement of range separation

Test sequence:

1) The two reflectors are placed in the test area at the same bearing and with a radial distance of more than 200 meters between the two. One reflector is kept fixed throughout the test.

2) The moveable reflector is slowly moved towards the fixed reflector until a 6 dB notch between the two returns is achieved (Figure 6).

3) At this point, record the distance between the two reflectors.

The test is passed if the recorded distance is less than the range separation required.

### BEARING SEPARATION

Figure 24 Measurement of bearing resolution

Test sequence:

1) The two reflectors are placed in the test area at the same radial distance beyond 1 NM from the radar to ensure the reflectors are placed in the far field region of the antenna radiation

pattern. The distance between the two reflectors along the azimuthal direction should be more 400 meters. One reflector is kept fixed throughout the test.

2) The moveable reflector is slowly moved towards the fixed reflector until a 6 dB notch between the two returns is achieved (Figure 8).

3) At this point, record the distance in bearing between the two reflectors.

The test is passed if the recorded distance is less than the bearing separation stated in the requirements.

## Target Positional accuracy

It is recommended to verify the target position accuracy utilising at least two reference targets at known positions. It is normally sufficient to measure at one or two points in the distance, typically with reflectors placed in the far field beyond 1 nautical mile from the radar.  For short range applications a similar test at the range(s) of interest may be more appropriate.

The measurement must be done with fixed targets, as the position of a floating target is not stationary. The optimum is to use small radar reflectors placed in a clutter free land area, e.g. a paved area or a beach.

The range and azimuth position of the target peak intensity in the radar video shall be monitored over minimum 20 scans and the position during each scan recorded. At the end of the measurement, the range and azimuth deviation between the recorded positions and the known reference position is calculated and the result compared to the tender requirements.

## Test Targets and their Callibration

Test targets, also referred to as the controlled target, is assumed to be a small vessel which typical for test of VTS systems.

The RCS of the test targets is needed to obtain quantitative and reproducible results during testing of range performance, and it is necessary to calibrate the RCS of the test targets against a known reference reflector.

A prerequisite for performing RCS calibration is furthermore that the radar receiver characteristics is known, or more specifically that the relationship between the received power and the output video level is known. If not, the characteristics must first be mapped out.

The calibration procedure must be performed in calm weather with a low sea state and a well-known reference reflector must be at hand. An example of a reliable reference reflector is a calibrated Luneberg reflector mounted on a floating support as shown in Figure 25. It is important that the height of the reference reflector resembles that of the test target to be calibrated. If not, the measurements can be influenced by lobing as discussed in section 5.4.5 which leads to erroneous RCS estimates for the test targets.

Figure 25 Example of a 2 m2 reference target prepared for floating.

Calibration sequence:

The controlled target and the reference reflector should be placed within 2 NM from the radar sensor and preferably within 0.5 to 1 NM. Moreover, the targets should be positioned within free line of sight of the radar sensor and away from objects yielding intense radar echoes.

With the reference target on board, the controlled target is positioned in the test area where the following steps are carried out:

1) The reference reflector is placed in the water on its floating support.

2) The controlled target moves away from the reflector until the two targets are clearly separated on the radar display.

3) The radar gain is adjusted to obtain a clear picture of the test area with the test and reference targets clearly visible and stable in the water, both in azimuth and range. Make sure that any automatic gain adjustment is disabled such that the gain will remain constant throughout the calibration procedure.

4) Record the intensity of the reference target echo during at least 30 consecutive scans. Convert all recorded intensities to received power levels using the receiver characteristics. Calculate the average value and variance of the received echo power. The average received power level, 𝑃ref,av, is the power level expected from a target with the RCS of the reference target, 𝜎ref. Thus, a calibration constant, 𝑎cal, may now defined as 𝑎cal=𝜎ref/𝑃ref,av.

5) The controlled target is then positioned with its stern pointing the direction of the radar, see Figure 18.

6) Record the intensity of the controlled target echo during at least 30 consecutive scans. Convert all recorded intensities to received power levels using the receiver characteristics. Convert the received power, 𝑃target, to an RCS value according to 𝜎target=𝑎cal∗𝑃target. Calculate the average value and variance of the measured RCS.

7) The controlled target changes its orientation in steps of 45 degrees and after each orientation change, the measurement in 6) is repeated. The last measurement is performed with the controlled target seen from astern.

8) Let the controlled target move in a circle cantered at the reference target and with a radius of approximately 100 meters. In each scan, record the intensity of the controlled target echo. Convert all recorded intensities to received power levels, 𝑃target, using the receiver characteristics and further to RCS values according to 𝜎target=𝑎cal∗𝑃target. Calculate the average value and variance of the measured RCS.

The RCS of the target has now been measured for several different orientations (step 6 and 7), and in addition a value for the overall RCS (the RCS averaged over all orientations) has been obtained in step 8.

Figure 26 Target aspect angle relative to the radar antenna

------------------ END ---------------------